# SECTION 01352 (01 81 13)

## LEED FOR SCHOOLS REQUIREMENTS

- GENERAL

- SECTION INCLUDES

- ***********************************************************************************************************************

- NOTE TO SPECIFIER:  The options indicated in Paragraph A below must be selected to indicate whether the Project is going for a USGBC LEED Certification or if the Project is to comply with LEED requirements but without going for a certification (ie:  “LEED Requirements Without Certification”).

- ***********************************************************************************************************************

    - This Section is in compliance with F.S. 255.2575 Energy-efficient and sustainable buildings (for Public Property and Publicly Owned Buildings).

    - Section includes general requirements and procedures for compliance with certain USGBC LEED for School prerequisites needed for Project to obtain [LEED Requirements without Certification] based on LEED for Schools 2009.

  - RELATED SECTIONS

    - Divisions 01 through 17 Sections for LEED for Schools requirements specific to the work of each of these Sections.  Requirements may or may not include reference to LEED for Schools.

    - DEFINITIONS

    - Chain-of-Custody Certificates:  Certificates signed by manufacturers certifying that wood used to make products was obtained from forests certified by an FSC-accredited certification body to comply with FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."  Certificates shall include evidence that manufacturer is certified for chain of custody by an FSC-accredited certification body.

    - LEED:  Leadership in Energy & Environmental Design.

    - Rapidly Renewable Materials:  Materials made from plants that are typically harvested within a 10-year or shorter cycle.  Rapidly renewable materials include products made from bamboo, cotton, flax, jute, rubber, straw, sunflower seed hulls, vegetable oils, or wool.

    - Regional Materials:  Materials that have been extracted, harvested, or recovered, as well as manufactured, within 500 miles of Project site.  If only a fraction of a product or material is extracted/harvested/recovered and manufactured locally, then only that percentage (by weight) shall contribute to the regional value.

    - Recycled Content:  The recycled content value of a material assembly shall be determined by weight.

      - "Post-consumer" material is defined as waste material generated by households or by commercial, industrial, and institutional facilities in their role as end users of the product, which can no longer be used for its intended purpose.

      - "Pre-consumer" material is defined as material diverted from the waste stream during the manufacturing process.  Excluded is reutilization of materials such as rework, regrind, or scrap generated in a process and capable of being reclaimed within the same process that generated it.

      - SUBMITTALS

    - General:  Submit additional LEED submittals required by other Specification Sections.

    - LEED submittals are in addition to other submittals.  If submitted item is identical to that submitted to comply with other requirements, submit duplicate copies as a separate submittal to verify compliance with indicated LEED requirements.

    - Project Materials Cost Data:  Provide statement indicating total cost for materials used for Project.  Costs exclude labor, overhead, and profit.  Include breakout of costs for the following categories of items:

      - Electrical.

      - Furniture.

      - Mechanical.

      - Plumbing.

      - Specialty items such as elevators and equipment.

      - Wood-based construction materials.

    - LEED Action Plans:  Provide preliminary submittals not less than 10 days before the Pre-Construction meeting indicating how the following requirements will be met:

- ***********************************************************************************************************************

- NOTE TO SPECIFIER:  Select the option below in Subparagraph 1 if the recycled content is 20 percent or greater.  Select the option below in Subparagraph 2 if the regional material content is 20 percent or greater.

- ************************************************************************************************************************

      - Credit MR 4.1[and Credit MR 4.2]:  List of proposed materials with recycled content.  Indicate cost, post-consumer recycled content, and pre-consumer recycled content for each product having recycled content.

      - Credit MR 5.1[and Credit MR 5.2]:  List of proposed regionally extracted and manufactured materials

    - LEED for Schools Progress Reports:  Concurrent with each Application for Payment, submit reports comparing actual construction and purchasing activities with LEED for Schools action plans.

      - Credit MR 4.1 [ and Credit MR 4.2]:  Recycled content.

      - Credit MR 5.1 [ and Credit MR 5.2]:  Regional materials.

- **********************************************************************************************************************

- NOTE TO SPECIFIER:  The Submittals in Paragraph D below are required to verify compliance with the LEED for Schools Prerequisites and Credits should the Project be registered with the USGBC.  Additional submittals required are specified in the Divisions 02 through 17 specification Sections.

- ***********************************************************************************************************************

    - LEED for Schools Documentation Submittals:

      - Credit EA 5:  Product data and wiring diagrams for sensors and data collection system used to provide continuous metering of building energy-consumption during required performance period.

- ***********************************************************************************************************************

- NOTE TO SPECIFIER:  Retain first option in Subparagraphs 3 & 4 below if retaining 20 percent; delete if retaining 10 percent.

- ************************************************************************************************************************

      - Credit MR 2.1 [and Credit MR 2.2]:  Waste reduction progress reports complying with Division-1 Section “Construction Waste Management”.

      - Credit EQ 3.1:

        - Construction indoor-air-quality management plan.

        - Product data for temporary filtration media.

        - Product data for filtration media used during occupancy.

        - Construction Documentation Photos:  Refer to Section 01320-Construction Progress Documentation

      - Credit EQ 3.2:

        - Report from testing and inspecting agency indicating results of indoor-air-quality testing and documentation showing compliance with indoor-air-quality testing procedures and requirements.

      - Credit MR 4.1[and Credit MR 4.2]:  Product data and certification letter indicating percentages by weight of post-consumer and pre-consumer recycled content for products having recycled content.  Include statement indicating costs for each product having recycled content.

      - Credit MR 5.1[and Credit MR 5.2]:  Product data for regional materials indicating location and distance from Project of material manufacturer and point of extraction, harvest, or recovery for each raw material.  Include statement indicating cost for each regional material and the fraction by weight that is considered regional.

      - Credit MR 7:  Product data and chain-of-custody certificates for products containing certified wood.  Include statement indicating cost for each certified wood product.

      - Credit EQ 4:  Product data for adhesives and sealants used inside the weatherproofing system indicating VOC content of each product used.  Indicate VOC content in g/L calculated according to 40 CFR59, Subpart D.

      - Credit EQ 4:  Product data for paints and coatings used inside the weatherproofing system indicating VOC content of each product used.  Indicate VOC content in g/L calculated according to 40 CFR 59, Subpart D.

      - Credit EQ 4:  Product data for products containing composite wood or agrifiber products or wood glues indicating that they do not contain urea-formaldehyde resin.

- PRODUCTS

  - RECYCLED CONTENT OF MATERIALS

- ************************************************************************************************************************NOTE TO SPECIFIER:  Retain first option in paragraph below if retaining 20 percent; delete if retaining 10 percent. Select minimum percent of recycled content for the Project.

- ************************************************************************************************************************

    - Credit MR 4.1[and Credit MR 4.2]:  Provide building materials with recycled content such that post-consumer recycled content plus one-half of pre-consumer recycled content constitutes a minimum of [10] [20] percent of cost of materials used for Project.

      - Cost of post-consumer recycled content of an item shall be determined by dividing weight of post-consumer recycled content in the item by total weight of the item and multiplying by cost of the item.

      - Cost of pre-consumer recycled content of an item shall be determined by dividing weight of pre-consumer recycled content in the item by total weight of the item and multiplying by cost of the item.

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Furniture may be used toward LEED for Schools Credit MR 3.1 through 7 if it is included consistently in calculations for all of them.

- ***********************************************************************************************************************.

      - Do not include [furniture,] [plumbing,] mechanical and electrical components, and specialty items such as elevators and equipment in the calculation.

  - REGIONAL MATERIALS

- *********************************************************************************************************************** NOTE TO SPECIFIER:  Retain first option in paragraph below if retaining 20 percent; delete if retaining 10 percent. Select minimum percent of recycled content for the Project.

- ************************************************************************************************************************

    - Credit MR 5.1[and Credit MR 5.2]:  Provide a minimum of [10] [20] percent of building materials (by cost) that are regional materials.

  - LOW-EMITTING MATERIALS

    - Credit EQ 4:  For field applications that are inside the weatherproofing system, use adhesives and sealants that comply with VOC limits specified in individual sections.

    - Credit EQ 4:  For field applications that are inside the weatherproofing system, use paints and coatings that comply with the VOC limits specified in individual sections.

    - Credit EQ 4:  Do not use composite wood or agrifiber products or adhesives that contain urea-formaldehyde resin.

- EXECUTION

  - CONSTRUCTION ACTIVITY POLLUTION PREVENTION

    - Prerequisite SS 1: Submit copies of the project drawings to document the erosion and sedimentation control measures implemented on the site.

    - Provide confirmation regarding the compliance path taken by the project (NPDES Compliance or Florida Department of the Environment).

  - COMMISSIONING

    - Refer to Section 01810-Commissioning.

  - REFRIGERANT REMOVAL

    - Prerequisite EA 3:  Remove CFC-based refrigerants from existing HVAC&R equipment indicated to remain and replace with refrigerants that are not CFC based.  Replace or adjust existing equipment to accommodate new refrigerant as described in Division 15 Sections.

    - Credit EA 4:  Remove clean-agent fire-extinguishing agents that contain HCFCs or halons and replace with agent that does not contain HCFCs or halons. See Division 10 Section “Fire Protection Specialties" for additional requirements.

  - MEASUREMENT AND VERIFICATION

- ************************************************************************************************************************NOTE TO SPECIFIER:  Retain one of the two options in the first paragraph below.

- ************************************************************************************************************************

    - Credit EA 5:  Implement measurement and verification plan consistent with [Option B:  Energy Conservation Measure Isolation] [Option D:  Calibrated Simulation, Savings Estimation Method 2] in the EVO's "International Performance Measurement and Verification Protocol (IPMVP) Volume III:  Concepts and Options for Determining Energy Savings in New Construction,"  and as further defined by the following:

      - <Insert measurement and verification plan design team submitted for credit>.

    - If not already in place, install metering equipment to measure energy usage.  Monitor, record, and trend log measurements.

    - Evaluate energy performance and efficiency by comparing actual to predicted performance.

    - Measurement and verification period shall cover at least one year of post construction occupancy.

  - MATERIALS AND RESOURCES

    - Prerequisite MR 1:  Verify that areas designated as recycling collection areas have been provided, per contract requirements, to meet the needs of the project.

  - INDOOR ENVIRONMENTAL QUALITY

    - Prerequisite EA 1:  All Mechanical ventilation systems shall be installed in accordance with the minimum requirements of ASHRAE 62.1-2004, Sections 4 through 7 or the Florida Building Code whichever is more stringent.

  - CONSTRUCTION INDOOR-AIR-QUALITY MANAGEMENT

    - Credit EQ 3.1:  Comply with SMACNA's "SMACNA IAQ Guideline for Occupied Buildings under Construction."

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Coordinate first subparagraph below with Division-1 Section "Temporary Facilities and Controls."  Identify air handlers and associated return-air inlets authorized by Owner for use during construction period.

- ************************************************************************************************************************

      - If Owner authorizes use of permanent heating, cooling, and ventilating systems during construction period as specified in Division-1 Section "Temporary Facilities and Controls," install filter media having a MERV per Division-15.

      - Replace all air filters immediately prior to occupancy.

    - Credit EQ 3.2: [Comply with one of the following requirements:]

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Retain one or more of three subparagraphs below. Retain option in paragraph above if retaining more than one subparagraph below. Project's mechanical engineer of record should verify that HVAC system design and equipment indicated are capable of delivering flush-out indicated.  Provide HVAC system and equipment operating information necessary to achieve credit.  If Project HVAC systems and equipment cannot suit requirement, consider requiring temporary systems and equipment.

- ************************************************************************************************************************

      - After construction ends, prior to occupancy and with all interior finishes installed, perform a building flush-out per Section 01354-Construction Indoor Air Quality Management.

      - If occupancy is desired prior to flush-out completion, the space may be occupied per Section 01354-Construction Indoor Air Quality Management.

      - Air-Quality Testing:

        - Conduct baseline indoor-air-quality testing, after construction ends and prior to occupancy, per Section 01354-Construction Indoor Air Quality Management.

END OF SECTION