# SECTION 01520 (01 52 00)

## CONSTRUCTION FACILITIES

- GENERAL

  - SECTION INCLUDES

    - Field Offices, Sheds, and Storage.

    - First Aid.

    - Sanitary Facilities.

  - RELATED DOCUMENTS

    - Section 01510-Temporary Utilities.

  - REQUIREMENTS

    - Construction of Field Offices and Sheds:

        - Structurally sound, weather tight, with floor raised aboveground.

      - Temperature Transmission Resistance.  Compatible with human occupancy and storage.

      - At Contractor’s option, portable or mobile building may be used.  Mobile homes, when used, shall be modified for field or office use.

    - Year Round Hurricane Precautions:  Provide appropriate tie-downs for temporary trailers, field offices, and sheds to be capable of withstanding wind velocity pressures according to ASCE 7.

  - QUALITY ASSURANCE

    - Regulations:  Comply with industry standards and applicable laws and regulations if authorities having jurisdiction, including but not limited to:

      - The Florida Building Code

      - State Requirements for Educational Facilities (SREF) requirements.

      - Health and safety regulations.

      - Police, Fire Department and Rescue Squad requirements.

      - Environmental protection regulations.

    - Refer to "Guidelines for Bid Conditions for Temporary Job Utilities and Services", prepared jointly by AGC and ASC, for industry recommendations.

    - Inspections:  Arrange for inspection and testing of temporary facilities by BCI and other authorities having jurisdiction under provisions of Section 014450-Quality Control.

  - CONDITIONS OF USE

    - Keep temporary services and facilities clean and neat in appearance.

    - Operate in a safe and efficient manner.

    - Take necessary fire prevention measures.

    - Do not overload facilities, or permit them to interfere with progress.

    - Do not allow dangerous or unsanitary conditions, or public nuisances to develop or persist on the site.

  - USE OF FACILITIES

    - Permanent and/or existing facilities:  Do not use for field offices or for storage. (except as authorized by Article 3.4 Operation, Termination and Removal.)

    - PRODUCTS

  - TEMPORARY BUILDINGS

    - Construction:

      - Portable or mobile buildings, or buildings constructed with floors raised above ground, securely fixed to foundations, with steps and landings at entrance doors.

      - Construction:  Structurally sound, secure, weather tight enclosures for office and storage spaces. Maintain during progress of Work; remove at completion of Work.

      - Temperature Transmission Resistance of Floors, Walls, and Ceilings: Compatible with occupancy and storage requirements.

      - Exterior Materials:  Weather resistant, finished in one color acceptable to Owner.

      - Interior Materials in Offices: Sheet type materials for walls and ceilings, pre-finished or painted; resilient floors and bases.

      - Lighting for Offices:  50 ft-C (538 lx) at desktop height, exterior lighting at entrance doors.

      - Fire Extinguishers: Appropriate type fire extinguisher at each office and each storage area as specified below.

      - Interior Materials in Storage Sheds:  As required to provide specified conditions for storage of products.

      - Provide incombustible construction for offices, storage and sheds located within the construction area, or within 30 feet of building lines.  Comply with requirements of NFPA 241.

      - Security Enclosure and Lockup:  Provide locking entrances to prevent unauthorized entrance, vandalism, theft and similar violations of security.

    - Environmental Control:

      - Heating, Cooling, and Ventilating for Offices:  Automatic equipment shall maintain 68 degrees F heating and 76 degrees F cooling.

      - Storage Spaces:  Provide heating, cooling and ventilation as needed to maintain products in accordance with Contract Documents and manufacturer’s recommendation; adequate lighting for maintenance and inspection of products.

    - Contractor Office and Facilities

      - Size:  For Contractor's needs and to provide space for Progress Meetings.

      - Telephone: As specified in Section 01510-Temporary Utilities.

      - Fax Machine.

      - Furnishings in Meeting Area:  Conference table and chairs to seat at least eight persons; racks and files for Contract Documents, submittals, and Project Record Documents.

      - Other Furnishings: Contractor's option.

      - Equipment:

        - 15 adjustable band protective helmets for visitors.

        - One 10 inch outdoor weather thermometer.

        - Other office related equipment at Contractor's option.

    - Owner and Project Consultant Office.

      - One office space for shared by the use of Owner and Project Consultant , with separate entrance doors with new lock and two keys per door.

      - Area:  Minimum office space 150 sq ft, minimum dimension of 8 ft.

      - Windows:  Minimum three; minimum total area of 10 percent of floor area, with operable sash and insect screens. Location shall provide views of construction area.

      - Electrical Distribution Panel:  Two circuits minimum, 110 volt, 60 Hz service.

      - Minimum four 110-volt duplex convenience outlets, one on each wall.

      - Telephone:  As specified in Section 01510-Temporary Utilities.  Two lines required for phones and Fax/Modem.

      - Fax Machine.

      - Sanitary Facilities:  Drinking fountain or electric water cooler and private lavatory-toilet facilities.

      - Furnishings:

        - One desk, 54 inch x 30 inch, with three drawers.

        - One drafting table, 36 inch x 72 inch surface, with one equipment drawer, and a full width parallel straight edge.

        - One metal, double-door storage cabinet under table.

        - Plan rack to hold the Drawings, Shop Drawings, and Record Documents.

        - One standard four-drawer legal-size metal filing cabinet with locks and two keys per lock.

        - Six linear feet of metal bookshelves.

        - Two swivel armchairs.

        - One drafting table stool.

        - One tack board, 36 inch x 30 inch.

        - One wastebasket per desk or table.

      - Equipment.

        - Contractor shall setup and maintain all computer and business equipment in Owner’s office throughout the duration of construction, through final completion.

    - Provide black and white color copier of 40 pages per minute copying capacity or better, with sorter, reduction & enlarging capability, automatic stapler and three-hole punch.  Use of copy machine shall be shared by Contractor, Owner, and Project Consultant.

    - Sanitary Facilities:  Include temporary toilets, wash facilities and drinking water fixtures.  Comply with regulations and health codes for the type, number, location, operation and maintenance of fixtures and facilities.  Install where facilities will best serve the Project's needs after coordinating location with Owner and Project Consultant.

      - Temporary Toilet Units:  Provide self-contained single occupant toilet units of the chemical, aerated recirculation, or combustion type, properly vented and fully enclosed with a glass fiber reinforced polyester shell or similar nonabsorbent material.  Shield toilets to ensure privacy.  Use of pit type privies will not be permitted.

      - Provide toilet tissue, paper towels, paper cups and similar disposable materials for each facility, including toilet facilities provided in temporary office trailers.  Provide covered waste containers for used material.

      - Existing Toilets:  Use of the Owner's existing toilet facilities:  Not permitted.

    - Wash Facilities:  Install wash facilities supplied with potable water at convenient locations for personnel involved in handling materials that require wash-up for a healthy and sanitary condition.  Dispose of drainage properly.  Supply cleaning compounds appropriate for each condition.

    - Protective Equipment:  Provide safety showers, eyewash fountains and similar facilities for convenience, safety and sanitation of personnel.

    - Drinking Water Facilities:  Provide electrically cooled containerized bottled water type drinking water units, including paper supply.  Provide drinking water at 45 to 55 degree Fahrenheit.  Provide other temporary drinking facilities as specified elsewhere or as appropriate to the site and the Work.

    - First Aid Supplies:  Provide in adequate quantity and in locations convenient to principal areas of the Work.  Comply with governing regulations.

    - Fire Extinguishers:  Provide hand carried, portable UL-rated; class "A" fire extinguishers for each temporary office and each similar space.  In other locations provide hand carried, portable, UL-rated, class "ABC" dry chemical extinguishers, or a combination of extinguishers of NFPA recommended classes for the exposures.

    - Covered Walkway:  Erect a structurally adequate protective covered walkway for passage of persons along pedestrian ways adjacent to the Work.  Coordinate with Project Consultant and Owner to resolve design and relationship of walkway to entrance gates, building entrances, other facilities, playgrounds and obstructions.  Comply with regulations of authorities having jurisdiction.

      - Construct using scaffold or shoring framing, waterproofed wood plank overhead decking, protective plywood enclosure walls, handrails, barricades, warning signs, lights, safe and well- drained walkways and similar provisions for protection and safe passage.

      - Extend the back wall beyond the structure to complete the enclosure fence.

      - Paint and maintain in a manner acceptable to the Owner and Project Consultant.

    - Storage and Fabrication Sheds:  Provide fully enclosed storage and fabrication sheds, sized, furnished and equipped to accommodate materials and equipment involved, including temporary utility service.

    - EXECUTION

  - INSTALLATION

    - Install office spaces ready for occupancy within 15 days after Notice to Proceed.

    - Employee Residential Occupancy:  Not allowed on Owner's Property.

    - Use qualified personnel for installation of temporary facilities.  Locate facilities as indicated within the Contract Documents.  For facilities not so indicated: locate where they will serve the Project adequately and result in minimum interference with performance of the Work.  Relocate and modify facilities as required.

    - Provide each facility ready for use when needed to avoid delay.  Maintain and modify as required.  Do not remove until facilities are no longer needed.

    - Fill and grade sites for temporary structures to provide surface drainage.

    - Construct temporary field offices and storage sheds on proper foundations.  Provide connections for utility services.

      - Secure portable or mobile buildings, when used, against break-ins and hurricane requirements per code.

      - Provide slip resistant steps and landings at entrance doors.

    - Mount thermometer at a convenient outside continuously shaded area.

  - PROTECTION OF TEMPORARY AND OTHER FACILITIES

    - Temporary Fire Protection:  Provide and maintain temporary fire protection facilities of the types needed to protect against reasonably predictable and controllable fire losses.  Comply with NFPA 10 "Standard for Portable Fire Extinguishers," and NFPA 241 "Standard for Safeguarding Construction, Alterations and Demolition Operations."

    - Store combustible materials in containers in fire safe locations.

    - Maintain unobstructed access to fire extinguishers, fire hydrants, temporary fire protection facilities, stairways and other access routes for fighting fires.  Prohibit smoking in fire hazard exposure areas.

    - Provide supervision of welding operations, combustion type temporary heating units, and similar sources of fire ignition.

    - Permanent Fire Protection:  At the earliest feasible date in each area of the Project, complete installation of the applicable permanent fire protection facility, including connected services, and place into operation and use.  Instruct key personnel on use of facilities.

  - ENVIRONMENTAL PROTECTION

    - Provide protection, operate temporary facilities and conduct construction in ways and by methods that comply with environmental regulations, and minimize the possibility that air, waterways and subsoil might be contaminated or polluted, or that other undesirable effects might result.

    - Use of tools and equipment, which produce harmful noise, shall be prohibited.  Restrict use of noise making tools and equipment to hours that shall minimize complaints from persons or firms near the site, or site occupants.

  - OPERATION, TERMINATION AND REMOVAL

    - Supervision:  Enforce strict discipline in use of temporary facilities.  Limit availability of temporary facilities to essential and intended uses to minimize waste and abuse.

    - Maintenance:  Maintain temporary facilities in good operating condition until removal.  Protect from damage by heat, humidity, and similar elements including brief periods of unexpected cold conditions.

      - Maintain operation of temporary, heating, cooling, humidity control, ventilation and similar facilities on a 24-hour day basis where required to achieve indicated results and to avoid possibility of damage.

      - Keep the office clean and orderly for use for small progress meetings.

      - Provide daily janitorial services for offices; periodic cleaning and maintenance for office and storage areas.

      - Maintain approach walks free of mud, water, and debris.

    - Termination and Removal:

      - Unless the Project Consultant requests that it be maintained longer, remove each temporary facility when the need has ended or when replaced by authorized use of a permanent facility, or no later than Substantial Completion.  Ensure removal of footings, foundations and other subsurface or underground construction.

      - Complete and restore permanent construction that may have been delayed because of interference with the temporary facility.

      - Repair damaged Work, clean exposed surfaces and replace construction that cannot be satisfactorily repaired.

      - Personnel remaining after Substantial Completion will be permitted to use permanent facilities, under conditions acceptable to and with the pre-agreement of the Owner.

      - Prior to Substantial Completion, clean and renovate permanent facilities that have been used during the construction period.

END OF SECTION