# SECTION 01550 (01 55 00)

## VEHICULAR ACCESS AND PARKING

- GENERAL

  - SECTION INCLUDES

    - Preparation.

    - Flag Persons.

    - Flares and Lights.

    - Traffic Signs and Signals.

    - Haul Routes.

    - Access Roads.

    - Construction Parking Controls.

    - Parking.

    - Existing Pavements and Parking Areas.

    - Permanent Pavements and Parking Facilities.

    - Maintenance.

    - Removal and Repair.

    - Mud From Site Vehicles.

  - RELATED SECTIONS

    - Section 01110-Project Management and Coordination:  Project coordination.

    - Section 01510-Temporary Construction:  Temporary construction.

    - Section 01520-Construction Facilities:  Temporary buildings and other temporary facilities.

    - Section 01580-Project Identification:  Traffic directional and control signage.

    - Section 02200-Earthwork.

    - Section 02747-Asphaltic Concrete Paving for Parking and Drives.

    - PRODUCTS

  - MATERIALS

    - Temporary Construction:  Contractor’s option.

    - Earthwork, Paving Base and Topping that Will Become Permanent Construction:  As specified in Specification Sections in Divisions 2 through 16.

  - SIGNS, SIGNALS, and DEVICES

    - Post Mounted and Wall Mounted Traffic Control and Informational Signs:  Specified in Section 01580-Project Identification and Section 02890-Signs and Signalization.

    - Temporary Traffic Control Signals:  As approved by local jurisdictions.

    - Traffic Cones and Drums, Flares and Lights:  As approved by local jurisdictions.

    - Flag Person Equipment:  As required by local jurisdictions.

    - EXECUTION

  - PREPARATION

- ************************************************************************************************************************

- NOTE TO SPECIFIER:  Select from the following.

- ************************************************************************************************************************

    - Clear Areas:  Provide surface [and storm] drainage of road, parking, area premises, and adjacent areas.

  - FLAG PERSONS

    - Provide trained and equipped flag persons to regulate traffic when construction operations or traffic encroach on public traffic lanes.

  - FLARES and LIGHTS

    - Use flares and lights during hours of low visibility to delineate traffic lanes and to guide traffic.

  - TRAFFIC SIGNS and SIGNALS

    - At approaches to site and on site, install at crossroads, detours, parking areas, and other amenities elsewhere as needed to direct construction and affected public traffic.

    - Install and operate traffic control signals as necessary to direct and maintain orderly flow of traffic in areas under Contractor’s control, and areas affected by Contractor’s operations.

    - Relocate as Work progresses, to maintain effective traffic control.

  - HAUL ROUTES

    - Consult with authority having jurisdiction, establish public thoroughfares to be used for haul routes and site access.

- ****** [OR] ******

    - A.	Drawings indicate haul routes designated by authorities for use of construction traffic.

    - Confine construction traffic to designated haul routes.

    - Provide traffic control at critical areas of haul routes to regulate traffic, and to minimize interference with public traffic.

  - ACCESS ROADS

    - Construct temporary access roads from public thoroughfares to serve construction area, of a width and load bearing capacity to provide unimpeded traffic for construction purposes.

    - Construct temporary bridges and culverts to span low areas and allow unimpeded drainage.

    - Extend and relocate, as Work progress requires, provide detours as necessary for unimpeded traffic flow.

    - Location as indicated on Drawings.

    - Provide unimpeded access for emergency vehicles.  Maintain 20-foot width driveways with turning space between and around combustible materials.

    - Provide and maintain access to fire hydrants and control valves free of obstructions.

  - CONSTRUCTION PARKING CONTROL

    - Control vehicular parking to prevent interference with public traffic and parking, access by emergency vehicles, and Owner's operations.

    - Monitor parking of construction personnel's vehicles in existing facilities.  Maintain vehicular access to and through parking areas.

    - Prevent parking on or adjacent to access roads or in non-designated areas.

  - PARKING

- ************************************************************************************************************************

- NOTE TO SPECIFIER:  Select from the following.

- ************************************************************************************************************************

    - [Arrange for] [Provide] [Construct] temporary parking areas to accommodate use of construction personnel.

    - When site space is not adequate, provide additional off-site parking.

    - Locate as indicated on Drawings.

  - EXISTING PAVEMENTS and PARKING AREAS

    - Use of designated existing on-site streets and driveways used for construction traffic is permitted.  Tracked vehicles not allowed on paved areas.

    - Use of designated areas of existing parking facilities used by construction personnel is not permitted.

    - Do not allow heavy vehicles or construction equipment in parking areas.

  - PERMANENT PAVEMENTS and PARKING FACILITIES

    - The base for permanent roads and parking areas may be used for construction traffic.

    - Avoid traffic loading beyond paving design capacity.  Tracked vehicles not allowed.

  - MAINTENANCE

    - Maintain traffic and parking areas in a sound condition free of excavated material, construction equipment, Products, mud or other debris.

    - Maintain existing and permanent paved areas used for construction; promptly repair breaks, potholes, low areas, standing water, and other deficiencies, to maintain paving and drainage in original, or specified, condition.

  - REMOVAL, REPAIR

    - Remove temporary materials and construction when permanent paving is usable.

    - Remove underground work and compacted materials to a depth of 2 feet; fill and grade site as specified.

    - Repair existing or permanent facilities damaged by use, to original or specified condition, whichever is most stringent.

    - Remove equipment and devices when no longer required.

    - Repair damage caused by installation.

    - Remove post settings to a depth of 2 feet.

  - MUD FROM SITE VEHICLES

    - Provide means of removing mud from vehicle wheels before entering streets.

END OF SECTION