# SECTION 01570 (01 57 00)

## TEMPORARY CONTROLS

- GENERAL

- SECTION INCLUDES

    - Erosion and Sediment Control.

    - Pest Control.

    - RELATED DOCUMENTS

    - Section 01110-Summary of Work:  Work sequence, Owner occupancy.

    - Section 01310-Project Management and Coordination:  Project coordination.

    - Section 01520-Construction Facilities:  Temporary buildings and other temporary facilities.

    - Section 01530-Temporary Construction:  Temporary construction.

    - Section 01560-Temporary Barriers and Enclosures.

    - Section 01740-Cleaning.

    - WATER CONTROL

    - Grade site to drain. Maintain excavations free of water.  Provide, operate, and maintain pumping equipment.

    - Protect site from puddling or running water.

    - Provide water barriers as required to protect site from soil erosion.

    - Dewatering Facilities and Drains:

    - For temporary drainage and dewatering facilities and operations not directly associated with construction activities included under individual specification sections, comply with dewatering requirements of applicable Division 2 Sections.

    - Where feasible, utilize the same facilities.

    - Maintain the site, excavations and construction free of water.

    - Comply with environmental limitations on dewatering as imposed by Broward County, the South Florida Water Management District, or other applicable jurisdictional authority.

    - EROSION AND SEDIMENT CONTROL

    - Plan and execute construction by methods to control surface drainage from cuts and fills, from borrow and waste disposal areas.  Prevent erosion and sedimentation.

    - Minimize amount of bare soil exposed at one time.

    - Provide temporary measures such as berms, dikes, and drains, to prevent water flow.

    - Construct fill and waste areas by selective placement to avoid erosive surface silts or clays.

    - Provide earthen embankments and similar barriers in and around excavations and subgrade construction sufficient to prevent flooding by runoff of storm water from heavy rains.

    - Periodically inspect earthwork to detect evidence of erosion and sedimentation; promptly apply corrective measures.

    - PEST CONTROL

    - Provide methods, means, and facilities to prevent pests, insects and rodents from infesting the area of the Work and adjacent premises.

    - Prior to construction of footings, foundations or slabs: Retain a local exterminator or pest control company to:

    - Recommend practices to minimize attraction and harboring of rodents, roaches and other pests.

    - Provide extermination and control procedures at regular intervals so the Project will be free of pests and their residues at Substantial Completion.

    - Ensure that pest, insect and rodent control operations are conducted in a lawful manner using environmentally safe materials.

    - Engage in exterminator to make final eradication of rodents, insects, and other pests as specified in Section 01570-Temporary Controls.

    - Repeat final treatments as necessary until Owner occupancy to ensure rodent, insect and pest free facilities.

    - Refer to Division 2 requirements for termiticide treatments and other pest control requirements.

    - REMOVAL OF UTILITIES, FACILITIES, AND CONTROLS

    - Remove temporary above grade or buried controls as warranted by the progress of the Work or prior to Substantial Completion.

    - Remove underground installations to a minimum depth of 2 feet.  Grade site as noted.

    - Clean and repair damage caused by installation or use of temporary work.

    - Restore existing facilities used during construction to original condition at start of work or as specified elsewhere in the Contract Documents.

    - Restore permanent facilities used during construction to specified condition.

    - PRODUCTS-Not Used

    - EXECUTION-Not Used

END OF SECTION