# SECTION 01572 (SECTION 02 42 00)

## CONSTRUCTION WASTE MANAGEMENT

- GENERAL

- SECTION INCLUDES

    - Administrative and procedural requirements for the following:

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Make selections below to suit Project.

- ***********************************************************************************************************************

      - Salvaging nonhazardous waste.

      - Recycling nonhazardous waste.

      - Disposing of nonhazardous waste.

  - RELATED SECTIONS

  - Section 02070-Minor Demolition for Remodeling.

  - Section 02110-Site Clearing.

  - Section 02222-Building Demolition.

  - DEFINITIONS

    - Construction Waste:  Building and site improvement materials and other solid waste resulting from demolition, construction, remodeling, renovation, or repair operations.  Construction waste includes packaging.

    - Disposal:  Removal off-site of construction waste and subsequent sale, recycling, reuse, or deposit in landfill or incinerator acceptable to authorities having jurisdiction.

    - Recycle:  Recovery of construction waste for subsequent processing in preparation for reuse.

    - Salvage:  Recovery of waste and subsequent sale or reuse in another facility.

    - Salvage and Reuse:  Recovery of waste and subsequent incorporation into the Work.

    - PERFORMANCE REQUIREMENTS

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Make selection below to suit Project.

- ***********************************************************************************************************************

    - General:  Develop waste management plan that results in landfill diversion through salvage and recycling of [50] [75] <Insert number> percent by weight of total construction waste generated by the Work.

- SUBMITTALS

- ************************************************************************************************************************NOTE TO SPECIFIER:  Make selection below to suit Project.

- ***********************************************************************************************************************

    - Waste Management Plan:  Submit [3] <Insert number> copies of plan not less than 10 days before the Pre-Construction meeting.

    - Waste Reduction Progress Reports:  Concurrent with each Application for Payment, submit [three] <Insert number> copies of report.  Include the following information:

      - Material category.

      - Generation point of waste.

      - Total quantity of waste in tons.

      - Quantity of waste salvaged, both estimated and actual in tons.

      - Quantity of waste recycled, both estimated and actual in tons.

      - Total quantity of waste recovered (salvaged plus recycled) in tons.

      - Total quantity of waste recovered (salvaged plus recycled) as a percentage of total waste.

- ************************************************************************************************************************NOTE TO SPECIFIER:  Make selection below to suit Project.

- ***********************************************************************************************************************

    - Waste Reduction Calculations:  Before request for Substantial Completion, submit [three] <Insert number> copies of calculated end-of-Project rates for salvage, recycling, and disposal as a percentage of total waste generated by the Work.

    - Recycling and Processing Facility Records:  Indicate receipt and acceptance of recyclable waste by recycling and processing facilities licensed to accept them.  Include manifests, weight tickets, receipts, and invoices.

    - Landfill and Incinerator Disposal Records:  Indicate receipt and acceptance of waste by landfills and incinerator facilities licensed to accept them.  Include manifests, weight tickets, receipts, and invoices.

    - LEED Submittal:  LEED letter template for Credit MR 2.1[and 2.2], signed by Contractor, tabulating total waste material, quantities diverted and means by which it is diverted, and statement that requirements for the credit have been met.

    - PRE-CONSTRUCTION MEETING

    - Waste Management Pre-Construction meeting:  Shall not occur without Waste Management Plan accepted by the Project Consultant and the Owner.  After award of Contract and prior to the commencement of the Work, the Contractor must schedule and conduct meeting with at a minimum the Owner and Project Consultant(s) at Project site to discuss the Waste Management Plan and to review methods and procedures related to waste management including, but not limited to the following:

      - Review and discuss Waste Management Plan including responsibilities of the Contractor.

      - Review requirements for documenting quantities of each type of waste and it’ disposition.

      - Review and finalize procedures for materials separation and verify availability of containers and bins needed to avoid delays.

      - Review procedures for periodic waste collection and transportation to recycling and disposal facilities.

      - Review waste management requirements for each trade.

      - WASTE MANAGEMENT PLAN

    - General:  Develop plan consisting of waste identification and waste reduction work plan. Include separate sections in plan for construction waste.  Indicate quantities by weight or volume, but use same units of measure throughout waste management plan.

    - Waste Identification:  Identify the materials to be diverted from disposal.  Indicate anticipated types and quantities of construction waste generated by the Work.  Include estimated quantities and assumptions for estimates.

    - Identify whether materials are sorted on-site or comingled.

    - Designate a specific area(s) on the construction site for segregated or comingled collection of recyclable materials and track recycled efforts throughout the construction process.

    - Identify construction haulers and recyclers to handle the designated materials.

    - Identify location of receiving agent.

    - Waste Reduction Work Plan:  List each type of waste and whether it will be salvaged, recycled, or disposed of in landfill or incinerator.  Include points of waste generation, total quantity of each type of waste, quantity for each means of recovery, and handling and transportation procedures.

      - Salvaged Materials for Reuse:  For materials that will be salvaged and reused in this Project, describe methods for preparing salvaged materials before incorporation into the Work.

      - Recycled Materials:  Include list of local receivers and processors and type of recycled materials each will accept.  Include names, addresses, and telephone numbers.

      - Disposed Materials:  Indicate how and where materials will be disposed of.  Include name, address, and telephone number of each landfill and incinerator facility.

      - Handling and Transportation Procedures:  Include method that will be used for separating recyclable waste including sizes of containers, container labeling, and designated location on Project site where materials separation will be located.

- PRODUCTS (NOT USED)

- EXECUTION

- PLAN IMPLEMENTATION

    - General:  Implement waste management plan as accepted by Project Consultant and Owner.  Acceptance by Project Consultant and Owner does not relieve the Contractor of responsibility of compliance with applicable environmental regulations.  Provide handling, containers, storage, signage, transportation, and other items as required to implement waste management plan during the entire duration of the Contract.

    - Waste Management Coordinator:  Designate a waste management coordinator to be responsible for implementing, monitoring, and reporting status of waste management work plan.  Coordinator shall be present at Project site full time for duration of Project.

    - Training:  Train workers, subcontractors, and suppliers on proper waste management procedures, as appropriate for the Work occurring at Project site.

      - Distribute waste management plan to everyone concerned within minimum of 3 days of accepted submittal return.

      - Distribute waste management plan to entities when they first begin work on-site.  Review plan procedures and locations established for salvage, recycling, and disposal.

    - Site Access and Temporary Controls:  Conduct waste management operations to ensure minimum interference with roads, streets, walks, walkways, and other adjacent occupied and used facilities.

      - Designate and label specific areas on Project site necessary for separating materials that are to be salvaged, recycled, and reused.

      - Comply with Division-1 Section 01560-Barriers and Enclosures for controlling dust and dirt, environmental protection, and noise control.

      - SALVAGING CONSTRUCTION WASTE

    - Salvaged Items for Reuse in the Work:

      - Clean salvaged items.

      - Pack or crate items after cleaning.  Identify contents of containers.

      - Store items in a secure area until installation.

      - Protect items from damage during transport and storage.

      - Install salvaged items to comply with installation requirements for new materials and equipment.  Provide connections, supports, and miscellaneous materials necessary to make items functional for use indicated.

    - Salvaged Items for Owner's Use:

      - Clean salvaged items.

      - Pack or crate items after cleaning.  Identify contents of containers.

      - Store items in a secure area until delivery to Owner.

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Make selection from paragraph below to fit Project requirements.

- ***********************************************************************************************************************

      - Transport items to Owner's storage area [on-site] [off-site] [designated by Owner].

      - Protect items from damage during transport and storage.

      - RECYCLING  CONSTRUCTION WASTE, GENERAL

    - General:  Recycle paper and beverage containers used by on-site workers.

    - Options:  Sort on-site or comingle.

    - RECYCLING CONSTRUCTION WASTE

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Paragraphs below in this Article are examples only; retain, delete, or add Project specific disposal, cleanup, or removal requirements to suit Project or recycling facilities.

- ***********************************************************************************************************************

    - Asphaltic Concrete Paving.

    - Concrete:  Remove reinforcement and other metals from concrete and sort with other metals.

    - Masonry:  Remove metal reinforcement, anchors, and ties from masonry and sort with other metals.

    - Wood Materials.

      - Separate lumber, engineered wood products, panel products, and treated wood materials.

      - Clean Cut-Offs of Lumber:  Grind or chip into small pieces.

      - Clean Sawdust:  Bag sawdust that does not contain painted or treated wood.

    - Metals:  Separate metals by type.

    - Asphalt Shingle Roofing:  Remove and dispose of nails, staples, and accessories.

    - Gypsum Board:

      - Remove edge trim and sort with other metals.  Remove and dispose of fasteners.

      - Stack large clean pieces on wood pallets and store in a dry location.

        - Clean Gypsum Board:  Grind scraps of clean gypsum board using small mobile chipper or hammer mill.  Screen out paper after grinding.

    - Acoustical Ceiling Panels and Tile.  Separate suspension system, trim, and other metals from panels and tile and sort with other metals.

    - Carpet and Pad if applicable.

    - Equipment:  Drain tanks, piping, and fixtures.  Seal openings with caps or plugs.  Protect equipment from exposure to weather.

    - Plumbing Fixtures:  Separate by type and size.

    - Piping.

    - Lighting Fixtures:  Separate lamps by type and protect from breakage.

    - Electrical Devices:  Separate switches, receptacles, switchgear, transformers, meters, panel boards, circuit breakers, and other devices by type.

    - Conduit.

    - Packaging:

      - Cardboard and Boxes:  Break down packaging into flat sheets.  Bundle and store in a dry location.

      - Polystyrene Packaging:  Separate and bag materials.

      - Pallets:  As much as possible, require deliveries using pallets to remove pallets from Project site.  For pallets that remain on-site, break down pallets into component wood pieces and comply with requirements for recycling wood.

      - Crates:  Break down crates into component wood pieces and comply with requirements for recycling wood.

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Make selection from below to fit Project requirements.

- ***********************************************************************************************************************

    - Site-Clearing Wastes:  Chip brush, branches, and trees [on-site] [at landfill facility].

    - DISPOSAL OF WASTE

- ***********************************************************************************************************************NOTE TO SPECIFIER:  Add other specific disposal, cleanup, and removal requirements to fit Project requirements.

- ***********************************************************************************************************************

    - General:  Except for items or materials to be salvaged, recycled, or otherwise reused, remove and transport waste materials from Project site and legally dispose of them in a landfill or incinerator acceptable to authorities having jurisdiction and the city where the project is located..

      - Except as otherwise specified, do not allow waste materials that are to be disposed of accumulate on-site.

      - Remove and transport debris in a manner that will prevent spillage on adjacent surfaces and areas.

    - Burning:  Do not burn waste materials.

    - Disposal:  Transport waste materials off Owner's property and legally dispose of them.

END OF SECTION