# SECTION 01580 (01 58 00)

## PROJECT IDENTIFICATION

- GENERAL

  - SECTION INCLUDES

    - Project identification sign.

    - Project informational signs.

    - Maintenance.

    - Removal.

  - RELATED SECTIONS

    - Section 01110-Summary of Work:  Contractor’s use of premises, Owner occupancy.

    - Section 01520-Construction Facilities:  Field offices and sheds.

    - Section 01550-Vehicular Access and Parking.

  - QUALITY ASSURANCE

    - Design sign and structure to withstand wind velocity as required by the latest edition of The Florida Building Code with amendments, and SREF.

    - Sign Painter:  Experienced as a professional sign painter for minimum 3 years.

    - Finishes and Painting:  Adequate to withstand weathering, fading, and chipping for duration of construction.

    - Signs other than those specified:  Not Permitted.

  - SUBMITTALS

    - Section 01300-Submittals:  Shop Drawings.

    - Show content, layout, lettering, color, foundation or footings, structure, sizes, and grades of members.

    - PRODUCTS

  - SIGN MATERIALS

    - Structure and Framing:  New materials, wood or metal, structurally adequate.

    - Sign Surfaces:  Exterior grade plywood with medium density overlay, minimum 3/4 inch thick, standard large sizes to minimize joints.  Increase thickness as required to span across framing members and to provide an even, smooth surface without waves or buckles.

    - Rough Hardware:  Galvanized.

    - Paint and Primers:  Exterior quality, 1 coat primer and 2 coats finish color.

        - Colors:

        - As required by regulatory agencies or as otherwise specified below or indicated on drawings.

        - Where no regulatory requirements exist:  Maintain uniform color scheme throughout site.

    - Lettering:  Exterior quality paints, contrasting colors as selected.

  - CONSTRUCTION PROJECT SIGNS

    - One painted sign of construction, design, size, and content shown on Drawings, location designated.  Customization of graphic design:  Not Permitted.

    - Before project sign acceptance by the Project Consultant as to size, design, type, location, and local regulations, Contractor and subcontractors may erect temporary signs for purposes of identifications and controlling traffic.

      - Furnish, erect, and maintain signs as required by applicable safety regulations or as necessary to safeguard life and property.

    - Upon starting construction.  Provide a 4 feet x 8 feet project sign containing the school name, project name, project number, members of the School Board of Broward County and superintendent of schools in office at the time of construction contract award, the year construction contract award, Project Consultant and Project Consultant’s consultant’s and Contractor.

      - Layout, color, construction, and location of sign require acceptance by Project Consultant.

      - At Contractor’s option, provide Project signage similar to 4 feet x 8 feet project sign, containing the same information, with subcontractor’s as Contractor may desire.

  - PROJECT INFORMATIONAL SIGNS

    - Painted informational signs of the same colors and lettering as Project Identification sign, or standard products; size lettering to provide legibility at 100-foot distance.

    - Provide at each field office, storage shed, and directional signs to direct and control traffic into and within site.  Relocate as Work progress requires.

    - Provide Florida Department of Transportation standard directional traffic signs to and within site.

- EXECUTION

  - INSTALLATION

    - Install project identification sign within 30 days after date of receipt of Document 00550-Notice to Proceed.

    - Erect at designated location.  If drawings do not indicate, coordinate with the Owner to select a location of high public visibility adjacent to main entrance to site.

    - Erect supports and framing on secure foundation, rigidly braced and framed to resist wind loadings.

    - Install sign surface plumb and level, with butt joints.  Anchor securely.

    - Paint exposed surfaces of sign, supports, and framing.

  - MAINTENANCE

    - Maintain signs and supports clean, repair deterioration and damage.

    - Repaint as necessary to present an “as new” appearance as required by the Project Consultant or Owner.

    - Relocate signs and provide additional signage as the Work progresses.

  - REMOVAL

    - Remove signs, framing, supports, and foundations at completion of the Work and restore the area.

END OF SECTION