# SECTION 01610 (01 60 00)

## BASIC PRODUCT REQUIREMENTS

- GENERAL

- SECTION INCLUDES

    - Product List Schedule.

    - General Product Requirements.

    - Toxic Substances.

    - RELATED DOCUMENTS

    - Section 01330-Submittal Procedures.

    - Section 01620-Product Options.

    - SUBMITTALS

    - Proposed Product List:

    - Prepare a schedule showing products specified in a tabular form acceptable to the Project Consultant as specified below.

    - Coordinate the product list schedule with the Contractor's Construction Schedule and the Schedule of Submittals.

    - Format: Prepare the product listing schedule with information on each item tabulated under the following column headings:

    - Related Specification Section number.

    - Generic name used in Contract Documents.

    - Proprietary name, model number and similar designations.

    - Manufacturer's name and address.

    - Supplier's name and address.

    - Installer's name and address.

    - Projected delivery date, or time span of delivery period.

    - Initial Submittal:  Within 15 days from receipt of Document 00550-Notice to Proceed, submit 3 copies of an initial product list schedule.

    - Provide a written explanation for omissions of data, and for known variations from Contract requirements.

    - At the Contractor's option, the initial submittal may be limited to product selections and designations that shall be established early in the Contract period.

    - Completed Schedule:  Within 45 days from receipt of Document 00550-Notice to Proceed, submit 3 copies of the completed product list schedule.

    - Provide a written explanation for omissions of data, and for known variations from Contract requirements.

    - Provide comprehensive listing of product selections and designations for all major products.

    - Project Consultant’s Action:

    - Will respond in writing to the Contractor within 2 weeks of receipt of the completed product list schedule.

    - No response within this time period constitutes no objection to listed manufacturers or products, but does not constitute a waiver of the requirement that products comply with Contract Documents.

    - Response will include:

          - A list of unacceptable product selections, containing a brief explanation of reasons for this action.

          - A request for additional data necessary for the review and possible acceptance of the products and manufacturers listed.

          - GENERAL PRODUCT REQUIREMENTS

    - Definitions:

    - Products:  Means new material, machinery, components, equipment, fixtures, and systems forming the Work.  Does not include machinery and equipment used for preparation, fabrication, conveying and erection of the Work.  Products may also include existing materials or components required for reuse.

    - Materials:  Products that are substantially shaped, cut, worked, mixed, finished, refined or otherwise fabricated, processed, or installed to form a part of the Work.

    - Equipment:  Product with operational parts, whether motorized or manually operated, that requires service connections such as wiring or piping.

    - Shall not use products removed from existing premises, other facilities or other construction sites, except as specifically permitted by the Contract Documents.

    - Provide interchangeable components of the same manufacturer, for similar components.

    - Provide products that comply with the Contract Documents, that are undamaged and, unless otherwise indicated, unused at the time of installation.

    - Provide manufactured and fabricated products that produced and assembled in accord with the best design, engineering and shop practices.

    - Provide products that are suitable for the service conditions.

    - Adhere to specified product, equipment and component capacities, sizes and dimensions.

    - Provide identical products when multiples of the same products are required.

    - Shall not use material or equipment for any purpose other than that designated or specified.

    - Provide products complete with all accessories, trim, finish, safety guards and other devices and details needed for a complete installation and for the intended use and effect.

    - Standard Products:  Where available, provide standard products of types that have been produced and used successfully in similar situations on other projects.

    - Continued Availability:  Where, because of the nature of its application, the Owner is likely to need replacement parts or additional amounts of a product at a later date, either for maintenance and repair or replacement, provide standard, domestically produced products for which the manufacturer has published assurances that the products and its parts are likely to be available to the Owner at a later date.

    - Nameplates:  In occupied spaces, except for required labels and operating data, manufacturer's or producer's nameplates or trademarks on exposed surfaces of products shall not be exposed to view.

    - Labels:  Locate required product labels and stamps on a concealed surface or, where required for observation after installation, on an accessible surface that is not conspicuous.

    - Equipment Nameplates:

    - Provide a permanent nameplate on each item of service connected or power operated equipment.

    - Locate on an easily accessible surface, which is inconspicuous in occupied spaces.

    - The nameplate shall contain the following information and other essential operating data:

    - Name of product and manufacturer.

    - Model and serial number.

    - Capacity.

    - Speed.

    - Ratings.

    - TOXIC SUBSTANCES

    - Chapter 1013.49, Florida Statutes, delineates the parameters for the use of toxic substances enumerated in the Florida Substance List that shall not be used in the construction, repair and maintenance of educational facilities.

    - Before any such substance may be used, the Contractor shall notify the District Superintendent, in writing, at least 3 working days prior to using the substance.  The notification shall contain:

    - Name of substance to be used.

    - Where substance is to be used.

    - When substance is to be used.

    - Other Hazardous Materials:  In addition to the restrictions placed by Chapter 1013.49, Florida Statutes, use of the products containing the following materials is strictly prohibited on School Board of Broward County, Florida projects:

    - Asbestos.

    - Polychlorinated biphenyl (PCB).

    - Cellulose or urea formaldehyde.

    - Lead.

    - Handling of toxic materials:

    - Storage:  Shall not store any chemical or otherwise toxic product in any size container outside of a building.  Provide proper secondary containment barriers for all stored chemicals or toxic materials.

    - Shall not discharge any volume of any material or chemical directly onto the ground, into any water source, or into any storm drain. Discharge materials or chemicals into sanitary sewer system in accordance with local, Broward County, and State of Florida requirements.

    - Do not store buckets, drums, and containers of chemicals or other toxic materials on site.   Dispose of on said containers off site in accordance with local, Broward County, and State of Florida requirements.

    - The Contractor shall remove all chemical products from the site at the completion of use of the chemicals.  Extra stock materials shall be properly stored on site and conveyed by the Contractor upon notice by the Owner to a storage area designated by the Owner.

    - Material Safety Data Sheets (MSDS) shall be maintained by the Contractor on site at all times for all chemicals/products.

    - Contractor's Responsibility:

    - Materials and chemicals used during the Work.

    - Spillage or storage of chemical materials on or near the site or any other location used in connection with prosecuting the Work causing an environmental problem or causing a notice of compliance to be issued by any authoritative Federal, State and Local governmental agency; shall be the responsibility of the Contractor for:

    - Any and all cleanup costs.

    - Any and all enforcement fines/penalties and any other associated cost and/or actions deemed necessary as to resolve the problem, to the satisfaction of the respective jurisdictional agency and the Owner.

    - Lead Safety:  Beginning April 22, 2010, federal law (EPA’s Final Rule 40 CFR Part 745) shall require that contractors and renovators performing renovation, repair, and painting projects that disturb lead based paint in homes, child care facilities, and schools built before 1978 that a child under age 6 visits regularly, to be certified and follow lead-safe work practices to prevent lead contamination.  Contractors and renovators must be EPA Certified and projects must comply with the EPA manual “Renovate Right, Important Lead Hazard Information for Families, Child Care Providers and Schools”.

- PRODUCTS-Not Used

- EXECUTION

- INSTALLATION OF PRODUCTS

    - Comply with manufacturer's instructions and recommendations for installation of products in the applications indicated.  Anchor each product securely in place, accurately located and aligned with other Work.

    - Clean exposed surfaces and protect as necessary to ensure freedom from damage and deterioration at time of Substantial Completion.

END OF SECTION