# SECTION 01730 (01 73 00)

## EXECUTION

- GENERAL

- SECTION INCLUDES

    - Application and Installation.

    - Cutting and Patching.

    - Erection.

    - Existing Products.

    - Selective Demolition.

    - RELATED DOCUMENTS

    - Section 01560-Temporary Barriers and Enclosures.

    - Section 01720-Preparation.

    - Section 01740-Cleaning.

    - Section 01780-Closeout Submittals.

    - Individual Product Specification Sections:

      - Cutting and patching incidental to work of the section.

      - Selective demolition incidental to work of the section.

      - Advance notification to construction entities of openings required in Work of those sections.

      - Limitations on cutting structural members and fire rated assemblies.

      - SUBMITTALS

    - Written Request:

      - Submit a written request to Project Consultant at least 10 days before any cutting or alteration affecting:

        - Work of other general contractor hired by the Owner.

        - Structural value or integrity of any element of the Work.

        - Integrity of weather-exposed or moisture-resistant elements or systems.

        - Efficiency, operational life, maintenance, or safety of building elements.

        - Visual qualities of sight-exposed elements.

        - Security of facility.

      - Written request shall include:

        - Identification of the Work.

        - Description of proposed work:

        - Scope of cutting, patching, alteration, or excavation.

        - Trades executing the work.

        - Products proposed to be used.

        - Extent of refinishing to be done.

        - Location and Description of affected work.

        - The need for cutting, alteration, or excavation and how it shall be performed; indicate why it cannot be avoided.

        - Effect on work of any separate Contractors.

        - Describe anticipated results in terms of changes to existing construction; include changes to structural elements and operating components as well as changes in the building’s appearance and other significant elements.

        - List utilities that will be disturbed or affected, including those that will be relocated and those that will be temporarily out-of-service.  Indicate how long the service will be disrupted.

        - Where cutting and patching involves addition of reinforcement to structural elements, submit details and engineering calculations to show how reinforcement is integrated with the original structure.

        - Effect on structural or weatherproof integrity of the Work.

        - Alternatives to cutting and patching.

        - Written permission of any separate contractor whose work will be affected.

        - Date and time the Work will be uncovered.

        - Date and time the Work shall be completed or restored.

      - Approval by the Project Consultant and Owner to proceed with cutting and patching does not waive the Project Consultant’s right to later require complete removal and replacement of a part of the Work found to be unsatisfactory.

    - Selective Demolition:

      - Submit schedule noting proposed sequence of operations for selective demolition work to Owner for review and approval prior to commencement of demolition.

      - Include coordination for shut-off, capping, continuation of utility services as required, together with details for dust, fume, moisture, and noise control protection.

      - Coordinate with Owner's continuing occupation of portions of existing building and with Owner's partial occupancy of completed new addition, alteration, and renovation.

    - Project Record Documents:  Accurately record actual locations of capped utilities, subsurface obstructions, and unanticipated structural, mechanical and electrical elements uncovered during demolition and submit under provisions of Section 01780-Closeout Submittals.

    - QUALITY ASSURANCE

    - Requirements for Structural Work:  Do not cut and patch structural elements in a manner that would reduce their load carrying capacity or load deflection ratio.

    - Obtain approval of the cutting and patching proposal before cutting and patching the following structural elements:

      - Foundation or footing construction.

      - Bearing and retaining walls.

      - Structural concrete.

      - Structural steel.

      - Lintels.

      - Timber and primary wood framing.

      - Structural decking.

      - Stair systems.

      - Miscellaneous structural metals.

      - Exterior curtain wall construction.

      - Equipment supports.

      - Piping, ductwork, vessels, and equipment.

      - Structural systems of special construction in Division 13.

    - Operational and Safety Limitations:

      - Shall not cut and patch operating elements or safety related components in a manner that would result in reducing their capacity to perform as intended, or result in increased maintenance, or decreased operational life or safety.

      - Obtain approval of the cutting and patching proposal before cutting and patching the following operating elements or safety related systems:

        - Shoring, bracing, and sheeting.

        - Primary operational systems and equipment.

        - Air or smoke barriers.

        - Water, moisture, or vapor barriers.

        - Membranes and flashings.

        - Fire protection systems.

        - Noise and vibration control elements and systems.

        - Control systems.

        - Communication systems.

        - Conveying systems.

        - Electrical wiring systems.

        - Special construction specified by Division 13 Sections.

    - Visual Requirements:

      - Shall not cut and patch construction exposed on the exterior or in occupied spaces, in a manner that would, in the Project Consultant's opinion, reduce the building's aesthetic qualities, or result in visual evidence of cutting and patching.

      - Remove and replace Work cut and patched in a visually unsatisfactory manner.

      - If possible retain the original installer or fabricator to cut and patch the following categories of exposed Work, or if not possible to engage the original installer or fabricator, engage another installer or fabricator recognized experienced and specialized firm:

        - Processed concrete finishes.

        - Stonework and stone masonry.

        - Ornamental metal.

        - Matched veneer woodwork.

        - Preformed metal panels.

        - Window wall system.

        - Stucco and ornamental plaster.

        - Acoustical ceilings.

        - Terrazzo.

        - Finished wood flooring.

        - Fluid applied flooring.

        - Carpeting.

        - Aggregate wall coating.

        - Wall covering.

        - Swimming pool finishes.

        - HVAC enclosures, cabinets or covers.

        - APPLICATION AND INSTALLATION

    - Materials and Workmanship:  Conform to the requirements of the contract documents with respect to the parts or kinds of work included.

    - Provide attachment and connection devices and methods necessary for securing Work.  Secure Work and products true to line and level.  Allow for expansion and building movement.

    - Visual Effects:  Provide uniform joint widths in exposed Work.  Arrange joints in exposed Work to obtain the best visual effect.  Refer questionable choices to the Project Consultant for final decision.

    - Install each component during weather conditions and Project status that will ensure the best possible results.  Isolate each part of the completed construction from incompatible material as necessary to prevent deterioration.

    - Coordinate temporary barriers and enclosures with required inspections and tests, to minimize the necessity of uncovering completed construction for that purpose.

    - Mounting Heights:  Where mounting heights are not noted, install individual components at standard and/or ADA mounting heights recognized within the industry and/or per SBBC or ADA requirements whichever is more stringent for the particular application indicated.  Refer questionable mounting height decisions to the Project Consultant for final decision.

    - ERECTION

    - Erect structural and other building elements within the dimensional tolerances and other requirements as specified in the respective specification sections of Divisions 2 through 16.

  - EXISTING PRODUCTS

    - Protect existing products or newly installed products during all demolition, cutting, patching and other construction operations.

    - PROTECTION

    - Protect existing products or newly installed products during all demolition, cutting, patching and other construction operations.

    - SELECTIVE DEMOLITION

    - Conform to Florida Building Code for demolition work, safety of structure, dust control and safeguards required during construction.

    - Notify affected utility companies before starting work and comply with their requirements.

    - Shall not close or obstruct egress or access width to exits.

    - Shall not disrupt building fire or life safety systems without 3 day prior written notice to the Owner.

    - If the Owner will be continuously occupying areas of building and site immediately adjacent to areas of selective demolition:

      - Conduct demolition work in manner that will minimize disruption of Owner's normal operations.

      - Provide minimum of 5 working days advance notice to Owner of demolition activities, which will impact Owner's normal operations.

      - PROTECTION OF ADJACENT PROPERTY

    - Cleaning And Protection:

      - During handling and installation, clean and protect construction in progress and adjoining materials in place.  Apply protective covering where required to ensure protection from damage or deterioration at Substantial Completion.

      - Clean and maintain completed work as frequently as necessary through the entire work period.  Adjust and lubricate operable components to ensure operability without damaging effects.

      - PRODUCTS

      - MATERIALS

    - Cutting and Patching:

      - Use materials that are identical to existing materials.

      - If identical materials are not available or cannot be used where exposed surfaces are involved, use materials that match existing adjacent surfaces to the fullest extent possible with regard to visual effect.

      - Use materials whose installed performance shall equal or surpass that of existing materials.

      - EXECUTION

      - INSPECTION

    - Before cutting existing surfaces, examine surfaces to be cut and patched and conditions under which cutting and patching is to be performed.

    - Take corrective action before proceeding, if unsafe or unsatisfactory conditions are encountered.

    - Before proceeding, meet at the site with parties involved in cutting and patching, including mechanical and electrical trades.

    - Review areas of potential interference and conflict.  Coordinate procedures and resolve potential conflicts before proceeding.

    - PREPARATION

    - Temporary Support:  Provide temporary support of Work to be cut.

    - Protection:

      - Protect existing construction during cutting and patching to prevent damage under provisions of and Section 01560-Temporary Barriers and Enclosures and Section 01720-Preparation.

      - Provide protection from adverse weather conditions for portions of the Project that might be exposed during cutting and patching operations.

    - Avoid interference with use of adjoining areas or interruption of free passage to adjoining areas.

    - Take all precautions necessary to avoid cutting existing pipe, conduit or ductwork serving the building, but scheduled to be removed or relocated until provisions have been made to bypass them.

    - CUTTING AND PATCHING

    - Asbestos:  If the Contractor encounters what is believed to be asbestos or other dangerous materials, refer to Section 00700-General Conditions.

    - General:

      - Employ skilled workers to perform cutting and patching.

      - Cut existing construction to provide for installation of other components or performance of other construction activities and the subsequent fitting and patching required to restore surfaces to their original condition at start of work.

    - Cutting:

      - Cut existing construction using methods least likely to damage elements to be retained or adjoining construction.

      - Where possible review proposed procedures with the original installer; comply with the original installer's recommendations.

        - In general, where cutting is required use hand or small power tools designed for sawing or grinding, not hammering and chopping.

        - Cut holes and slots neatly to size required with minimum disturbance of adjacent surfaces.

        - Temporarily cover openings when not in use.

        - To avoid marring existing finished surfaces, cut or drill from the exposed or finished side into concealed surfaces.

        - Cut through concrete and masonry using a cutting machine such as a carborundum saw or diamond core drill.

        - Comply with requirements of applicable Sections of Division 2 where cutting and patching requires excavating and backfilling.

    - Bypass utility services such as pipe or conduit, before cutting, where services are shown or required to be removed, relocated or abandoned:

      - Cutoff pipe or conduit in walls or partitions to be removed.

      - Cap, valve or plug and seal the remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after bypassing and cutting.

    - Patching:

      - Patch with durable seams that are as invisible as possible.  Comply with tolerances specified for the same or similar Work in Divisions 2 through 16.

      - Where feasible, inspect and test patched areas to demonstrate integrity of the installation.

      - Restore exposed finishes of patched areas and extend finish restoration into retained adjoining construction in a manner that will eliminate evidence of patching and refinishing:

        - Where removal of walls or partitions extends one finished area into another, patch and repair floor and wall surfaces in the new space to provide an even surface of uniform color and appearance.

        - Remove existing floor and wall coverings and replace with new materials, if necessary to achieve uniform color and appearance.

        - Where patching occurs in a smooth painted surface, extend final paint coat over entire unbroken containing the patch, after the patched area has received primer and second coat.

        - Patch, repair or rehang existing ceilings as necessary to provide an even plane surface of uniform appearance.

    - Cleaning:

      - Thoroughly clean areas and spaces where cutting and patching is performed or used as access.

      - Remove completely paint, mortar, oils, putty and items of similar nature.

      - Thoroughly clean piping, conduit and similar features before painting or other finishing is applied.

      - Restore damaged pipe covering to its original condition at start of work.

      - SELECTIVE DEMOLITION

    - Asbestos:  If the Contractor encounters what is believed to be asbestos or other dangerous materials, refer to Section 00700-General Conditions.

    - General Requirements:

      - Perform selective demolition work in a systematic manner.

      - Use such methods as required to complete Work indicated on Drawings in accordance with selective demolition schedule, governing regulations.

        - Cease operations, evacuate, and notify the Owner's Representative immediately if safety of structure appears to be endangered.

      - Take precautions to support structure until determinations is made for continuing operations.

      - Maintain protected egress and access to the Work.

    - Demolition

      - Disconnect, remove, cap, and identify designated utilities within demolition areas.

      - Demolish in an orderly and careful manner.  Protect existing supporting structural members and all items to remain.

      - Promptly remove debris to avoid imposing excessive loads on supporting walls, floors, roofs, or framing.

      - If unanticipated mechanical, electrical or structural elements conflict with intended function or design are encountered, investigate, measure both nature, extent of the conflict.  Submit report to Owner in written, accurate detail.

      - Pending receipt of directive from Owner, rearrange selective demolition schedule as necessary to continue overall job progress without delay.

    - Salvage Items

      - Existing equipment, fixtures as noted on drawings or as directed by Project Consultant or Owner, for re-use:  Removed from site, stored, reinstalled by Contractor as otherwise required by the Contract Documents.

    - Disposal Of Demolished Materials:

      - Remove debris; rubbish other materials resulting from selective demolition operations from site as required by Section 01740-Cleaning and other provisions of the Contract Documents.

      - If hazardous materials are encountered during demolition operations, comply with applicable regulations, laws, and ordinances concerning removal, handling and protection against exposure or environmental pollution.

      - Burning of removed materials shall not be permitted on project site.

    - Clean Up And Repair Of Selectively Demolished Spaces:

      - Upon completion of selective demolition work, remove tools, equipment, and demolished materials from site.  Remove protections, leave interior areas broom clean.

      - Repair demolition performed in excess of that required.

      - Return damaged structures, surfaces to remain to condition existing prior to commencement of selective demolition work.

      - Repair adjacent construction on surfaces soiled or damaged by selective demolition work.

END OF SECTION