# SECTION 01740 (01 74 00)

## CLEANING

- GENERAL

  - SECTION INCLUDES

    - Progress  cleaning requirements during construction operations.

    - Final Cleaning prior to building occupancy.

    - Site Maintenance.

  - RELATED DOCUMENTS

    - Document 00520-Agreement Form.

    - Section 01350-Special Procedures.

    - Section 01354–Indoor Air Quality Management.

    - Section 01410-Regulatory Requirements.

    - Section 01520-Construction Facilities.

    - Section 01530–Temporary Construction.

    - Section 01570-Temporary Controls.

    - Section 01720-Preparation.

    - Specification Sections Division 2 through 17:  Cleaning for specific products or elements of the Work.

  - REFERENCES

    - GREENGUARD Product Emission Standard for Children & Schools.

    - Green Seal Environmental Standard for Industrial and Institutional Cleaners - GS-37.

  - SUBMITTALS

    - Product Data:  Submit manufacturer’s technical data for each type of resilient flooring and accessory.

      - Low Emitting Materials.

    - Submit manufacturer’s Material Safety Data Sheet Indicating VOC limits of all cleaning products.

    - Submit manufacturer’s certification that all cleaning comply with of the GREENGUARD Product Emission Standard for Children & Schools or Green Seal Certified Product.Green Seal Environmental Standard for Industrial and Institutional Cleaners - GS-37.

  - GENERAL REQUIREMENTS

    - Maintain premises and public properties free from accumulations of waste, debris, and rubbish, caused by operations.

    - At completion of Work, remove waste materials, rubbish, tools, equipment, machinery and surplus materials, and clean all sight-exposed surfaces; leave project clean and ready for occupancy.

  - QUALITY ASSURANCE

    - Cleaning:

      - Employ experienced workers or professional cleaners for final cleaning.

      - Clean each surface or unit to the condition expected in a normal, commercial building cleaning and maintenance program.

      - Comply with manufacturer's instructions:

        - Use only those cleaning methods recommended by manufacturer of the surface materials to be cleaned.

        - Use cleaning materials only on surfaces recommended by manufacturer of the surface materials to be cleaned.

  - SAFETY REQUIREMENTS

    - Standards:  Maintain Project in accord with safety and insurance standards.

    - Cleaning and disposal operations shall comply with local ordinances and anti-pollution laws.

      - Shall not burn or bury rubbish and waste materials on project site.

      - Shall not dispose of volatile wastes such as mineral spirits, oils, or paint thinner in storm or sanitary drains.

      - Shall not dispose of any wastes into canals, streams or waterways.

  - COLLECTION AND DISPOSAL REQUIREMENTS

    - Utilize waste disposal franchise specified by municipal jurisdictional authority if required in the municipality in which the project site is located.

    - Collect waste from construction areas and elsewhere daily:

      - Comply with requirements of NFPA 241 for removal of combustible waste material and debris.  Enforce requirements strictly.

      - Handle dangerous, or unsanitary waste materials separately from other waste by properly containerizing and appropriately ventilating.

      - Dispose of material in a lawful manner.

  - PEST CONTROL

    - Refer to Section 01570-Temporary Controls.

- PRODUCTS

  - MATERIALS

    - All cleaning products shall comply with the GREENGUARD Product Emission Standard for Children & Schools or Green Seal Certified Environmental Standard Products for Industrial and Institutional Cleaning - GS-37.

    - Use only cleaning materials recommended by product manufacturer suitable for surface to be cleaned.

- EXECUTION

  - DURING CONSTRUCTION

    - Execute cleaning to ensure that site and grounds, and public properties adjacent to site are maintained free from accumulations of waste materials and rubbish.

    - Wet down dry materials and rubbish to lay dust and prevent blowing dust.

    - Each day during progress of Work, clean immediate work site and adjacent areas, and dispose of project related waste materials, debris and rubbish.

    - Remove waste materials, debris and rubbish from site and legally dispose at public or private dumping areas off Owner's property.  Coordinate with municipality’s franchise as specified above.

    - Vacuum clean interior of building areas:

      - When surfaces are ready to receive painting.

      - Prior to the installation of finish flooring materials.

      - Prior to application or installation of materials, finishes or equipment, that would be degraded by dust or dirt.

      - On an as-needed basis to maintain clean work areas through substantial and final completion.

    - Handle materials in a controlled manner with as few handlings as possible; shall not drop or throw materials from heights.

    - Schedule cleaning operations so that dust and other contaminants resulting from cleaning process shall not fall on wet, newly painted surfaces.

  - CLEANING PRIOR TO SUBSTANTIAL COMPLETION

    - Conduct inspection of sight-exposed interior and exterior surfaces and concealed spaces:  Ensure clean condition and removal of debris with access.

    - Complete the following cleaning operations before requesting Substantial Completion Inspection:

      - Remove labels that are not permanent labels.

      - Clean transparent materials, including mirrors and glass in doors and windows.

        - Remove glazing compound and other substances that are noticeable vision obscuring materials.

        - Replace chipped or broken glass and other damaged transparent materials.

      - Clean exposed exterior and interior hard surfaced finishes to a dust free condition, free of stains, films and similar foreign substances:

        - Restore reflective surfaces to their original reflective condition.

        - Leave concrete floors broom clean.

      - Remove paint spots and smears, stains, marks, and dirt to provide clean surfaces.

      - Clean glass, hardware, fixtures, casework, and equipment.

      - Vacuum carpeted and other “soft” finished surfaces:

        - Remove and repair stains, discolorations, abrasions, cuts or other damage.

        - Do not “spot patch” sheet or roll products unless approved by the Project Consultant and Owner.

      - Wipe surfaces of mechanical and electrical equipment:

        - Remove excess lubrication and other substances.

        - Clean plumbing fixtures to a sanitary condition.

        - Clean light fixtures and lamps, including reflectors, bulbs and diffusers.

      - Clean ducts, blowers, coils, diffusers and other elements of air conditioning systems.

      - Replace air conditioning filters.

      - Clean the site, including landscape development areas, of rubbish, litter and other foreign substances:

        - Sweep paved areas broom clean; remove stains, spills and other foreign deposits.

        - Rake grounds that are neither paved nor planted, to a smooth even textured surface.

      - Complete final cleanup requirements, including touch-up painting.

      - Touch-up and otherwise repair and restore marred exposed finishes.

      - Wash concrete surfaces, tile floors and walls, other impervious floor and wall surfaces.

  - FINAL CLEANING

    - Site amenities and manufactured items:

      - In preparation for final completion or occupancy, conduct final inspection of sight-exposed exterior surfaces.

      - Remove grease, dust, dirt, stains, label, fingerprints, and other foreign materials, from sight-exposed exterior finished surfaces.

      - Repair, patch and touch up marred surfaces to specified finish, to match adjacent surfaces.

    - Broom clean driveways, parking, sidewalks, patios, playcourts; rake clean other exterior surfaces or grounds.

    - Maintain cleaning until Owner occupancy.

END OF SECTION