# SECTION 01750 (01 75 00)

## STARTING AND ADJUSTING

- GENERAL

  - SECTION INCLUDES

    - Starting systems.

    - Demonstration and Instructions.

    - Testing, Adjusting, and Balancing.

    - Building Commissioning.

  - RELATED DOCUMENTS

    - Section 01310-Construction Progress Documentation.

    - Section 01430-Quality Assurance.

    - Section 01450-Quality Control.

    - Section 01770-Closeout Procedures.

    - Section 01780-Closeout Submittals.

    - Section 01820-Demonstration and Training.

    - Section 15995-Start-Up and Certification of Air, Water and Control Systems.

  - STARTING SYSTEMS

    - Coordinate schedule for start-up of various equipment and systems.

    - Notify Project Consultant and Owner 7 days prior to start-up of each item.

    - Verify that each piece of equipment or system has been checked for proper lubrication, drive rotation, belt tension, control sequence, or other conditions, which may cause damage.

    - Verify that tests, meter readings, and specified electrical characteristics agree with those required by the equipment or system manufacturer.

    - Verify wiring and support components for equipment are complete and tested.

    - Execute start-up under supervision of responsible Contractor’s and Manufacturer's personnel in accordance with manufacturers' instructions.

    - When specified in individual specification Sections, require manufacturer to provide authorized representative to be present at site to inspect, check and approve equipment or system installation prior to start-up, and to supervise placing equipment or system in operation.

    - Submit a written report in accordance with Section 01310-Construction Progress Documentation that equipment or system has been properly installed and is functioning correctly.

  - DEMONSTRATION AND INSTRUCTIONS

    - Demonstrate operation and maintenance of systems, assemblies, equipment and products to Owner's personnel as required under provisions of Section 01820-Demonstration and Training.

  - TESTING, ADJUSTING, AND BALANCING

    - The Owner will appoint, employ, and pay for services of an independent test and balance consultant to perform observe construction, verify the Work, and perform other services related to HVAC systems in accordance with Section 15995-Start-Up and Certification of Air, Water and Control Systems.  When new or modified HVAC installations are complete and ready for test, Contractor shall complete TEST AND BALANCE REQUEST form and submit same to PM.

    - Cooperate and facilitate the operations of the Owner’s Test and Balance Consultant.

    - SBBC Test and Balance Consultant shall determine the procedures to be followed, relying on information as needed from the Contractor.  The Contractor shall provide any requested information to the Test and Balance Consultant in writing within 10 days following such request.

    - The Contractor shall make available to the Test and Balance Engineer, upon request of the PM, a complete and current set of Construction Documents and other pertinent information, including but not limited to drawings, specifications, HVAC/mechanical shop drawings and calculations, Florida Energy Code submittals and calculations, and product/, material cut sheets, operating manuals, warranties, and other manufacturer information.  Where changes in the work are not reflected in the above material, the Contractor shall provide as-built documentation of all such changes.

    - A copy of the Balance Consultant’s Test and Balance report will be forwarded to the Contractor by the PM and incorporated into the Project Punch List.

- PRODUCTS-Not Used

- EXECUTION-Not Used

END OF SECTION