# SECTION 10190 (10 21 23.16)

## CUBICLE CURTAIN

- GENERAL

  - SECTION INCLUDES

    - Cubicle curtain and surface-mounted track.

  - REFERENCES

    - Aluminum Association (AA).

    - American Standard for Testing of Materials (ASTM):

      - B221-Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

      - B546-Electric Fusion-Welded Ni-Cr-Co-Mo Alloy (UNS N06617), Ni-Fe-Cr-Si Alloys (UNS N08330 and UNS N08332), Ni-Cr-Fe-Al Alloy (UNS N06603), Ni-Cr-Fe Alloy (UNS N06025), and Ni-Cr-Fe-Si Alloy (UNS N06045) Pipe.

  - SUBMITTALS

    - Samples:  Provide assembly consisting of 12 inch long piece of cubicle curtain track with carrier access and end stop.  One clip anchor for fastening track to grid system of acoustical ceilings.

    - Shop Drawings:  Showing layout of track and method of anchorage.

    - Product Data:  Cubicle curtain and track.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver material in original package marked to identify the contents, brand name, and the name of the manufacturer or supplier.

    - Store in dry and protected location.  Store so as to not bend or wrap the tracks.

    - Do not open packages until contents are needed for installation, unless verification inspection is required.

- PRODUCTS

  - CUBICLE CURTAIN TRACK

    - Channel Track:  Extruded aluminum, ASTM B221, alloy 6063, temper T5 or T6, channel shaped, with smooth inside receway for curtain carriers.

    - Curtain Carriers:  Nylon carriers, with either nylon wheels on metal, or nylon axles.  Equip each carrier with either stainless steel hooks with swivel, or stainless steel bead chain and hook assembly.  Hook for bead chain may be the same material and finish as the bead chain or may be chromium-plated steel.

    - Cubicle fabric curtain with 12 inch nylon mesh top:  Curtain to be 100 percent polyester permanently and inherently flame resistant 15 oz. per lineal yard, with 20 inches 100 percent nylon mesh with 1/2 inch openings.  Easily launderable to 160 degrees F. with anti-bacterial and stain resistant properties.  Project Consultant shall choose color and pattern.  High tensile and tear strength.  Provide standard stainless steel hooks.

    - End Stop Connectors, Ceilings Flanges and Other Accessories:  Fabricated from the same material with the same finish as the tracks.

    - At end of each section of track, make provision for insertion and removal of carriers.  Design to prevent accidental removal of carrier.  Any operating mechanism shall be removable with common tools.

  - FASTENERS

    - Exposed Fasteners, Screws and Bolts:  Stainless steel or chromium/nickel plated brass.

    - Concealed Fasteners, Screws and Bolts:  Hot-dip galvanized.

    - Metal Clips:  Anchor curtain tracks to exposed grid of lay-in acoustical tile ceilings, with concealed metal (butterfly) type or two-piece snap locking type or high strength spring steel.

  - FINISHES

    - Aluminum:  Finish numbers for aluminum specified is in accordance with The Aluminum Association’s Designation System.  Chemically etched medium matte, with clear anodic coating, Class II Architectural, 0.4mils thick or white baked enamel.

    - Stainless Steel:  No.304.

  - FABRICATION

    - Weld and grid smooth joints of fabricated components.

    - Form tracks and bends of lengths that will produce the minimum number of joints.  Make track sections up to 16 feet without joints.  Form corner bend on a 12-inch radius.

    - Provide steel anchor plates, supports, and anchors for securing components to building construction.

    - Form flat surface without distortion.

    - Shop assemble components and package complete with anchors and fittings.

- EXECUTION

  - INSTALLATION

    - Install tracks after finish painting and ceiling finishing operations are complete.

    - Reinforce ceiling grid with extra hangers at curtain track location.

    - Install track level and securely anchor to the ceiling to form a rigid installation.

    - Anchor surface mounted curtain tracks directly to exposed grid of lay-in acoustical tile ceilings with concealed metal clips, spaced approximately 24 inches on center.

    - Anchor surface mounted curtain tracks to gypsum board ceilings with a minimum of 1/8-inch diameter fastenings or concealed clips spaced not more than three feet on center.

    - Securely fasten end stop caps to prevent their being forced out by the striking weight of carriers.

    - Remove damaged or defective components and replace with new components or repair to the original condition.

    - Install track neat rigid, plumb, level and true, and securely anchored to the overhead construction.

    - Carrier units shall operate smoothly and easily over the full range of travel.

END OF SECTION