# SECTION 10400 (10 14 00)

## IDENTIFYING DEVICES

- GENERAL

  - SECTION INCLUDES

    - Work of this Section includes, but is not limited to, the following:

      - Plaque.

      - School Name Letters.

      - Exterior Handicapped Signs at Parking Spaces.

      - Emergency Exit Signs.

      - Capacity Signs.

      - Exit Route.

      - Toilet Room Handicapped Signs.

      - Exterior and Interior Space Number and Name Signs.

      - Building Directional Signs.

      - Safety Signs.

  - RELATED SECTIONS

    - Section 10520-Fire Protection Specialties

  - REFERENCES

    - Florida Building Code (FBC).

    - Florida Department of Transportation (FDOT)-Manual of Uniform Traffic Control Devices (MUTCD).

    - U.S. Architectural and Transportation Barriers Compliance Board, “Children Accessibility Requirements”.

  - SUBMITTALS

    - Submit Manufacturer’s technical data and installation instructions for each type of identifying device required.

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of assembly.

    - Submit shop drawings showing sign dimensions, letterform and letter heights.

    - Submit 1 full size sample for the following identifying devices showing style, specified color, and method of attachment.  If approved, sample shall be incorporated in the Work.

      - Exterior /Interior space number and name signs.

      - School name letter (one).

      - ADA signs.

    - Submit a comprehensive schedule of all space names and numbers for each building as well as quantities and locations of all other signs specified.

    - Submit plaque rubbings for approval by the Owner.

  - QUALITY ASSURANCE

    - Comply with the following codes, standards, and specifications:

      - Florida Building Code, Chapter 11.

      - U.S. Architectural and Transportation Barriers Compliance Board, “Children Accessibility Requirements”.

  - DELIVERY, STORAGE and HANDLING

    - Package signs in labeled name groups.

    - Store and protect products from soiling, damage, and degradation from exposure to excessive heat, moisture, and humidity.

  - ENVIRONMENTAL REQUIREMENTS

    - Do not install signs when ambient temperature is below 70 degrees F.  Maintain minimum temperature during and after installation of signs.

- PRODUCTS

  - MANUFACTURERS

    - Manufacturer must have not less than 5 years experience in the fabrication of types signs as specified herein.  Manufacturers must comply with the specifications listed below.

    - Substitutions:

      - Will be considered by the Project Consultant and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

  - MATERIALS

    - All identifying device products are to contain recycled content.

  - SIGN STANDARDS

    - Provide signs that comply with the following:

      - Tactile characters/symbols shall be raised the required 1/32 inch from sign face.  Glue-on letters will not be accepted.

      - All text shall be accompanied by Grade 2 Braille.

      - All letters, numbers and symbols shall contrast with their background, white characters on a dark, silver gray, background. Characters and background shall have a non-glare finish.

      - Radius corners 3/8 inch.

    - Plaque:  Mfr:  New Hermes Incorporated; Product:  Exterior GravoTac, XT Polymer:  #211

      - Thickness:  1/8 inch substrate.

      - Color:  Silver Grey 269-707 with #010 1/32 inch thick color white 200-505, with adhesive backing laminated to Silver Gray substrate.

      - Material:  Non-static, fire-retardant and self-extinguishing and impervious to most acids, alkalies, alcohol, solvents, abrasives and boiling water.

    - Signs are to be rotary engraved.  The sign fabricator is to follow all material specification of the material manufacturer.

    - Letter font shall be Condensed Helvetica upper case.

    - Size of letters and numbers shall be as follows:

      - Space Numbers:			3/4 inch minimum.

      - Space Name:				5/8 inch minimum.

      - Symbol Size:				3 inch minimum

      - Building Name and Number:		1 inch minimum.

    - Standard Grade 2 Braille shall be below copy.

    - Signage Installation Heights:  60 inches from floor to centerline of sign.  Locate on the wall adjacent to the latch side of the door.

    - Mounting:  Provide construction adhesive behind sign and 4 stainless steel tamper-proof screws per skin; provide butterfly anchors at gypsum drywall; provide lead shields in masonry.

    - Caulk:  Seal perimeter of sign to wall with Clear silicone sealant where sign is not completely flush with wall.

  - CAST ALUMINUM PLAQUE

    - Provide cast aluminum plaque as manufactured by Andco Industries Corporation, Art in Bronze, or equivalent as follows:

      - Material:  High-grade aluminum ingots, alloy F-214.

      - Border Style:  No. 2 double lines.

      - Letter Style:  Helvetica satin finish.

      - Background:  Pebble charcoal finish.

      - Mounting:  Type concealed anchors.

      - Size:  In accordance with standard detail.

      - Caulk:  Seal perimeter of plaque to wall with thin neat bead of Clear silicone sealant.

  - SCHOOL NAME LETTERS

    - Provide letters meeting the following requirements:

      - Type:  Metropolitan 1000 Letter Style.

      - Color:  Dark Bronze “Durepox” (ten year warranty).  Other finishes may be submitted for approval in accordance with Section 01630, Products and substitutions.

      - Size and Number:

        - 18 inches high x 40 letters.

        - 10-1/2 inches average width.

        - 2-1/8 inches strike width.

        - 3-3/4 inches depth.

      - Anchors:  Stainless steel drilled in place concealed anchors minimum 2 anchors per letter.

      - Material:  Welded aluminum back channel fabricated non-illuminated letters.

      - Manufacturer:  Andco Industries Corporation or equivalent.

      - Caulk:  Seal perimeter of each letter to wall with thin neat bead of Clear silicone sealant.

  - EXTERIOR HANDICAPPED SIGNS AT PARKING AREAS

    - Provide signage for each handicapped parking space per Florida Building Code and Florida Department of Transportation requirements.

  - EMERGENCY EXIT SIGNS

    - Provide signs in accordance with requirements of Article 2.2 above, with red background, Exterior Gravotac 1/8 inch core Red #248-707, and white raised lettering reading.  At secondary means of egress/emergency egress openings:  “EMERGENCY EGRESS – KEEP AREA CLEAR” and “EMERGENCY ESCAPE”.  At emergency rescue openings: “EMERGENCY RESUCE – KEEP AREA CLEAR”. Braille lettering is not required.  Sign dimensions:  1-1/2 inches high by width required for copy.  Radius corners 3/8 inch locate next to all emergency egress windows, doors or other openings.

    - Mounting shall be with Construction adhesive and 4 stainless steel non-removable head screws using butterfly anchors at drywall and lead using shields at masonry.  Mount at locations as directed by Project Consultant in the field.

    - Caulk:  Seal perimeter of signs to wall with thin neat bead of clear silicone sealant where sign is not flush to wall.

  - CAPACITY SIGNS

    - Provide maximum capacity sign in a frame, provide one at main entrance door in each instructional space and each assembly space with a capacity of 50 or more persons.

    - Provide signs minimum of 1-1/2 inches high by length required, reading “MAXIMUM CAPACITY”.  After or below “maximum capacity” provide number of occupants allowed.

    - Fabricated signs in accordance with requirements specified in Article 2.2 above, with Silver Gray core and White lettering.  Braille lettering is not required.

    - Mounting shall be with construction adhesive and 4 stainless steel non-removable head screws using butterfly anchors at drywall and lead shields where mounted on masonry surfaces.

    - Caulk:  Seal perimeter of signs to wall with thin neat bead of clear silicone sealant where sign is not flush to wall.

  - EXIT ROUTE SIGNS

    - In rooms with 6 or more students:  Provide exit route sign in a frame, provide one in every occupied space at main exit doors, consisting of 10 inch high by 12 inch wide, 3/4 inch by 3/4 inch black anodic architectural aluminum channel frame with metal clips reinforced mitered corners.

    - Frames shall have 1/8-inch clear matte plexiglass face and 1/8-inch tempered hardboard back with 4 tamperproof screws fastened through hardboard back to wall.  Install the card message between face and back.

  - TOILET ROOM HANDICAPPED SIGNS

    - Provide one sign depicting International Symbol of Accessibility (wheelchair) at each toilet room equipped with facilities for the handicapped. Include a male or female pictogram as well as Name. Room number and Braille lettering is required.  Size shall be 8 inch x 8 inch.

    - Color shall be white on a blue background.

    - Fabricated signs in accordance with requirements specified in Article 2.2 above.

    - Mounting signs in accordance with requirements specified in Article 2.2.

  - EXTERIOR AND INTERIOR SPACE NAME AND NUMBER SIGNS

    - Provide one sign at each door leading into each space.

      - Size:  6-inch x 6-inch at all classrooms and laboratories.  Exterior signs and signs at all other spaces to also be 6 inch x 6 inch.

    - Fabricate signs in accordance with requirements specified in Article 2.2 above.

    - Mounting:  Signs shall be installed on the wall adjacent to the latch side of the door.  If there is no space on the latch side of the door, including double leaf doors, signs shall be placed on the nearest adjacent wall.

      - Mounting location shall be such that a person may approach within three inches of signage without encountering protruding objects or standing within the swing of a door.

      - Mount signs in accordance with requirements specified in Article 2.2 above.

  - BUILDING DIRECTIONAL SIGNS

    - Provide Directional Signs to Administrative Office.  Copy to Read:  “ALL VISITORS TO REPORT TO ADMINISTRATION”.  Provide arrow directional indicator to compliment copy.  Provide 4 signs with arrow in each direction; 8 signs total.

    - Size:  Comply with requirements for Exterior Signs.  Braille lettering is not required.

    - Fabricated and install signs in accordance with requirements specified in Article 2.2 above.

  - SAFETY SIGNS

    - Provide at all mechanical and electrical rooms a sign to read as follows:  “STORAGE NOT PERMITTED”.

    - Fabricated signs in accordance with requirements specified in Article 2.2 and 2.6 above, with red background and white raised letters, 1-1/2 inches high by width needed for copy.  Radius corners 3/8 inch.

    - Mount on wall beside doors in accordance with requirements specified in Article 2.2 above.

    - Provide signs with copy as follows:

      - "FIRE EXTINGUISHER INSIDE" at each main door of a room containing a fire extinguisher.

      - "FIRE ALARM PULL STATION INSIDE" at each main door of a room containing a pull station.

      - "ELEVATOR EMERGENCY" adjacent to exterior alarm bell.

      - "STORAGE NOT PERMITTED" at mechanical, electrical, and kiln rooms.

      - "IN CASE OF FIRE DO NOT USE ELEVATOR. USE STAIRWAYS" at elevator hall call stations.  Refer to elevator specifications in Division-14

      - "NO SMOKING" as indicated on Drawings.

    - Signs not required when sufficient Pull Stations and Extinguishers are provided in the corridors.

- EXECUTION

  - EXAMINATION

    - Verify that surfaces are ready to receive work.

    - Verify location, placement and alignment of signage prior to installation.

  - INSTALLATION

    - Install in accordance with manufacturer’s printed instructions.

    - Install signs after receiving surfaces are finished in locations as indicated or directed.

    - Install sign on receiving surface plumb and level.

    - Clean and polish each sign following installation.

END OF SECTION