# SECTION 10675 (10 56 13)

## METAL STORAGE SHELVING AND CABINETS

- GENERAL

  - SECTION INCLUDES

    - Metal storage shelving including storage cabinets, work benches, flammable liquid storage cabinets and necessary accessories indicated on the Drawings and specified in this Section.

  - RELATED SECTIONS

    - Section 01572-Construction Waste Management

    - Section 06100-Rough Carpentry.

    - Division 15 for venting for flammable liquid storage cabinets.

  - REFERENCES

  - International Organization for Standardization (ISO) 14021–1999; Environmental Labels and Declarations

    - (National Fire Protection Association) NFPA 30-Flammable and Combustible Liquids Code.

    - (Occupational Safety and Health Administration) OSHA 29

  - SUBMITTALS

    - Product Data:  Submit manufacturer’s catalog data and installation instructions for each type of item specified.

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of assembly.

    - Shop Drawings:  Submit showing layout, details, dimensions and assembly and anchorage.  For flammable liquid storage cabinets, show ventilation connections.

  - QUALITY ASSURANCE

    - Provide certification that flammable liquid storage cabinets comply with OSHA 29 CFR 1910.106D and NFPA 30 requirements.

- PRODUCTS

  - MANUFACTURERS

  - Subject to compliance with requirements, provide products of one of the following:

      - Equipto Steel Products.

      - Hallowell, division of List Industries.

      - Justrite, for flammable liquid storage.

      - Penco Products, Inc.

      - Tri-Boro Shelving.

  - MATERIALS

    - All metal storage and cabinet products are to contain recycled content.

    - Design and Construction:  18 gage clip-type adjustable shelving of modular unit construction of individual bolted frame assemblies, constructed in such a way that any section can be relocated and each shelf can be independently adjusted without the use of tools.

    - Shelving Unit Sizes:  Provide shelving in sizes as indicated on the Drawings.

      - Unit Sizes:

        - Type 1:  87 inches high x 12 inches deep, with top and six adjustable shelves, closed base fronts.

        - Type 2:  87 inches high x 18 inches deep, with top and six adjustable shelves, closed base fronts.

        - Type 3:  87 inches high x 24 inches deep, with top and six adjustable shelves, closed base fronts.

        - Above-noted Shelving:  Extend from wall to wall in rooms indicated on the Drawings to receive metal storage shelving.  Provide standard width units of 30 inches, 36 inches and 42 inches.

      - Provide metal shelving in Rooms as noted on the plans.

    - Wall Attachment Hardware:  Provide manufacturer’s recommended brackets and mounting screws for wall attachment.

  - WORKBENCH

    - Provide freestanding unit of heavy-duty welded steel construction as manufactured by Equipto Storage Products.

      - Substitutions:

        - Will be considered by the Project Consultant and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

    - Size:  34 inch x 72 inch x 37 inch high.

    - Top:  Steel.

    - Provide unit with manufacturer’s standard half shelf underneath top.

    - Prove steel stringer for rigidity.

    - Finish:  Manufacturer’s standard color finish; color as selected by the Project Consultant.

  - STORAGE CABINET

    - Provide cabinet as manufactured by Equipto Storage Products.

      - Substitutions:

        - Will be considered by the Project Consultant and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

    - Twenty two gage steel storage cabinet measuring 48 inch wide x 24 inch deep by 7’-0” high, with built-in lever lock (3-point locking) and 4 adjustable shelves for installation.

    - Doors rotate on top and bottom pin hinges, swinging a full 180 degrees.

    - Fully enclosed, 16 gage independent, welded steel frame.

    - Approximate Weight:  225 pounds.

    - Finish:  Manufacturer’s standard color finish; color as selected by the Project Consultant.

  - FLAMMABLE LIQUID STORAGE CABINET

    - Provide cabinets as manufactured by Justrite.

      - Substitutions:

        - Will be considered by the Project Consultant and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

    - Provide cabinets with the following salient features:

      - Welded steel construction.

      - Dual 2-inch vents.

      - 2-inch deep leak proof sill in base of cabinet.

      - Three-point locking, and concealed, self-closing mechanism.

      - Fusible Link.

      - Adjustable galvanized steel shelves with 1/2 inch lip at edges.

      - Double-wall thermal barrier construction.

      - Adjustable leveling feet.

- EXECUTION

  - EXAMINATION

    - Do not proceed with the work of this Section until conditions detrimental to the proper and timely completion of the work have been corrected in an acceptable manor.

  - INSTALLATION

    - Set adjustable shelving units accurately in place in locations as indicated on the Drawings.

    - Touch-up marred surfaces.  Replace units damaged beyond cosmetic correction as directed by the Project Consultant.

    - Install intermediate shelves at equal spacing in each unit.

    - Anchor shelving to wall surfaces with anchoring devices as recommended by the manufacturer.

    - Install cabinets in accordance with manufacturer’s printed instructions.

  - CLEANING

    - Clean shelving and cabinets with a damp cloth, leaving them in clean condition, free from dirt, smears, fingerprints and other foreign matter.

END OF SECTION