# SECTION 10810 (10 28 13)

## TOILET ACCESSORIES

- GENERAL

  - SECTION INCLUDES

    - Extent of each type of toilet accessory is indicated on drawings or is specified herein.

    - Types of toilet accessories required include, but are not limited to, the following:

      - Grab Bars.

      - Hand Dryer.

      - Medicine Cabinet/Mirror Units.

      - Mirror Units.

      - Mop and Broom Holders.

      - Paper Towel Dispensers.

      - Sanitary Napkin Disposal Units.

      - Sanitary Napkin/Tampon Vendors.

      - Shower Curtain.

      - Shower Seat.

      - Toilet Tissue Dispensers.

      - Towel Bars.

      - Utility Shelf Units.

      - Waste Receptacle.

    - Work Not Included:  Soap dispensers will be furnished and installed by the Owner.  However, wall blocking shall be required and provided under work of this contract.

  - RELATED SECTIONS

    - Section 01572-Construction Waste Management.

    - Section 06100-Carpentry

    - Section 07920-Joint Sealants.

  - DESIGN REQUIREMENTS

    - Provide surface-mounted toilet accessories unless indicated otherwise.  Attach to walls with heavy-duty stainless steel tamper-proof fasteners.

    - Provide stainless steel finish for all toilet accessories unless otherwise noted.

  - SUBMITTALS

    - Product Data:  Submit manufacturer’s technical data and installation instructions for each toilet accessory.

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of assembly.

    - Schedule:  Provide a toilet accessory schedule keyed in to the contract set room numbers indicating required number and type of accessory for each space.

    - Samples:  If requested by Project Consultant, submit full-size samples of specific units to Project Consultant for review of design and operation.  Acceptable samples will be returned and may be used in work.

    - Setting Drawings:  Provide setting drawings, templates, instructions, and directions for installation of anchorage devices and cutout requirements in other work.

  - QUALITY ASSURANCE

    - Inserts and Anchorages:  Furnish inserts and anchoring devices, which must be set in concrete or built into masonry.  Coordinate delivery with other work to avoid delay.  Provide wood blocking and bracing requirements for units installed in gypsum board partition systems.

    - Products:  Provide products of same manufacturer for each type of accessory unit and for units exposed in same areas, unless otherwise specified.

- PRODUCTS

  - ACCEPTABLE MANUFACTURERS

    - Available Manufacturers:  Subject to compliance with requirements, manufacturers offering toilet accessories which may be incorporated in the work include the following:

      - American Specialties, Inc. (ASI).

      - (Basis of Design) Bobrick Washroom Equipment, Inc.

      - Bradley Corporation.

      - Georgia Pacific, for toilet tissue.

  - MATERIALS

    - All toilet accessory products are to contain recycled content.

  - ACCESSORY ITEMS

    - General:  Contractor shall provide these accessories.  The following list of manufacturers and model numbers identify the unit quality.  Equivalent models from the above list of manufacturers will be considered by the Owner.

    - Grab Bars:

      - Wall-mounted type.

      - Theft-proof concealed fasteners.

      - Heavy duty with peened non-slip gripping surface.

      - 1-1/4 inches diameter.

      - Stainless steel finish.

      - Bobrick Model No. B-5806 Series.

    - Hand Dryer:

      - Operation:  Automatic hands-free.

      - Mounting:  Recessed wall mounted hand dryer with max 4 inch protrusion from wall.  Must be ADA compliant.

      - Cover:

        - Cast-iron cover.

        - Finish:  High-gloss vitreous enamel cover or porcelain enamel.

        - Color:  White.

      - Nozzle:  Two-position, chrome plated, vandal resistant.

      - One-piece cast aluminum base with vandal-resistant air-inlet vanes on sides and bottom.

      - Motor shall be 1/10 HP, 115 v AC, 20 AMP, 2300 watts, 50/60 HZ, single phase.

      - Dryer will shut-off when hands are removed.

      - Unit shall be UL listed.

      - Warranty:  10 years against manufacturing defects, brushes 3 years.

      - (Basis of Design) Mfr:  Bobrick; Product:  Model No. B-750.

        - Other Acceptable Manufacturers:

          - Mfr:  American Specialties, Inc. (ASI); Product:  Model 0195.

          - Mfr:  Bradley Corporation; Product:  Model 2897-28.

          - Mfr:  World Dryer, Corporation; Product:  Model-A, Model XRA5Q.

    - Medicine Cabinet/Mirror Units (Non-Clinic Areas Only):

      - Recessed type.

      - Type 304 stainless steel construction with satin finish on exposed surfaces.

      - Mirror:  No. 1 quality, 1/4 inch float-plate glass electrolytically copper-plated.

      - Door has a full-length stainless steel piano hinge.

      - Bobrick Model No. B398 Series.

    - Mirror Units (Handicap Accessible Toilets& Staff Areas Only):

      - Size:  18 inch x 30 inch.

      - Surface-mounted.

      - One-piece roll formed frame units 3/4 inch x 3/4 inch, type 304 stainless steel angle with satin finish.

      - 1/4 inch tempered glass mirror.

      - Bobrick Model No. B-2908.

    - Mirror Units (Handicap Accessible Toilets & Student/Public Areas):

      - Size:  18 inch x 30 inch.

      - Surface-mounted.

      - Frameless stainless steel mirror with bright polish finish.

      - Bobrick Model No.  B-1556.

    - Mop and Broom Holders:

      - Provide one each at every custodial closet, over the mop sink.

      - Size:  36 inch long with 4 mop holders and 3 hooks.

      - Wall-surface mounted.

      - Stainless steel finish.

      - Bobrick Model No. B-224.

    - Paper Towel Dispenser:

      - Provide one in each single person toilet room.

      - Surface-mounted.

      - Dispense 400 C-fold paper towels.

      - Bobrick Model No. B-262

    - Sanitary Napkin Disposal:

      - Satin Stainless steel finish.

      - Removable 1.2-gallon plastic receptacle and self-closing panel covers opening.

        - Mounted in partition:  Bobrick B-354.

        - Recess Mounted:  Bobrick B-353.

        - Surface Mounted:  Bobrick B-254.

    - Sanitary Napkin/Tampon Vendors:

      - Recessed mounted type.

      - Dispensing 31 sanitary napkins and 22 tampons.

      - Single coin mechanism for 25-cent operation.  Coin mechanism to be one-hand operable and not require tight turning or twisting of the hand.

      - Bobrick Model No. B-3500.  For surface mounting use Bobrick Model B-2800.

    - Shower Curtain Rod:

      - Type 304 Stainless Steel Satin Finish.

      - 18 gage 1-1/4 inch diameter rod.

      - Bobrick series B-6047.

    - Shower Seat:

      - Folding.

      - Compliant with ADA barrier free guidelines.

      - Reversible for left or right hand installation.

      - Frame and mounting brackets are type 304 stainless steel, with self-locking mechanism, solid phenolic seat.

      - Bobrick B-5181.

    - Toilet Tissue Dispenser (for typical stalls):

      - Jumbo Senior roll tissue dispenser.

      - 12 inch diameter roll.

      - Stainless Steel Satin Finish.

      - Georgia Pacific model 59493.  Prior to bid or purchase, verify this model is compatible with paper products currently being used by the Owner.

    - Toilet Tissue Dispenser (for ADA stalls & individual toilet rooms):

      - Jumbo Junior roll tissue dispenser.

      - 11 inch diameter roll.

      - Stainless Steel Satin Finish.

      - Georgia Pacific model 58493.  Prior to bid or purchase, verify this model is compatible with paper products currently being used by the Owner.

    - Towel Bar:

      - Extra heavy-duty type.

      - Size:  24 inches wide.

      - 1 inch diameter.

      - 18 gage Type 304 stainless steel, satin finish.

      - Concealed mounting.

      - Bobrick No. B-530.

    - Utility Shelf Units:

      - Size;  8 inch wide, 24 inches long.

      - 18 gage, Type 304 stainless steel with satin finish, with hemmed front edge for safety.

      - Bobrick Model No. B-298.

    - Waste Receptacle (For Group Toilet Rooms Only):

      - Satin finish stainless steel seamless beveled flange.

      - Removable 12-gallon receptacle locks into cabinet.

      - Bobrick model B-3644.

      - With reusable heavy duty vinyl liners.  Bobrick # 3944-12.

  - FASTENERS

    - Fabricate concealed mounting devices and fasteners from stainless steel and within code compliant accessible heights.

    - Finish exposed mounting devices and fasteners to match particular accessory.

    - Fasteners:  Stainless steel theft-resistant type.  Use hex-bolts in lieu of anchored screw wherever feasible.

- PRODUCTS

  - INSTALLATION

    - Install toilet accessories in accordance with manufacturer’s printed directions and within code compliant accessible heights.

    - Finish paint or tile behind all toilet accessories.

    - Secure toilet accessories to support substrate with stainless steel fasteners and anchors of types necessary for rigid anchorage to substrate construction.  Provide back-up plates as required.

    - Install plumb and level, securely and rigidly anchored to substrate.

    - Conceal evidence of drilling and fitting in adjacent surface.

    - Provide a neat, clean sealant bead around all toilet accessories at wall surface.

    - Special Tools:  Deliver properly identified special tool of each type for theft-proof fasteners to Owner.

    - Keys:  Deliver three keys for each keyed alike accessory group to Owner at completion.

  - CLEANING

    - After installation, clean toilet accessories in a manner not to damage finish and leave in condition satisfactory to Project Consultant.

END OF SECTION