# SECTION 10820 (22 41 24)

## SHOWER ENCLOSURES

- GENERAL

  - SECTION INCLUDES

    - Shower Enclosure.

  - RELATED SECTIONS

      - Section 04200-Concrete Masonry.

      - Section 07920-Joint Sealants.

      - Section 09250-Gypsum Board.

      - Section 09951-Resinous Flooring.

      - Section 10810-Toilet Accessories.

      - Division 15 for plumbing.

  - REFERENCES

    - Florida Building Code (FBC), Chapter 11.

  - SUBMITTALS

    - Submit manufacturer’s product data describing full range of features, dimension, profiles, installation instructions and maintenance data, also provide the following:

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of assembly.

  - QUALITY ASSURANCE

    - Roll-in shower enclosures shall meet accessibility requirements of the FBC Chapter 11.

- PRODUCTS

  - MANUFACTURER

    - Subject to compliance with requirements, provide products and systems from the following:

      - Aqua Bath.

    - Substitutions:

      - Will be considered by the Project Consultant and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

  - MATERIALS

    - All shower enclosure products are to contain recycled content.

    - Roll-In Shower:  Aqua Bath Model #C6536BF enclosure with the following features:

      - ADA compliant.

      - One-piece molded acrylic design.

      - Class B backside fire rating; flame spread:  less than 30.

      - Approximate inside dimensions:  60 inch x 36 inch.

      - Approximate weight:  275 pounds.

      - Molded soap tray at 24-3/4 inches above the enclosure floor.

      - Three-sided wrap around grab bar, 1-1/2 inch diameter, 18-gage stainless steel.

      - Eighteen gage, 1-inch diameter shower curtains rod.

      - Two wall brackets at 42 inch and 66 inch above the enclosure floor on the back wall for hand held shower.

      - Self-caulking brass drain.

      - Completely waterproof.

      - Full height white vinyl shower curtain and curtain rings.

    - Roll-In Shower:  Aqua Bath Model #C4136BF-FUS enclosure with the following features:

      - ADA compliant.

      - One-piece molded acrylic design.

      - Class B backside fire rating; flame spread:  less than 30.

      - Approximate inside dimensions:  36 inch x 36 inch.

      - Approximate weight:  230 pounds.

      - Molded soap tray at 28-3/4 inches above the enclosure floor.

      - Three-sided wrap around grab bar, 1-1/2 inch diameter, 18-gage stainless steel.

      - Eighteen gage, 1-inch diameter shower curtains rod.

      - Two wall brackets at 42 inch and 66 inch above the enclosure floor on the back wall for hand held shower.

      - Self-caulking brass drain.

      - Completely waterproof.

      - Full height white vinyl shower curtain and curtain rings.

  - FINISH

    - Provide roll-in shower enclosure in one of manufacturer’s standard colors.

    - Color:  As selected by the Project Consultant.

- EXECUTION

  - INSTALLATION

    - Install roll-in shower enclosures in accordance with manufacturer’s printed instructions.

    - Caulk perimeter of enclosure to surrounding wall construction with an acrylic based one-part paint grade sealant.

END OF SECTION