# SECTION 11450 (11 31 00)

## RESIDENTIAL APPLIANCES

- GENERAL

  - SECTION INCLUDES

    - Residential appliances.

    - Types of residential appliances required include the following:

      - Refrigerators.

      - Freezer.

      - Range.

      - Oven.

      - Cooking Center.

      - Cook Top.

      - Washer.

      - Dryer.

      - Garbage Disposal.

      - Dishwasher.

      - Range Hood.

  - RELATED WORK

      - Section 06400-Architectural Woodwork.

      - Division 15-Plumbing.

      - Division 16-Electrical.

  - SUBMITTALS

    - Product Data:  Submit manufacturer's specifications and installation instructions for each type of residential appliances including data indicating compliance with requirements.  Submit operating and maintenance instructions for each item of residential appliance.

      - Energy Efficiency:

        - Submit documentation for ENERGY STAR qualifications for all residential appliances provided under this Section.

  - QUALITY ASSURANCE

    - Certification Labels:  Provide residential appliances that comply with standards and bear certification labels as follows:

      - Energy Ratings:  Provide energy guide labels with energy cost analysis (annual operating costs) and efficiency information as required by Federal Trade Commission.

        - Provide ENERGY STAR Rated appliance where available.

      - UL Standards:  Provide residential appliances with UL labels.

    - Provide residential appliances by a single manufacturer to the greatest extend possible for the entire project.

  - DELIVERY, STORAGE and HANDLING

    - Deliver products to project site in manufacturer's undamaged protective containers, after spaces to receive them have been fully enclosed.

  - WARRANTY

    - Submit manufacturer's standard written warranty for each item of residential appliance.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturers:  Subject to compliance with the specified requirements, provide products by one of the following manufacturers:

      - Frigidaire.

      - General Electric Company (GE).

      - Kenmore.

      - Whirlpool Corporation.

  - EQUIPMENTS

    - Refrigerator-Side by Side:

      - 22.0 cubic feet capacity.

      - Provide with icemaker.

      - 3 adjustable glass cabinet shelves.

      - 4 door shelves.

      - Textured steel doors and case.

      - Color:  White.

    - Refrigerator-Top Freezer:

      - 21.0 cubic feet capacity.

      - Provide with icemaker.

      - 4 cabinet shelves.

      - 3 door shelves.

      - Texture steel doors and case.

      - Color:  White.

    - Refrigerator-Top Freezer (Home Economics):

      - 18.0 cubic feet capacity.

      - Glass cabinet shelves.

      - 4 glass cabinet shelves.

      - 2 door shelves.

      - Energy saver switch.

      - Texture steel doors and case.

      - Color:  White

    - Refrigerator-Bottom Freezer (Home Economics):

      - 22.2 cubic feet capacity.

      - Provide with icemaker.

      - Wire sliding freezer basket.

      - 4 adjustable cabinet shelves.

      - 5 door bins.

      - Textured steel doors.

      - Color:  White.

    - Freezer-Upright:

      - 20.3 cubic feet capacity.

      - Frost Free.

      - 5 cabinet shelves.

      - 5 doors shelves.

      - Textured steel case and door.

      - Color:  White.

    - Range-Free Standing Electric 30 Inch:

      - Self-cleaning oven.

      - White porcelain enamel cook top.

      - Glass oven door with window.

      - 2 oven shelves.

      - Broiler pan with grid.

      - Lift-off oven door.

      - Electronic oven timer.

      - Infinite heat controls.

      - Oven light.

      - Two 6 inch and two 8 inch heating elements.

      - Color:  White.

    - Double Wall Oven (Home Economics).

      - Microwave upper oven.

      - Conventional self-cleaning lower oven.

      - Automatic oven timer.

      - Oven "ON" light.

      - 2 lower oven shelves.

      - Color:  White on White.

    - Modular Cook top (Home Economics):

      - 30 inch electric with 23-1/8 inch chrome-plated permanent coil module (one 6 inch and one 8 inch elements).

      - Heating element "ON" indicator.

      - Provide left side with Gr 111 Module JXDL44N and Coil Heating Element Calrod Module JDXC44RWH.

      - Infinite heat rotary controls.

      - Knobs to front.

      - White porcelain enamel cook top finish.

    - Washer-Electric:

      - Front Load.

      - 5 water temperature system.

      - 5 foot cord.

      - Super large capacity/4.0 cubic feet.

      - 26 cycles

      - Auto water levels.

      - Bleach dispenser.

      - Fabric softener dispenser.

      - Color:  White.

    - Dryer-Electric:

      - Capacity:  King-size 6.0 cubic feet.

      - Front Load.

      - Automatic dry control.

      - 4 heat selections.

      - Upfront lint filter.

      - End-of-Cycle signal.

      - 6 dry cycles.

      - Color:  White.

    - Garbage Disposal:

      - 3/4 HP.

      - Continuous feed.

      - Wall switch control.

      - Grinding speed:  2700 rpm.

      - Removable splashguard.

      - Sound insulation.

    - Dishwasher-Built In:

      - Controls:  Electric.

      - Cycles/Options:  5/5.

      - Potscrubber cycle.

      - Normal wash cycle.

      - Water saver cycle.

      - Super upper rack with split cup shelf.

      - Deluxe flat rack.

      - Quiet power insulation package.

      - 60 Hertz, 120 Volts, 8.6 Amps;

      - Color:  White/White.

    - Range Hood-30 Inch:

      - Ventilating Type for mounting below wall cabinets.

      - 2 speed fan.

      - Washable filter.

      - 160 CFM complete with duct, wall or roof cap and shutter.

      - Built in cook top light.

      - Color:  White/White.

- EXECUTION

  - INSTALLATION

    - General:  Comply with manufacturer's instructions and recommendations.

    - Built-in Equipment:  Securely anchor units to supporting cabinetry or countertops with concealed fasteners.  Verify that clearances are adequate for proper functioning and rough openings are completely concealed.

    - Freestanding Equipment:  Place units in final locations after finishes have been completed in each area.  Verify that clearances are adequate for proper functioning and rough openings are completely concealed.

    - Utilities:  Refer to Division 15 and 16 for plumbing and electrical requirements.

  - ADJUST AND CLEAN

    - Testing:  Test each item of residential equipment to verify proper operation.  Make necessary adjustments.

    - Accessories:  Verify that accessory items required have been furnished.

    - Cleaning:  Remove packing material from residential equipment items and leave units in clean condition, ready for operation.

END OF SECTION