# SECTION 13130 (13 34 23.13)

## PORTABLE BUILDINGS

- GENERAL

  - SECTION INCLUDES

    - Provide 1 portable building as specified.  Work includes, but is not limited to, building foundations, blocking up, leveling, seaming, anchoring, tie-downs, walkway paving, stripping, handicapped ramps, stairs, and all hook-ups (electrical, intercom, sewer, fire alarm, water, ITV, and telephone).

  - REFERENCES

    - Florida Building Code (FBC).

    - NFPA 70:  National Electric Code.

    - State Requirements for Educational Facilities (SREF).

  - SUBMITTALS

    - Product Data:  Provide complete descriptive literature on the portable buildings, including windows, markerboards, tackboards, roofing material, lighting, air conditioning system, finish surfaces, insulating values, electrical devices, and general construction information.

    - Shop Drawings:  Drawings and specifications signed and sealed by a Florida Registered Engineer.

  - QUALITY ASSURANCE

    - Certify the portable buildings to meet or exceed the requirements of the State Requirements for Educational Facilities (SREF).

    - Portable Buildings:

      - Conform to the requirements of the Florida Building Code.

    - Provide bonding and grounding per NFPA 70.

  - DELIVERY, STORAGE and HANDLING

    - Coordinate delivery and setup time with project phasing plan.

    - Protect structure, finish surfaces and equipment during handling and installation.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturers:  Subject to compliance with the specified requirements, provide products by one of the following manufacturers:

      - Diamond Engineered Space, Inc. 931 Taft-Vineyard Road where all relocated material Orlando, Florida 32826 (800) 228-5246

      - Scottsman Group, Inc. Williams Mobile Office P.O. Box 30185 Tampa, Florida 30185 (800) 782-1500

      - Space Master' International 2591 W. Orange Blossom Trail Apopka, Florida 32712 (407) 889-8600

  - PORTABLE CLASSROOM UNIT:

    - Classroom Units:

      - One complete factory assembled portable classroom unit.

      - Nominal size:  24 feet x 46 feet.

      - Unit:

        - New or in new condition at time of delivery.

        - Includes, but not be limited to the following; air conditioning, emergency lighting, general 	lighting, doors, windows, carpeting, ITV, fire alarm, telephone, intercom/ clock (call back button), electrical duplexes, exterior lights (porch), and entry awning.

      - Quantity Required:  One.

    - Certification:  Certify the unit to comply with the rules of the State Requirements for Educational Facilities (SREF).

- EXECUTION

  - INSTALLATION

    - Install portable building in compliance with the construction documents, the approved shop drawings, the manufacturer’s instructions, and at the location determined by the Owner.

    - Prepare site for portable by:

      - Removing any asphaltic concrete paving as necessary or required.

      - Relocate landscaping material.

      - Prepare for foundation.

      - Re-route irrigation lines, etc.

    - Provide pier foundations, blocking-up, anchoring, and tie downs as required.

    - Set secure in place, plumb, and level.  Anchor/tie-down portable to resist wind load per FBC.

    - Provide trenching and backfilling for electrical utilities, and water service.

    - Provide construction and connection of communication/intercom system from portable to existing school system.

    - Construct and connect fire alarm system for portable to existing school system.

    - Construct and connect ITV system for portable to existing school system.

    - Construct and connect telephone/data system for portable to existing school system.

    - Provide miscellaneous conduit and wiring for portable electrical system.

    - Provide electrical enclosures.

    - Provide load centers and panel boards.

    - Provide complete electrical service to portable.

    - Construct and install stairs, ramps, and asphaltic paving as required.

  - FIELD QUALITY CONTROL

    - Test each item of operational equipment to demonstrate that it is operating properly, and that the controls and safety devices are functioning.  Repair or replace devices found to be defective.

      - Demonstrate that the following items operate properly:

        - Control devices.

        - Power operating devices.

        - HVAC systems.

        - Switches, relays, remotes control switches, etc.

  - ADJUST and CLEANING

    - Prior to occupancy, clean all interior and exterior surfaces.

    - Adjust all doors/latches to operate freely without binding.

    - Verify that all lamps, wall switches, electrical devices, smoke detectors' fire alarms, etc. are operational.

    - Touch-up marred finishes to match adjacent surfaces.

  - MAINTENANCE

    - Provide maintenance on portable building for roofs, ramps stairs, seaming, etc. on an as needed basis.

  - REMOVAL of PORTABLE BUILDINGS and ACCESSORIES

    - At the appointed time, remove portable buildings with associated paving, fencing, foundations, tie-downs, anchorage’s, electrical and plumbing hook-ups, stairs ramps, etc.  Remove portables from the site and restore the grounds to their original condition.  Restore fencing, asphaltic paving, sod, irrigation lines, landscape material, etc.

END OF SECTION