# 16050 (26 05 00)

## BASIC MATERIALS AND METHODS

- GENERAL

  - SECTION INCLUDES

    - Provide all labor, materials, necessary equipment and services to complete the Basic Materials and Methods work, as indicated on the drawings, as specified herein or both, except as for items specifically indicated as “NIC ITEMS”.

    - Including but not necessarily limited to the following:

      - This section details equipment and methods common to more than one section of Division 16, Electrical, including:

        - Excavation.

        - Foundations and Supports.

        - Vibration Isolation.

        - Accessibility.

        - Painting.

  - RELATED WORK

    - Division 1 – General Requirements (all applicable sections).

    - Division 2 – Site work (all applicable sections).

    - Division 16 – Electrical (all applicable sections).

  - QUALITY ASSURANCE

    - Design Criteria:

      - The drawings are diagrammatic and due to small scale do not show in detail all required features of the work, and be supplemented by the Contractor’s knowledge and experience, and all items which are required to properly execute the full intent and meaning shall be considered as being shown and/or specified.

      - Right is reserved to make reasonable changes in outlet locations prior to roughing-in.

  - SUBMITTALS

    - Submit the following under provisions of Section 01330, “Submittal Procedures”.

      - Schematics and Interconnection Diagrams.

      - Detail Drawings and Shop Drawings.

      - Assembly, Operation and Maintenance Drawings & Manuals.

      - Manufacturer’s Data.

  - DELIVERY, STORAGE AND HANDLING

    - Conform to Section 01663, “Product Delivery, Storage and Handling Requirements”.

      - Keep all openings closed with plugs or caps to prevent intrusion of foreign matter.

      - Manufacturer’s Nameplates: Insure visible location of nameplate for each major component of equipment with manufacturer’s name, address, model number and rating.

  - JOB CONDITIONS

    - Sequencing Scheduling:

      - Coordinate adequate clearances for installation and maintenance of equipment and physical and electrical requirements of items and or equipment requiring connections.

      - Perform all necessary work to join with or receive other work, provide electrical service, extend conduits and make all necessary connections as required to prevent interruptions of service in any area.

      - Provide and extend as necessary, temporary electrical service for timely completion of Work for the Project.

- PRODUCTS

  - MATERIALS

    - Provide equipment to meet requirements of each respective section as indicated and suitable for installation shown.

    - Where two or more units of same equipment class are furnished, provide each from same manufacturer.

    - Provide raceways and enclosures for conductors throughout all systems as shown and specified.

    - Provide equipment and devices appropriate for environment and service intended, which are not constructed with housings for mounting and enclosing all live parts.

- EXECUTION

  - INSTALLATION

    - Excavation:

      - Conform to requirements of Section 02300, “Earthwork”.

      - Pitch all ducts away from building.

    - Foundations and Supports:

      - Mount equipment, conduit, duct and raceway on or suspended from building structure as indicated and specified.

      - Provide mounting boards for electrical equipment and telephone as follows:

        - For telephone and terminal cabinets provide backboards: Marine grade, plywood, 3/4 inch new and painted on all sides and edges with 2 coats of flame retardant flat gray oil paint for terminal cabinets and voice/data backboards.

        - For electrical panels provide hot dipped galvanized unistruct supports.

      - Assure adequate clearance in front equipment per N. E. C. Article 110-26.

    - Accessibility:

      - Locate with reference to finished buildings the installation of pull boxes, wire ways and other items requiring inspection, removal or replacement.

    - Painting:

      - Refer to Section 09900, “Painting” and manufacturer’s recommendations.

      - Recoat shop coat damaged in delivery or during construction.

      - Do not paint any nameplate, label or tag of any equipment.

    - Coordination with Existing Facilities Operation:

      - Alterations affecting any functioning existing facilities must not be started until all materials, supplies, equipment, labor, etc. is available so that continuous progress will be made to complete that work in the shortest time possible.

      - Work interfering with the existing facilities will be immediately suspended on request of the owner’s authorized representative and scheduled to such time as required without additional cost, unless such time involves overtime.  In this case additional payment will be made as otherwise provided for herein.

    - Ownership of Items Removed During Construction:

      - Conform to Section 01110, “Summary of Work”.

END OF SECTION