# SECTION 16090 (26 04 00)

## ALTERATIONS AND ADDITIONS TO EXISTING WORK

- GENERAL

  - SECTION INCLUDES

    - Provide all labor, materials, necessary equipment and services to complete the alterations and additions work, as indicated on the drawings, as specified herein or both, except as for items specifically indicated as “NIC ITEMS”.

  - RELATED WORK

    - Section 16132 – Conduit, Fittings and Supports.

    - Section 16400 – Service and Distribution.

    - Section 16510 – Interior Luminaires.

  - SUBMITTALS

    - Submit shop drawings and product data under provisions of with Section 01330, “Submittal Procedures”.

  - DESCRIPTION

    - These provisions are in addition to the requirements of building modifications as specified in other Sections of the specification.

    - Existing buildings will be occupied during construction.

    - Provide all additions and alterations to existing work required to maintain a complete and proper electrical installation.  Relocate existing electrical work for other trades required to complete the work and to maintain building in service.

    - Relocate luminaires, pull boxes, electrical ducts and other similar items, to permit the installation of new equipment.

    - Install new conduits, conductors, wiring and wiring devices, to maintain temporary and permanent use of electrical facilities.

    - Remove existing surface mounted work that is not used and blank off concealed outlets.

    - Reconnect existing work to be maintained and all outlet boxes and devices accessible after installation of all trades.

    - Expose new work in existing areas on walls in unfinished areas and concealed in new construction.  Where cutting and patching is to be provided, match finishes with existing surface finishes.  In existing finishes areas, conceal all work in finishes.

- PRODUCTS

- ##NOT USED##

- EXECUTION

  - SEQUENCE OF WORK

    - Schedule electrical feeders, branch wiring, signal wiring and other similar work to correspond with the sequence of work necessary to demolish, remove and construct new work.

  - OPENINGS IN EXISTING WORK

    - Verify exact locations and materials before performing work.  Project Consultant must give written approval for cutting of structural members and bearing walls.

  - VERIFICATION OF EXISTING WORK

    - Where shown on drawings, work which is “existing” is assumed to be in place and suitable for the necessary alterations and additions required.  Field check Items and alterations included as may be necessary for proper installation.

  - REMOVAL AND OWNERSHIP OF EXISTING WORK

    - Conform under provisions of Section 01110, “Summary of Work”.

    - Remove existing electrical work where necessary.  Unless otherwise specified, or indicated by Owner as obsolete or unusable, equipment and materials remain the property of the Owner.

    - Deliver property of Owner to a location where directed by the Owner and promptly remove other items from the job site.

END OF SECTION