# SECTION 16129 (26 05 34)

## TERMINAL LUGS

- GENERAL

  - SECTION INCLUDES

    - Provide all labor, materials, necessary equipment and services to complete the Terminal Lugs work, as indicated on the drawings, as specified herein or both, except as for items specifically indicated as “NIC ITEMS”.

  - RELATED WORK

    - Section 16120 – Wires and Cables.

    - Section 16220 – Motor Starters.

    - Section 16400 – Services and Distribution.

    - Section 16413 – Safety Switches.

    - Section 16430 – Low Voltage Switchgear.

    - Section 16440 – Panelboards.

  - SUBMITTALS

    - Submit product data under provisions of Section 01330, “Submittal Procedures”.

- PRODUCTS

  - MATERIALS

    - Terminal lugs: of the pressure indent type requiring the use of a special tool and UL rated for copper conductors.

    - Branch circuit conductor terminal lugs for stranded wire sizes No.10 AWG and smaller: the uninsulated ring or spade type.

      - T & B “STA-KON”.

      - Burndy “HYLUG”.

      - 3M “SCOTCHLOK” crimp terminals.

    - Branch or feeder circuit conductor terminal lugs for wire sizes No. 8 AWG to No. 2 AWG:

      - T & B “COLOR-KEYED”.

      - Burndy “HYLUG”.

      - Anderson “VERSA-CRIMP”.

    - Branch or feeder circuit conductor terminal lugs for wire sizes No. 1 AWG and larger: of the two-bolt whole type having bolt spacing conforming to NEMA Standards.  Lugs:

      - T & B “COLOR-KEYED”.

      - Burndy “HYLUG”.

      - Anderson “VERSA-CRIMP”.

- EXECUTION

  - INSTALLATION

    - Provide complete installation under strict provisions of manufacturer’s standard printed specifications, instructions and recommendations.

END OF SECTION