SECTION 02070 (02 41 14)

MINOR DEMOLITION FOR REMODELING

- GENERAL

- SECTION INCLUDES

- Removal of designated building equipment and fixtures, and disposition of debris legally off-site.

- RELATED SECTIONS

- Section 01110-Summary of Work.

- Section 01352-Indoor Air Quality (IAQ) Management,

- Section 01560-Temporary Barriers and Enclosures.

- Section 01572-Construction Waste Management.

- Section 01770-Closeout Procedures.

- SUBMITTALS

- Submit schedule indicating proposed sequence of operations for selective demolition work to Owner for review and approval prior to commencement of work. Include method of demolition and plan for remodeling work, coordination for shut-off, capping, continuation of utility services as required, together with details dust noise control protection.

- Certification: Submit copy of demolition firms’ current license to operate in Broward County, Florida.

- Coordinate with Owner's continuing occupation of portions of existing building and with Owner's partial occupancy of completed new addition, alteration, and renovation.

- PROJECT RECORD DOCUMENTS

- Submit under provisions of Section 01780, “Closeout Submittals”.

- Accurately record actual locations of capped utilities, subsurface obstructions, and unanticipated structural, mechanical and electrical elements uncovered during demolition.

- QUALITY ASSURANCE

- Organize and perform demolition work to avoid damage to construction intended to remain.

- Handle waste materials as specified in Section 01572–Construction Waste Management.

- Demolition and Transportation of Debris:  Shall comply with applicable codes and regulations governing these operations.  Fees are paid by the Contractor.

- Conduct demolition and removal operations in an expedient manner, with precautions taken to prevent demolition site from being an “attractive nuisance.”

- Notify the Board and A/E of any conditions capable of affecting the safety of occupants of adjacent buildings, the normal use of these facilities, or the physical condition of the structures.

- In case of accidental disruption of utilities or the discovery of previously unknown utilities, stop work immediately and notify the Board and A/E.

- Do not continue work until the Board, A/E and Contractor agree on a plan to correct the situation or identify utility service line.

- REGULATORY REQUIREMENTS

- Conform to Florida Building Code, including applicable supplements for demolition work, safety of structure, dust control and safeguards required during construction.

- Notify affected utility companies before starting work and comply with their requirements.

- Do not close or obstruct egress width to exits.

- Do not disable or disrupt building fire or life safety systems without 3 day prior written notice to the Owner.

- If the Contractor believes asbestos bearing or other hazardous products have been encountered during demolition, immediately stop work in the affected area.  Evaluate the affected area and notify the Owner.  Do not resume Work in the affected area until written direction from the Owner is received.

- Lead Safety:  Beginning April 22, 2010, federal law (EPA’s Final Rule 40 CFR Part 745) shall require that contractors and renovators performing renovation, repair, and painting projects that disturb lead based paint in homes, child care facilities, and schools built before 1978 that a child under age 6 visits regularly, to be certified and follow lead-safe work practices to prevent lead contamination.  Contractors and renovators must be EPA Certified and projects must comply with the EPA manual “Renovate Right, Important Lead Hazard Information for Families, Child Care Providers and Schools”.

- SEQUENCING

- Sequence work in phases under the provisions of Section 01110, “Summary of Work”.

- SCHEDULING

- Schedule work under the provisions of Section 01110, “Summary of Work”.

- Schedule demolition work in gymnasium area to coincide with time restrictions for interior work as specified in Section 01110, “Summary of Work”.

- Describe demolition removal procedures and schedule.

- JOB CONDITIONS

- Owner will be continuously occupying areas of building and site immediately adjacent to areas of selective demolition. Conduct demolition work in manner that will minimize disruption of Owner's normal operations. Provide minimum of five (5) working days advance notice to Owner of demolition activities, which will severely impact Owner's normal operations.

- Existing work not specified for removal that is temporarily removed, damaged exposed or in any way disturbed or altered by removal work shall be repaired, patched, or replaced to the Board and A/E’s satisfaction at no additional costs to the Board.

- Provide barriers and warning devices to protect the public and users of adjacent facilities.

- If hazardous materials are encountered during demolition operations, comply with applicable regulations, laws, and ordinances concerning removal, handling and protection against exposure or environmental pollution.

- PRODUCTS

- ## Not Used ##

- EXECUTION

- PREPARATION

- Cover, protect adjacent finished building surfaces (walls, floors, ceilings, etc.), furniture, equipment and fixtures to remain from soiling or damage when selective demolition work is performed in rooms or areas from which items have not been removed.

- Erect; maintain dust-proof partitions, closures as required to prevent spread of dust or fumes to occupied portions of the building.

- Where selective demolition, which will create excessive dust, occurs immediately adjacent to or within occupied portions of building, construct dust-proof partitions or barriers to mitigate spread of airborne dust or debris.

- Provide weatherproof closures for exterior openings resulting from selective demolition work.

- Locate, identify, stub off and disconnect utility services that are not indicated to remain.

- Provide by-pass connections as necessary to maintain continuity of service to occupied areas of building. Provide minimum of five (5) working days advance notice to Owner if shutdown of services is necessary.

- DEMOLITION REQUIREMENTS

- Perform selective demolition work in a systematic manner. Use such methods as required to complete work indicated on Drawings in accordance with selective demolition schedule, governing regulations.

- Cease operations, evacuate, and notify the Owner's Representative immediately if safety of structure appears to be endangered. Take precautions to support structure until determinations is made for continuing operations.

- Maintain protected egress and access to the work.

- DECONSTRUCTION

- Inspect and evaluate existing materials indicated for reuse on site.

- Disassemble existing construction scheduled to be removed for recycling or reuse.

- Salvage materials for recycling and reuse as indicated.

- Materials scheduled for reuse and recycling off site per “Section-01572 Construction Waste Management”.

- DEMOLITION

- Disconnect, remove, cap, and identify designated utilities within demolition areas.

- Demolish in an orderly and careful manner. Protect existing supporting structural members and all items to remain.

- Promptly remove debris to avoid imposing excessive loads on supporting walls, floors or framing.

- If unanticipated mechanical, electrical or structural elements which conflict with intended function or design are encountered, investigate, measure both nature, extent of the conflict.  Submit report to Owner in written, accurate detail.

- Pending receipt of directive from Owner, rearrange selective demolition schedule as necessary to continue overall job progress without delay.

- SALVAGE ITEMS

- Existing equipment, fixtures stored on site, protected under cover, as indicated on drawings or as directed by Project Consultant or Owner, for re-use stored on site, protected under cover, reinstalled by contractor as required.

- DISPOSAL OF DEMOLISHED MATERIALS

- Remove debris; rubbish other materials resulting from selective demolition operations from building site.  Legally transport, recycle, or dispose of materials off site on a regular basis.

- Accumulation of debris on the site will not be allowed.

- Burning of removed materials will not be permitted on project site.

- REPAIR

- Repair demolition performed in excess of that required. Return damaged structures, surfaces to remain to condition existing prior to commencement of selective demolition work.  Repair adjacent construction on surfaces soiled or damaged by selective demolition work.

- CLEAN UP

- Upon completion of selective demolition work, remove tools, equipment, and demolished materials from site.  Remove protections, leave interior areas broom clean.

END OF SECTION