# SECTION 02222 (02 41 16.13)

## BUILDING DEMOLITION

- GENERAL

- SECTION INCLUDES

- This Section requires removal and disposal, off site, of the following:

- Building structures as indicated on drawings, except salvageable items that Owner will remove prior to start of work.

- Entrance drive, parking areas, covered walkway, playfields and courts, etc. to limits indicated on drawings.

- Building foundations and supporting walls to a uniform depth of 12 inches below lowest foundation elevation.

- RELATED SECTIONS

- Section 01352-Indoor Air Quality (IAQ) Management.

- Section 01572-Construction Waste Management.

- Section 02200-Earthwork.

- JOB CONDITIONS

- Occupancy:  Structures to be demolished will be vacated and use discontinued prior to start of work.  Asbestos abatement will be completed by others under a separate contract with the Broward County School Board prior to demolition work performed under this contract.

- Notice: The Contractor is subject to potentially severe fines imposed by the United States Environmental Protection Agency in the event that the Contractor damages or in any other way violates the protective envelope surrounding the work of the Asbestos Abatement Contractor.  Conduct or supervise all construction operations to prevent interference with concurrent asbestos abatement operations or damage to the Asbestos Abatement Contractors protective envelope.  Instruct all construction personnel to avoid contact with removed materials and unauthorized entry into restricted portions of the building until they are released to the Contractor for his operations.

- If asbestos or other hazardous material is encountered on site, the Contractor is to stop work in the area of discovery and notify Project Consultant and Owner immediately.  In cases where there is an immediate health hazard to persons on, or off site, the Contractor should take action to immediately alleviate the emergency.

- CONDITION OF STRUCTURES

- Owner assumes no responsibility for actual condition of structures to be demolished.

- Conditions existing at time of inspection for bidding purpose will be maintained by Owner.  However, variations within structure may occur by Owner's removal and salvage operations prior to start of demolition work.

  - QUALITY ASSURANCE

  - Handle waste materials as specified in Section 01572-Construction Waste Management.

  - SALVAGED MATERIALS

  - Items of salvageable value to Contractor may be removed from structure as work progresses. Transport salvaged items from site as they are removed.

  - Storage or sale of removed items will not be permitted on site.

  - EXPLOSIVES

- A.	Use of explosives will not be permitted.

- TRAFFIC

- Conduct demolition operations and removal of debris to ensure minimum interference with roads, streets, walks, and other adjacent occupied and used facilities.

- Do not close or obstruct streets, walks, or other occupied or used facilities without permission from authorities having jurisdiction.  Provide alternate routes around closed or obstructed traffic ways if required by governing regulations.

- PROTECTIONS

- Ensure safe passage of persons around area of demolition.  Conduct operations to prevent damage to adjacent buildings, structures, and other facilities and injury to persons.

- Erect temporary covered passageways as required by authorities having jurisdiction.

- Provide interior and exterior shoring, bracing, or support to prevent movement, settlement, or collapse of structures to be demolished and adjacent facilities to remain.

- DAMAGES

- Promptly repair damages caused to adjacent facilities by demolition operations.

- UTILITY SERVICES

- Maintain existing utilities indicated to stay in service and protect against damage during demolition operations.

- Do not interrupt existing utilities serving occupied or used facilities, except when authorized in writing by authorities having jurisdiction.  Provide temporary services during interruptions to existing utilities, as acceptable to governing authorities.

- Owner will shut off utilities serving structures. Disconnecting and sealing indicated utilities before starting demolition operations is part of this work.

- Utility Services: Do not start demolition work until utility disconnections have been completed and verified in writing.

- PRODUCTS

- ## NOT USED ##

- EXECUTION

- DEMOLITION

- Pollution Controls: Use water sprinkling, temporary enclosures, and other suitable methods to limit dust and dirt rising and scattering in air.  Comply with governing regulations pertaining to environmental protection.

- Do not use water when it may create hazardous or objectionable conditions such as ice, flooding, and pollution.

  - Clean adjacent structures and improvements of dust, dirt and debris caused by demolition operations.  Return adjacent areas to condition existing prior to start of work.

  - BUILDING DECONSTRUCTION

  - Inspect and evaluate existing materials indicated for reuse on site.

  - Disassemble existing construction scheduled to be removed for recycling or reuse.

  - Salvage materials for recycling and reuse as indicated.

  - Materials scheduled for reuse and recycling off site as specified in Section - 01351 Construction Waste Management.

  - BUILDING DEMOLITION

  - Demolish buildings completely and remove from site.  Use such methods as required to complete work within limitations of governing regulations.

  - Small structures may be removed intact when acceptable to Owner and approved by authorities having jurisdiction.

  - BELOW-GRADE CONSTRUCTION

  - Demolish foundation walls and other below-grade construction, including concrete slabs, to a depth of not less than 12 inches below lowest foundation level.

  - FILLING BASEMENT AND VOIDS

  - Completely fill below-grade areas and voids resulting from demolition of structures.

  - Use satisfactory soil materials as defined in ASTM D 2487, consisting of stone, gravel and sand, free from debris, trash, roots and other organic matter.

  - Prior to placement of fill materials, ensure that areas to be filled are free of standing water, trash and debris.

  - Place fill materials in horizontal layers not exceeding 12 inches in loose depth.  Compact each layer at optimum moisture content of fill material to a density equal to original adjacent ground, but not less than 98 percent density when tested under provisions of ASTM D 1557-91, Method D, unless subsequent excavation for new work is required.

  - After fill placement and compaction, grade surface to meet adjacent contours and to provide flow to surface drainage structures.

  - DISPOSAL OF DEMOLISHED MATERIALS

  - General:  Remove promptly from site-accumulated debris, rubbish and other materials resulting from demolition operations.

  - Burning of combustible materials from demolished structures will not be permitted on site.

  - Removal:  Transport materials removed from demolished structures and recycle or legally dispose off site.

END OF SECTION