# SECTION 02280 (31 31 00)

## SOIL TREATMENT

- GENERAL

- SECTION INCLUDES

- Soil treatment below slabs-on-grade for subterranean insects.

- Soil treatment at interior and exterior foundation perimeter, for subterranean insects.

- Soil treatment under all concrete floor slabs on grade or fill.

- Where exterior foundation walls abut paving, sidewalk slabs, or other permanent surfacing.

  - RELATED SECTIONS

  - Section 02200-Earthwork.

  - Section 02931-Sodding.

  - Section 02950-Trees, Plants, and Groundcover.

  - Section 03100-Concrete Formwork.

  - SYSTEM DESCRIPTION

  - Termiticide and Herbicide application to soil under new buildings and athletic paved areas as indicated on drawings and specified in this section.

  - Use of chlordane, heptachlor, aldrin, or dieldrin is not allowed.

  - SUBMITTALS

  - Submit the following for review before starting work:

  - Product data, including manufacturer’s specifications, chemical analysis, with recommended dilution, application directions, and safety precautions.

  - Sample copy of applicator’s warranty for review.

  - Applicator’s experience evidence with copies of current local and state licenses and current Certified Operator-in-Charge certificate.

  - QUALITY ASSURANCE

  - Work shall be done by a bonded Contractor whose principal business is pest control and termite treatment and can show evidence of at least 5 years of successful operation in this field.  Contractor to be licensed according to the State of Florida and Broward County Requirements to apply soil treatment solution.

  - Products:

      - Comply with United States Environmental Protection Agency (EPA) regulations for termiticides.

      - Comply with requirements of the Broward County Health Department and Office of Natural Resource Protection.

      - Registered with Florida Department of Agriculture and Consumer Services.

  - STORAGE 	and PROTECTION

  - To avoid surface flow or over spray of toxicant from application site, do not apply soil poisons when soil or fill is excessively wet or after heavy rains.

  - Unless treated areas are to be immediately covered, take precautions to prevent disturbances of treatment by human or animal contact.

  - Comply with applicable laws, codes, and ordinances of Federal, State, and local regulatory agencies having jurisdiction over use of soil poisons.

  - Provide warning signs and instruct workers to use protective measures for their safety.

  - PROJECT RECORD DOCUMENTS

  - Submit documents under provisions of Section 01780, “Closeout Submittals”.

  - Submit warranty:  Properly executed, signed by a duly authorized officer or principal and notarized.

  - WARRANTY

  - Upon completion of soil treatment and as a condition of substantial completion, furnish the Owner with a written warranty, from the applicator, that shall provide that:

  - Application was made at concentration, rates, and methods complying with manufacturer’s written instructions.

  - Effectiveness of treatment is warranted for not less than 5 years without additional cost to the Owner, by means of a 5-year repair and replacement bond.

  - Upon evidence of subterranean termite activity, retreat area at no additional charge to the Owner.  Additional treatment shall be sufficient to prevent termites from attacking building or its contents.

  - Upon occurrence of damage to building or to its contents within warranty period, retreat soil and replace damage at no cost to the Owner.

  - Warranty bond shall be drawn in favor of the Owner, successor, or assigns and shall be non-cancelable by all parties to the contract except the Owner.

  - PRODUCTS

  - MATERIALS

  - Soil Treatment:

  - Mfr:  BASF, product:  Termidor SC.  EPA Registration No. 7969-210.

  - Mfr;  Bayer Environmental Science, product:  Premise 75.  EPA Registration No. 432-1332.

  - Mfr:  FMC Corporation, product:  Dragnet SFR. EPA Registration No. 279-3062.

  - Mfr:  Syngenta Crop Protection, Inc., product:  Demon Max. EPA Registration No. 100-1218.

  - EXECUTION

  - APPLICATION

  - Apply termiticide mixture to the following:

  - Soil and earth that will be covered by or lie next to buildings.

  - Masonry foundations.

  - Areas around pipes and conduits penetrating slabs on fill to provide a lethal barrier to subterranean termites

  - Apply termiticide mixture after subgrade has been made ready for placement of any floor slab vapor barrier, and as soon as practical before placement of concrete slabs and caps on masonry piers.  Piling, pile caps, grade beams, foundation walls, and below grade waterproofing shall have been completed.

  - Soil Conditions:  Apply termiticide mixtures when moisture content soil is sufficiently low to allow uniform distribution of chemical throughout specified areas.

  - Application Under Slabs on Fill:

  - Apply termiticide mixtures uniformly to all areas beneath concrete slabs-on-grade, including beneath walkways and entrance platforms and beneath sidewalks within 5 feet of buildings.

  - Apply at rate per manufacturer’s written instructions.

  - Ground areas beneath concrete slabs-on-grade and paving abutting building slabs shall be similarly treated for a distance not less than 3 feet from building.

  - Application Along Foundation Walls, Pipes, and Conduits:

  - Treat critical areas along both sides of exterior and interior foundation walls, columns, and around utility pipes, conduits, ducts and other similar items extending through soil beneath and next to new construction, to a depth, width, and rate per manufacturer’s written instructions.

  - Mix chemical with soil as it is placed against walls and utility lines.

  - Apply at rate per manufacturer’s written instructions.

  - Application to Masonry Foundation Walls:  Treat voids of unit masonry foundation walls, top of course occurring at or just above grade level, with additional treatment   Apply at rate per manufacturer’s written instructions.

  - Retreatment of Disturbed Soil:  Retreat soil surfaces disturbed after treatment and before placement of slabs and covering structures.

  - CLEAN UP

  - Improper disposal of pesticide, spray mixture, or rinsate is a violation of federal law.  Comply with manufacturer’s instructions for disposal of these materials and empty containers.  Do not allow supplies of chemicals to remain on site unattended.

END OF SECTION