# SECTION 02580 (32 17 23)

## PAVEMENT MARKING

- GENERAL

  - RELATED SECTIONS

    - 02551-Maintenance of Traffic.

    - 02952-Asphaltic Concrete Repair and Sealing.

    - 02890-Traffic Signs and Signalization.

  - REFERENCE

    - Florida Department of Transportation (FDOT) 1986 Standard Specifications for Road and Bridge Construction, except basis of payment and other pay measurement requirements.

    - Manual on Uniform Traffic Control Devices for Streets and Highways published by the U.S. Department of Transportation, Federal Highway Administration, 1978.

    - Miami-Dade County Highway Department, Signalization and Traffic Section.

  - SUBMITTALS

    - Submit for review before starting work, paint tests, as specified in Section 971 of FDOT Specifications and as applicable to specified material.

  - QUALITY ASSURANCE

    - Perform all work in a neat and accurate manner.

    - All equipment shall be of a type and design that will readily obtain the required uniformity of application of the pavement markings both as to thickness of coating and alignment.

  - MAINTENANCE

    - Spare Paint:  Provide the Board with a minimum of 5 gallons of traffic paint from the same batch used in application of pavement markings.  Also provide paint specifications and the manufacturer’s identification number of the paint used.

- PRODUCTS

  - MATERIALS

    - Traffic Paint:  According to the requirements as specified in Section 971-12 of the FDOT Specifications and colors indicated on Drawings.

    - Court Paint:  By Caribbean or accepted equivalent.

      - #260-0307 Yellow.

      - #260-0315 White.

- EXECUTION

  - INSPECTION

    - Do not proceed with the work of this section until conditions detrimental to the proper and timely completion of the work have been corrected in an acceptable manner.

  - APPLICATION

    - Time of Application:  Painting shall be done only during daylight hours and, as far as practical, shall be terminated in time to allow sufficient drying by sunset.

    - Weather Limitations:  No paint shall be applied when any moisture is present on the surface to be painted or when air temperature is below 40 degrees F.  Painting shall not be done when winds are sufficient to cause spray dust.

    - Preparation of Surface to be Painted:  The surface to be painted shall be cleaned by compresses air or other effective means immediately before the start of painting.  Surface shall be clean and dry when the paint is applied.  Any vegetation or loose soil shall be removed from the pavements before striping is begun.

    - Mixing Paint:  The paint shall be thoroughly mixed before it is poured into the painting machine and no thinning of the paint in the machine will be allowed.  Before the start of each days work the paint container, the connections and the spray nozzles in the machine shall be thoroughly cleaned with a suitable cleaner.

    - Paint Application:  The traffic markings shall be of the specified dimensions with clean, true edges and without sharp breaks in the alignment.  A uniform coating of paint shall and the finished markings shall contain no light spots or paint skips.  Any stripes not having a uniform, satisfactory appearance, both day and night shall be corrected.

    - Rate of Paint Application:  The minimum rate of applications for paint shall be as follows:

      - Four-inch solid parking stall strips:  18.5 gallons per mile.

      - Any other width of stripe or markings:  A direct proportion of the above.

    - Required Film Thickness:  The minimum wet film thickness for all painted areas shall be 15 mils.

    - Court Paint:  Lay out yellow basketball lines over white volleyball lines.

    - Alignment of Strips:  Where a stripe deviates from the correct alignment, as indicated by the string line, by more than one inch in any 20 feet length, it shall be obliterated and the stripe corrected as specified in paragraph “Corrective Measures.”

  - PROTECTION

    - Protection of Stripes:  All newly painted stripes, or other markings, shall be protected until the paint is sufficiently dry to allow vehicles to cross marking without damage from the tires.

    - Repair of Damaged Areas:  Any portions of the stripes damaged by passing traffic or from and other cause shall be repainted at the Contractor’s expense.

  - DIMENSION AND ALIGNMENT TOLERANCE

    - Dmensions:  No marking shall be less than the specified width.  No markings shall exceed the specified width by more than 1/2 inch.  Alignment tolerances shall be as specified in paragraph 3-01 H.

    - Correction Rates:  Any corrections of variation in the width of in the alignment of stripes shall not be made abruptly but the stripes shall be returned to the design width at the rate of at least 10 feet for each 1/2 inch of correction.

  - CORRECTIVE MEASURES

    - Painted markings failing to meet the specifications, including the permissible tolerances and the appearance requirements, or are marred, damaged by traffic, or from other causes, shall be corrected at the Contractor’s expense.  Drip and splattered paint shall be removed.  Whenever it is necessary to remove paint it shall be done by means that will not damage the underlying surface of the pavement.  When necessary to correct a deviation that exceeds the permissible tolerance in alignment, that portion of the stripe affected shall be removed and repainted according to these specifications.

    - Corrective Devices:  Misalignment, defective surfaces, etc., shall be corrected by chemical agents, or by any other type of mechanical device that will effectively remove the paint without damage to the pavement surface, or prevent the reapplication of markings.

END OF SECTION