# SECTION 02890 (34 41 14)

## TRAFFIC SIGNS AND SIGNALIZATION

- GENERAL

  - SUMMARY

    - The Scope of work of this section includes the provision of various traffic signs throughout Site to ensure safe vehicular and pedestrian traffic.

    - This Section includes the following:

      - Aluminum Panel Signs.

      - Pavement Markings.

      - Pedestrian Signals assembly, School Flashers, Detector assembly, Mast Arms, Power Supplies and Components.

  - SUBMITTALS

    - Product Data:  For each product indicated.

    - Shop Drawings:  Include plans, elevations, sections, details, wiring diagrams and attachments to other work.

      - Verify dimensions by field measurements before fabrication and indicate measurements on Shop Drawings.

      - Provide message list for each sign, including large scale details of wording and lettering.

    - Samples:  For each sign material indicated that involves color selection.

  - QUALITY ASSURANCE

    - Regulatory Requirements:

      - Signs and Signalization shall conform to Standards of Broward County and Florida Department Of Transportation (FDOT) Standard Index 600-660, 11860-11865, 17302-17346, 17355,17870 and FDOT Specifications and Construction Details.

      - Coordinate all work to be performed with appropriate approvals from Broward County and local municipal traffic engineering departments.

      - All Signalization and Warning Flashing Components shall conform to FDOT and Broward County Roadway and Traffic Design Standards and Manual Of Uniform Traffic Devices.

      - Traffic regulating signs:  Conform to the colors, dimensions and requirements of the Manual on Uniform Traffic Control Devices for Streets and Highways (ANSI D6.1).

      - Signage and pavement markings:  Conform to FDOT and Broward County Traffic Engineering Department Requirements.

      - Americans With Disabilities Act (ADA):  Provide conforming pavement markings and signage.

- PRODUCTS

  - MATERIALS:

    - Sign panels:  Aluminum Association Alloy 6061-T6, .080 gauge, unless otherwise     specified.

    - Support members:  Aluminum Association Alloy 6061-T6.

    - Bolts:  Aluminum Association Alloy 2024-T4 with an anodic coating 0.0002 inch thick minimum and chromate sealed.

    - Lockwashers:  Aluminum Association Alloy 7075-T6 with an anodic coating 0.0002-inch thick minimum and chromate sealed.

    - Nuts:  Aluminum Association Alloy 6262-T9.

    - Concrete:  3000 psi at 28 days.

    - Reflective sheeting:  FDOT Type A.

  - FABRICATION:

    - Prepare sign blanks and fabricate reflectorized sign faces to conform to the applicable requirements of FDOT Standard Specifications for Road and Bridge Construction, 1996, Sections 7004 and 700-5.

- EXECUTION

  - INSTALLATION

    - General:  Locate signs and accessories where indicated, using mounting methods in compliance with manufacturer's written instructions and all applicable codes and Standards of  the FDOT Roadway and Traffic Design Manual.

      - Install signs level, plumb, and at heights indicated, with sign surfaces free from distortion and other defects in appearance.

  - FIELD QUALITY CONTROL

    - Routine inspection of proposed work must be coordinated with Broward County Traffic Engineering and the Broward County School District inspection department.

END OF SECTION