# SECTION 02953 (32 01 16.76)

## ASPHALT PAVING FOR RESURFACING EXISTING

### PLAY COURTS AND TRACKS

- GENERAL

  - SECTION INCLUDES

    - Provide all materials, equipment and labor to complete the resurface paving work shown on plans.

- PRODUCTS

  - MATERIALS

    - Use first grade products of reputable manufacturers or suppliers.  Any substitution must be approved for use by the Owner prior to installation.

  - TACK COAT

    - Asphalt cement, Penetration Grade 85-100 for emulsified asphalt, Grade RS-2.

  - WEARING SURFACE

    - Asphaltic Concrete, Type S-III, Modified for track, outside basketball courts, tennis courts. Conform to Florida State Road Department Standard Specifications for Road and Bridge Construction, latest edition.

- EXECUTION

  - PREPARATION OF EXISTING PAVEMENT

    - Grass and Weed Extermination:

      - Apply weed killer over entire asphalt surface to be re-surfaced.  Use Round Up or approved equal as per manufacturer’s recommendations for noncrop areas. Repeat as required.

    - Patching and Leveling:

      - Prior to resurfacing, patch the existing asphalt surface and level as required to assureproper drainage.

  - INSTALLATION

    - Tack Coat:  Apply a tackcoat at rate of 1 gallon per 10 square yards.

      - When weather conditions delay installation of wearing surface, seal the surface with emulsified asphalt at rate of 2 gallons per 10 square yards.

    - Wearing Surface:  minimum 1 inch thickness and laid only when the surface is dry and when weather conditions are suitable.

      - Spread mixture by an approved mechanical spreader.

      - Spreading by hand is approved only in areas where it is impracticable to use equipment.

      - Compacting mixture:  Done by rolling with the following equipment and sequence:

        - Seal rolling: using tandem steel rollers weighing between 5 tons to 8 tons, and following as close behind the spreaders as possible.

        - Final rolling:  Done with 5 ton to 8 ton tandem steel rollers and done after the seal rolling is completed, but before the temperature has dropped below 140 degrees Fahrenheit.  Continue this rolling until all roller marks have been eliminated.

      - Open edges:  sawcut straight.

      - Upon completion of the resurfacing:

        - The finished surface:  such that it will not vary more than 1/4 inch in 15 feet when measured in any direction.

        - Contractor:  flood the resurfaced area to confirm that all water drains off properly with no “bird baths”.

        - Perform this procedure in the presence of the Owner.

        - Rework areas that pocket water to a depth of more than 1/16 inch until positive drainage is achieved.

      - Testing:  Compaction tests and other testing as required by the specification and as directed by the Owner are under the direction of a Professional Engineer or Testing Laboratory, registered and licensed in Florida.  Testing costs will be paid for by the Owner, except those tests failing to meet requirements and all retesting required as a result of failure to meet specifications.  The Contractor is to recompact unsatisfactory work and pay for retesting.

END OF SECTION