# SECTION 04270 (04 23 00)

## GLASS UNIT MASONRY

- GENERAL

  - SUMMARY

    - Related Sections:

      - 04200-Unit Masonry.

      - 07920-Joint Sealers.

  - REFERENCES

    - ASTM International (ASTM):

      - C144 Specification for Aggregate for Masonry Mortar.

      - C150 Specification for Portland Cement.

      - C270 Specification for Mortar for Unit Masonry.

      - E119 Test Methods for Fire Tests of Building Construction and Materials.

      - A82	 Specifications for Steel Wire, plain, for concrete reinforcement.

      - ASTM A153	 Specifications for Zinc Coating (hot-dip) on iron and steel hardware.

    - Florida Building Code.

    - American Concrete Institute (ACI):

      - ACI 530/530.1-99 Building Code Requirements and Specifications for Masonry Structures.

  - SUBMITTALS

    - Submit properly identified manufacturer's literature including installation instructions before starting work.

    - Submit 1 full size glass masonry unit proposed to be an accepted equivalent for testing by the Owner.

  - QUALITY ASSURANCE

    - Certification:  Provide certification by a recognized testing laboratory stating that glass block assembly meets a minimum 3/4-hour fire rating for installation in 1-hour walls.

- PRODUCTS

  - MANUFACTURERS

    - Glass Unit Masonry:

      - Pittsburgh Corning Corporation (PC).

      - Substitutions:

        - Will be considered by the Project Consultant and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

  - MATERIALS

    - Glass Blocks:  Units made of clean, colorless glass:

      - Thickset by PC, Endura, Decora, or Vue pattern with sizes and shapes as indicated on drawings.

      - Vistabrick by PC or accepted solid glass equivalent with sizes and shapes as indicated on Drawings.

    - Mortar:

      - Portland Cement: ASTM C150, Type I, Domestic.

      - Masonry Cement: ASTM E119, Domestic.

      - Sand: ASTM C144.

      - Water: Potable.

      - Mortar Mix: ASTM C270, type S proportioned by volume:

      - Prefabricated wire truss type reinforcement is required.  Both horizontal and vertical at not more than 6 inch cc galvanized to meet ASTM A153 Class B-2.  Acceptable manufacturers:

        - A.A. Wire Products.

        - Duro Wall.

        - Hohman of Bernard.

- EXECUTION

  - INSTALLATION

    - Lay glass masonry units plumb, true to line, with level and accurately spaced courses as indicated on drawings and according to manufacturer's printed instructions.

    - Comply with Florida Building Code, and ACI 530/530.1

  - CLEANING

    - Remove surplus mortar from faces of glass blocks and wipe dry.

    - Tool joints smooth and concave, before mortar takes final set.

    - Accomplish cleaning by means of an ordinary household scrub brush having stiff bristles.

    - Do not use abrasive cleaners (steel wool, wire brush, or acid) when removing mortar or dirt from the faces of glass block.

END OF SECTION