# SECTION 05551 (05 55 16)

## STAIR NOSING

- GENERAL

  - SECTION INCLUDES

    - Two-Component stair nosing systems.

    - Solid Bronze stair nosing systems.

    - Single Component stair nosing systems.

    - Non-slip abrasive strips and bars.

    - Stair nosing systems for special applications.

    - Stair nosing systems for renovations.

- ***************************************************************************************************************

- NOTE TO SPECIFIER:  Make a selection of from the following.

- ***************************************************************************************************************

    - Cast [Aluminum] [Iron] stair nosing systems.

  - RELATED SECTIONS

    - Section 01572-Construction and Waste Management.

    - Concrete work:  Section 03300.

    - Unit Masonry:  Section 04200.

  - REFERENCES

    - Publications listed herein are part of this specification to the extent referenced.  The criteria established in the specifications shall take precedence over the standards referenced herein. (Examples of reference standards are given below.)

    - ASTM International (ASTM):

      - ASTM B 221 Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Shapes and Tubes

    - State of California Title 24, Stripping code for the visually impaired.

    - International Organization for Standardization (ISO) 14021–1999; Environmental Labels and Declarations

    - OSHA Barrier Free Code

  - SYSTEM DESCRIPTION

- ***************************************************************************************************************

- NOTE TO SPECIFIER:  Make selections from the following.

- ***************************************************************************************************************

    - Stair nosing assemblies shall be [anchored to substrate] [cast-in-place].

    - Stair nosing treads shall be [solid] [ribbed].

    - Stair nosing tread surfaces shall be slip resistant.

    - Stair nosing treads shall be removable and replaceable.

    - Retrofit stair nosings shall be [bevel back] [square back].

    - Stair nosing treads shall meet OSHA Barrier-Free Code requirements for stair design in public buildings.

    - Stair abrasive bars shall be [1 inch] [2 inch] wide anti-slip bars.

    - Abrasives with cementious based resins shall not be acceptable.

  - QUALITY ASSURANCE

    - Manufacturer:  Furnish assemblies from 1 manufacturer with a minimum of 10 years of experience in the fabrication of stair nosing systems.

    - Installer:  Firm with not less than 3 years of successful experience in the installation of systems similar to those required by this project and acceptable to the manufacturer of the system.

  - SUBMITTALS

    - Product Data:

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of assembly.

    - Submit manufacturer’s specifications and technical data, including Material Safety Data Sheets, installation instructions, as required, and catalog cuts and templates where required to explain construction and to provide for incorporation into the project.

    - Submit certificates and/or copies of independent test reports or research reports showing compliance with specified material and performance requirements.

    - Submit shop drawings showing complete fabrication details for all stair nosings, including required anchorage to surrounding construction.

    - Submit three 6 inch samples of the specified system.

  - DELIVERY, STORAGE AND HANDLING

    - The manufacturer shall fill stair nosing sub-channels with a protective material to provide protection during construction.

    - Prior to shipment, the manufacturer shall cover the abrasive with heavy-duty tape to eliminate concrete infiltration and staining.

    - Deliver stair nosing system to jobsite in new, clean, unopened crates of sufficient size and strength to protect materials during transit.

    - Store components in original containers in a clean, dry location.

  - WARRANTY

    - Submit manufacturer’s warranty that materials furnished will perform as specified for a period of not less than 1 year when installed in accordance with manufacturer’s recommendations.

- PRODUCTS

  - ACCEPTABLE MANUFACTURERS

    - Balco, Inc., Grating Pacific, Inc., American Safety Tread, Co.

  - MATERIALS

    - All manufactured stair nosing products are to contain recycled content.

    - Stair Nosing System:

- ***************************************************************************************************************

- NOTE TO SPECIFIER:  Make selection from the following.

- ***************************************************************************************************************

      - Aluminum:  ASTM B 221, alloy 6063-T5 for extrusions.

      - Bronze:  Alloy 385 for extrusions.

      - Cast Aluminum:  Class 30 Aluminum, tread plate.

      - Cast Iron:  Class 35, tread plate.

    - Abrasive:  Two part Epoxy combined with aluminum oxide grit.

    - Fasteners required for complete installation to manufacturer’s instructions

- ***************************************************************************************************************

- NOTE TO SPECIFIER:  Make selection from the following.

- ***************************************************************************************************************

      - Two Component Stair Nosings: Standard and Tread plate securing screws.

      - Single Component Stair Nosings: [Standard] [Barbed] [Continuous] [Strap] anchors.

      - Special Applications Stair Nosings: [Standard] [Concrete Screws].

      - Stair Nosings for Renovations: [Wood Screws] [Strap Anchors] [Concrete Screws]

      - Cast Stair Nosings: Wing Anchor.

  - FABRICATION

    - Fabricate stair nosing assemblies as detailed.  Provide anchors and accessories necessary for complete installation.

      - Provide specified anchors and, where required, tread plate securing screws.

      - Surfaces to be embedded in concrete shall be coated with a clear acrylic lacquer.

    - Package components with anchors.

  - FINISHES

    - Aluminum subchannels and tread plates shall be:

      - Mill finish.

      - Heat-treated for strength.

      - Clear acrylic lacquer coated for components to be embedded in concrete.

    - Bronze tread plates shall be:

      - Satin finish.

    - Color to be selected by the Architect.

    - Cast Iron shall be prime painted.

- EXECUTION

  - EXAMINATION

    - Installer shall examine conditions under which work is to be performed and shall notify the contractor in writing of unsatisfactory conditions. Installer shall not proceed until unsatisfactory conditions have been corrected in a manner acceptable to the installer.

  - INSTALLATION

    - Install stair nosings in accordance with the most recent adopted edition of the Florida Building Code, The Authority having jurisdiction, the industry standards applicable to the work, and the manufacturer’s written installation instructions.

      - Sub-channels of two component stair nosings shall be installed with the pour.

      - Abrasive tread plates of two component stair nosings shall be installed in the final stages of construction.

    - Work shall be aligned plumb, level, and, where required, flush with adjacent surfaces and rigidly anchored to the substrate.

  - CLEANING

    - Clean exposed surfaces as recommended by the manufacturer.

  - PROTECTION

    - Protect finished work from damage by work of other trades.

    - Finished units shall be without damage. Units damaged during shipping or construction shall be repaired in accordance with the contract requirements.

  - GENERAL RESPONSIBILITY

    - Any variation from this specification resulting in additional cost to any other contractor or subcontractor on this project shall be the sole financial responsibility of the contractor for the work of this section.

END OF SECTION