# SECTION 06400 (06 40 00)

## ARCHITECTURAL WOODWORK

- GENERAL

  - SECTION INCLUDES

    - Casework.

    - Work Surfaces, Backsplashes, and Side splashes.

    - Hardware for Casework including locks and magnetic catches.

    - Tracks for sliding drawer and cabinet door assemblies.

    - Plastic Laminate.

    - Glazing.

    - Sealant.

  - RELATED SECTIONS

      - Section 01572-Construction Waste Management

      - Section 06100-Carpentry.

      - Section 09650-Resilient Flooring.

  - REFERENCES

    - AWI-Architectural Woodwork quality Standards Illustrated; Architectural Woodwork Institute.

    - BHMA A156.9-American National Standard for Cabinet Hardware; builders Hardware Manufacturers Association (ANSI/BHMA).

    - GREENGUARD Product Emission Standard for Children & Schools.

    - GSA CID A-A1936-Adhesive, contact, Neoprene rubber; Federal Specifications and Standards.

    - NEMA LD 3-high-Pressure Decorative Laminates; National Electrical Manufacturers Association.

    - PS 1-construction and Industrial Plywood; National Institute of Standards and Technology (Department of Commerce).

    - PS 20-American softwood Lumber Standard; National Institute of Standards and Technology (Department of Commerce)

    - Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services

    - FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."

    - ASTM - American Society for Materials and Testing

      - D6007-Determination of Formaldehyde Concentration in Air from Wood Products.

      - D6330-Determination of Volatile Organic Compounds (Excluding Formaldehyde) Emissions from Wood-Based Panels.

  - SUBMITTALS

    - Shop Drawings:  Indicate AWI Quality Grades, plans, elevations, component profiles, details of construction and installation, dimensions, finishes, fittings and fastenings, hardware and accessories.

    - Product Data:  Submit data for items used by architectural woodworker.

      - Low Emitting Materials.

        - Submit manufacturer’s Material Safety Data Sheet Indicating VOC limits of all products.

        - Submit manufacturer’s certification that all products comply with Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services or GREENGUARD Product Emission Standard for Children & Schools.

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of assembly.

      - Regional Materials:

        - Sourcing location(s): Indicate location of extraction, harvesting, and recovery; indicate distance between extraction, harvesting, and recovery and the project site.

        - Manufacturing location(s): Indicate location of manufacturing facility; indicate distance between manufacturing facility and the project site.

        - Product Value:  Indicate dollar value of product containing local/regional materials; include materials cost only.

        - Product Component(s) Value:  Where product components are sourced or manufactured in separate locations, provide location information for each component. Indicate the percentage by weight of each component per unit of product.

      - Sustainable Forestry:

        - Forest Stewardship Council (FSC):  Provide of Chain-of-custody certificates signed by the lumber supplier validating compliance with FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."

        - Submit copies of invoices indicating cost data and the FSC certification numbers for each product.

        - Include evidence that mill is certified for chain of custody by an FSC-accredited certification body.

    - Samples:

      - Submit plastic laminate manufacturer’s sample chain of color chips and samples of hardware.

      - Submit samples of each wood species with finishes.

  - QUALITY ASSURANCE

    - Lumber Grading:  Comply with NIST Voluntary Product Standard PS20.

    - Standards:  Grades as defined by AWI “Quality Standards”.

    - Cabinets shall be designed to support 75 lbs per linear foot per row of shelving.  This includes design of attachment to wall or blocking.

  - MOCK-UP:

    - Product mock-up of full size base cabinet, which includes plumbing accessories and fittings.

    - Locate where directed.

    - Mock-up may not remain as part of the work.

  - DELIVERY, STORAGE, AND HANDLING

    - Protect units from damage and moisture.

- PRODUCTS

  - MATERIALS

    - All engineered wood, plastic laminate, and hardware materials to contain recycled content.

  - PLASTIC LAMINATED FINISHED CASEWORK

    - Quality Grade:  AWI Custom.

    - Construction:  Details conforming to AWI Flush overlay design (unless shown otherwise on Drawings).

  - WOOD:

    - All wood products shall comply with FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."

    - All engineered wood products such as plywood and fiberboard shall comply with Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services or GREENGUARD™ Product Emission Standard for Children & Schools.

    - Face Frame Rail:  3/4 inch thick lumber.

    - Plywood:  3/4 inch thick for counters, doors, drawer faces, shelves, cabinet bodies etc.

      - Countertops in which sinks occur must have a core of exterior grade plywood.

    - Fiberboard:  Cabinet backs and drawer bottoms:  1/4-inch thickness minimum, tempered with factory applied gloss surface.

    - Shelves:  3/4 or 1 inch thickness plywood per AWI and Article 1.3 C above.

    - Particleboard is not permitted.

  - LAMINATE MATERIALS:

    - Plastic Laminate:  NEMA LD 3, GP-50 General Purpose type; 0.050 inches thick; for horizontal and vertical applications; colors as selected by Project Consultant.

    - Cabinet Liner:  NEMA LD 3, BK20, Backing grade, undecorated plastic laminate; 0.020 inches thick plus or minus 0.004 inches; Color:  White.

    - Acceptable manufacturers

      - Formica.

      - Nevamar.

      - Pionite.

      - Wilsonart.

  - HARDWARE:

    - Types:

      - Drawer Slides:  Grant Model No. 329, self-closing clear chromate finish.

      - Surface Mounted Adjustable Shelf Standards:  Knape & Vogt #80; satin. All standards within 6 inches of the end of the shelving and space not more than 30 inches apart.

      - Surface Mounted Adjustable Shelf Brackets:  Knape & Vogt #180.

      - Recessed Mounted Adjustable Shelf Standards:  Knape & Vogt Mfg. Co. No. 255 aluminum shelf standards; length to suit application; unfinished.

      - Recessed Mounted Adjustable Multiple Holes Shelf Clips:  Knape & Vogt Mfg. Co. No. 239; zinc-plated steel.

      - Fixed Shelf Brackets:  Hafele America No. 287.31.035 and No. 287.31.044; finish: stainless steel.

      - Full Mortise Hinges: Stanley Model No. CB1960; Standard weight, 3 knuckle concealed bearing, steel hinge; stainless steel non-rising pin; US32D finish.

      - Cabinet Hinges:  RPC 376-26D-5 knuckle wrap around type allowing 270 degree turn at end of cabinet work unit.

        - Doors up to 48 inches high:  Provide 2 hinges.

        - Doors Greater than 48 inches:  Provide 3 hinges.

      - Cabinet Pulls:  Stanley 4484-US 26D, wire pull.

        - Substitutions:  Will be considered by the A/E and Owner when submitted per requirements of Division-0 and Division-1, and Section 01630-Substitution Procedures.

      - Continuous Hinges:  Stanley No. STS314-1/4; Type 302 stainless steel; plain finish.

      - Cabinet Locks and Cylinders:  Schlage No. CL1000-Series Door Lock or Olympus 700 SC; Solid Brass Cylinder; 626 finish.

      - Drawer Locks and Cylinder:  Schlage No. CL2000 Series Drawer Lock or Olympus 800 SC; Solid Brass Cylinder; 626 finish.

      - Cabinet Keys:  All casework in individual rooms shall be keyed alike.

      - Surface Bolts:  Stanley No. CD4060; solid brass; US3 finish.

      - Grommets:  Doug Mockett & Co., Inc. No. EDP-3 Set; Color:  Light Grey.

  - GLAZING:

    - 1/8 inch thick tempered clear and in compliance with Section 08800-Glazing.

  - ACCESSORIES:

      - Adhesive:  Type recommended by laminate manufacturer to suit application.

        - Toxicity/IEQ:  Comply with South Coast Air Quality Management District (SMAQMD) Rule #116 or GREENGUARD Product Emission Standard for Children & Schools.

      - Glass:  As specified in Section 08800-Glazing.

      - Fasteners:  Size and type to suit application.

      - Bolts, Nuts, Washers, Lags, Pins, and Screws:  Of size and type to suit application; corrosive-resistant finish in concealed locations and chrome-plated finish in exposed locations.

  - FABRICATION

    - Shop assemble casework for delivery to site in units easily handled and to permit passage through building openings.  Field fabricated cabinetwork is not acceptable.

    - Fit shelves, doors, and exposed edges with plastic laminate edging.  Use full-length pieces only.

    - Door and Drawer Fronts:  3/4 inch thick.

    - When necessary to cut and fit on site, provide materials with ample allowance for cutting.  Provide trim for scribing and site cutting.

    - Apply plastic laminate finish in full-uninterrupted sheets consistent with manufactured sizes.  Make corners and joints hairline.  Locate counter butt joints minimum 2 feet from edge of sink cutouts.

    - Cap exposed plastic laminate edges with material of same finish and pattern.

    - Mechanically fasten backsplash to countertops with steel brackets at 16 inches on center.

    - Apply cabinet liner to reverse side of plastic laminate finished surfaces.

    - Provide cutouts for plumbing fixtures, inserts, appliances, outlet boxes, and other fixtures and fittings.  Verify locations of cutouts from on-site dimensions.  Seal cut edges.

    - Accurately locate and securely install all hardware.  Job adjust to operate properly and re-align doors, drawers, etc. as required for proper operation.

    - Counter tops to have rounded corners in the horizontal plane

- EXECUTION

  - PREPARATION

    - Verify adequacy of backing and support framing.

    - Verify location and sizes of utility rough-in associated with work of this section.

    - Field measurements:  Verify all critical dimensions in field prior to fabrication.

  - INSTALLATION

    - Do not proceed with casework installation until resilient flooring has been installed under casework locations.

    - Set and secure casework in place rigid, plumb, and level.

    - Carefully scribe casework, which is against other building materials, leaving gaps of 1/16 inch maximum.  Do not use additional overlay trim for this purpose.

    - Use purpose designed fixture attachments at concealed locations for wall mounted components.

    - Use threaded steel concealed joints fasteners to align and secure adjoining cabinet units and counter tops.

    - Secure cabinet and counter bases to floor using appropriate angles and anchorages.

    - Counter-sink anchorage devices at exposed locations used to wall mount components, and conceal with solid plugs of material to match surrounding material.  Finish flush with surrounding surfaces.

    - After installation, fill gaps between casework and walls, soffits, etc. with sealant. Provide continuous bead of sealant at joint between cabinet base and flooring.  Color shall match cabinets.

    - Adjust doors, drawers, hardware, fixtures and other moving or operating parts to function smoothly and correctly.

    - Clean casework, counters, shelves, hardware, fittings and fixtures.

END OF SECTION