# SECTION 07145 (07 16 00)

## CEMENTITIOUS WATERPROOFING

- GENERAL

  - SECTION INCLUDES

    - Cement-base waterproof coating for concrete and masonry surfaces.

    - Work includes all applicable sealants, and repairs needed to ensure a complete waterproof system for concrete and masonry components at locations indicated.

  - RELATED SECTIONS

    - Section 03300-Cast In Place Concrete.

    - Section 04200-Unit Masonry.

  - SUBMITTALS

    - Materials list of items proposed to be provided under this Section.

    - Product Data

      - Low Emitting Materials.

        - Submit manufacturer’s Material Safety Data Sheet Indicating VOC limits of all products.

        - Submit manufacturer’s certification that all products comply with Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services or GREENGUARD Product Emission Standard for Children & Schools.

    - Manufacturer's specifications and other data needed to prove compliance with the specified requirements.

    - Manufacturer's current recommended installation procedures which, when reviewed by the Architect, will become the basis for accepting or rejecting actual installation procedures used on the Work.

    - Written documentation of applicator's qualifications, including reference projects of similar scope and complexity, with current phone contacts of architects and owners for verification.

    - Warranty.

  - QUALITY ASSURANCE

    - Applicator qualifications:

      - Engage an experienced applicator who is certified in writing by waterproofing manufacturer as qualified to install manufacturer’s waterproofing.  Applicator shall have at least three years experience in installing materials of types specified and shall have successfully completed at least three projects of similar scope and complexity.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver materials to Project site in original containers with seals unbroken, labeled with manufacturer’s name, product brand name and type, date of manufacture, shelf life, and directions for storing and mixing with other components.

    - Store materials in their original undamaged containers in a clean, dry, protected location and within the temperature range required by waterproofing manufacturer.  Protect stored materials from direct sunlight.

    - Remove and replace material that cannot be applied within its stated shelf life.

  - WARRANTY

    - Provide against defective materials and workmanship for a period of one-year following date of completion.

      - Manufacturer's standard warranty covering materials.

      - Applicator's standard warranty covering workmanship.

- PRODUCTS

  - MANUFACTURERS

    - Subject to compliance with the specified requirements, provide products by the following:

    - Cementitious waterproof coating systems:

      - “Eucosil” by The Euclid Chemical Company.

      - “MasterSeal 581” by BASF Construction chemicals LLC.

    - Quick setting, non-shrink, waterproof patching mortar:

      - “Speed Plug” by The Euclid Chemical Company.

      - “MasterSeal 590” by BASF Construction chemicals LLC.

    - Acrylic bonding agent:

      - “Flex-Con” by The Euclid Chemical Company.

      - “Masteremaco A 660” by BASF Construction chemicals LLC.

    - Substitutions:

      - Will be considered by the A/E and Owner when submitted per requirements of Division-0, Division-1, and Section 01630-Product Substitution Procedures.

    - Toxicity/IEQ:  Comply with Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services or GREENGUARD Product Emission Standard for Children & Schools

  - WATER

    - Water used shall be clean and potable.

  - ACCESSORIES

    - Provide other materials not specifically described but required for a complete and proper installation, as selected by the Contractor and approved by the manufacturer as compatible, subject to review of the Project Consultant.

- EXECUTION

  - SURFACE PREPARATION

    - Surfaces to be coated must be clean.  Chip, sandblast, or grind off all defective materials, glazed smooth concrete, previous coatings, paint and foreign matter such as grease, form oil block filler, curing compounds and efflorescence.

    - Repair all cracks, joints, holes or breaks with quick setting non-shrink waterproof mortar.

    - Cut back form ties 1 inch and patch flush and smooth.

    - Pre-Dampen the surface prior to application of waterproof coating.

  - MIXING

    - Per manufacturer’s written instructions.

  - INSTALLATION

    - Apply EUCOSEAL with a stiff fiber brush at an application rate of 2 lb. per 10 FT2. Scrub well into damp wall filling all pores and finish with final strokes in one direction.  Keep a wet edge. Allow 24 hours to dry.  Apply second coat at a rate of 1 lb per 10 FT2.

    - Level Wall, Trowel Application:  Mix 2 parts of Eucoseal with 1 part clean white silica sand and trowel apply material to wall at approximately 1/8 inch.

END OF SECTION