# SECTION 09215 (09 26 00)

## VENEER PLASTER

- GENERAL

  - SECTION INCLUDES

    - Interior veneer plaster applied to sheet gypsum base substrate surfaces.

    - Acoustic accessories are included.

  - RELATED SECTIONS

      - Section 09125-Plaster Ceiling suspension systems for metal framing and lath.

      - Section 09220-Portland Cement Stucco for finish.

      - Section 09250-Gypsum Board for backing materials.

  - REFERENCES

    - ASTM C442-Gypsum Backing Board, Gypsum Coreboard, and Gypsum Shaftliner Board.

    - ASTM C472-Physical Testing of Gypsum, Gypsum Plasters and Gypsum Concrete.

    - ASTM C588-Gypsum Base for Veneer Plasters.

  - SUBMITTALS

    - Product Data:  Submit manufacturer’s specifications and installation instructions.

  - QUALITY ASSURANCE

    - References:

      - Application of Gypsum Base to Receive Gypsum Veneer Plaster, ANSI/ASTM C844.

      - Application of Gypsum Veneer Plaster, ANSI/ASTM C843.

      - Fire Rated Partitions: Listed Assembly by U.L.

  - ENVIRONMENTAL CONDITIONS

    - Provide ventilation during plastering operation until plaster is dry.

- PRODUCTS

  - MANUFACTURER

    - Subject to compliance with requirements, provide products and systems from one of the following:

      - Mfr:  Georgia Pacific; Product:  Dens-Cote Veneer Plaster.

      - (Basis of Design) Mfr:  Gold Bond Building Products; Product:  See materials below..

      - Mfr:  U.S. Gypsum Company; Product:  Imperial Finish.

    - Basis of Design/Standard of Comparison:  Materials as furnished by Gold Bond are specified herein as a standard of quality and performance.

  - MATERIALS

    - Plaster: One component Uni-Kal, ASTM C587.  Compressive strength: 2500 PSI per ASTM C472.

    - Backing Board: ANSI/ASTM C588 and C442.

      - Regular Board:  Kal-Kore.

      - Fire-Resistant Board:  Fire-Shield Kal-Kore Type X.

    - Joint Reinforcing:  Kal-Mesh.

    - Corner Bead:  Kal-Korner Bead.

    - Casing Bead:  No. 233 Casing Bead.

    - Fasteners:  As recommended by the manufacturer’s instruction.

  - MIX DESIGN

    - Develop plaster mix in accordance with manufacturer’s instruction.

- EXECUTION

  - INSTALLATION

    - Install gypsum board in accordance with ANSI/ASTM C844 and requirements of Section 09250.

    - Cut and position accurately.  Bring panel edges together, but do not force into place.  Locate points over framing members and stagger them on opposite side of partition.  Avoid joining panels at corners of doors and window frames.  Fasten in accord with manufacturer’s instructions.

    - Apply joint reinforcing over all joints and interior angles.  Install proper corners and casing beads.

  - APPLICATION OF PLASTER

    - Apply gypsum veneer plaster in one or two coats in accordance with ANSI/ASTM C843.

    - Apply plaster with sufficient material and pressure to provide a good bond on the gypsum base.  Double back over the area just troweled with the same mixer batch to total thickness of 3/32 inch minimum.

    - Extend plaster behind all tack boards, marker boards and casework including upper and lower cabinets, and 6 inches above acoustic ceilings.

    - Begin finish troweling at time of initial set, using water sparingly, to achieve a smooth finish.

    - Finish surface to flat, smooth hard finish.  In corridors provide “knockdown” finish.

END OF SECTION