# SECTION 09310 (09 30 13)

## CERAMIC TILE

- GENERAL

  - SECTION INCLUDES

    - Types of work in this section include, but are not limited to the following:

      - Floor and wall tile.

      - Trim and special shapes.

      - Setting and Grouting materials

      - Marble thresholds and window sills.

      - Sealing of expansion and other joints in tile work with elastomeric joint sealants.

  - RELATED SECTIONS

  - Section 01572-Construction Waste Management.

    - Section 09250-Gypsum Board.

    - Section 09330-Quarry Tile.

    - Division 15 floor drains.

  - REFERENCES

    - American National Standard Specifications for Ceramic Tile (ANSI).

      - A108.4-Installation of Ceramic Tile with Organic Adhesives or Water Cleanable Tile-Setting Epoxy.

      - A108.5-Installation of Ceramic Tile with Dry-Set Portland Cement Mortar.

      - A108.10-Installation of Grout in Tilework.

      - A118.1-Standard Specification for Dry-Set Portland Cement Mortar.

      - A118.4-Standard Specification for Latex-Portland Cement Mortar.

      - A137.1-Standard Specification for Ceramic Tile.

      - C1028-Determining the Static Coefficient of Friction of Ceramic Tile and Other Like Surfaces by the Horizontal Dynamometer Pull-Meter Method.

    - American Society for Materials and Testing (ASTM).

      - C503-Marble Dimension Stone (Exterior).

      - C920-Elastomeric Joint Sealants.

    - GREENGUARD Product Emission Standard for Children & Schools.

    - ISO 14021–1999; Environmental Labels and Declarations.

    - RFCI FloorScore Program.

    - Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services.

    - TCNA Handbook for Ceramic Tile Installation.

  - DEFINITIONS

    - Definition:  Ceramic tile includes ceramic surfacing units made from clay or other ceramic materials.

  - PERFORMANCE REQUIREMENTS

    - Static Coefficient of Friction for Floor Tile:  Provide products with the following values as determined by testing per ASTM C1028.

      - Level surface:  Minimum 0.6 wet result.

      - Stair Treads:  Minimum 0.6 wet result.

      - Ramp surfaces:  Minimum 0.8 wet result.

  - SUBMITTALS

    - Product Data:  Submit manufacturer’s technical information and installation instructions for materials required, except bulk materials.  Include complete maintenance recommendations.

      - Low Emitting Materials.

        - Submit manufacturer’s Material Safety Data Sheet Indicating VOC limits of all products.

        - Submit manufacturer’s certification that all products comply with GREENGUARD Product Emission Standard for Children & Schools, RFCI FloorScore Program, or the Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services.

      - Recycled Content:

        - Indicate recycled content; indicate percentage of pre-consumer and post-consumer recycled content per unit of product.

        - Indicate relative dollar value of recycled content product to total dollar value of product included in project.

        - If recycled content product is part of an assembly, indicate the percentage of recycled content product in the assembly by weight.

        - If recycled content product is part of an assembly, indicate relative dollar value of recycled content product to total dollar value of the assembly.

    - Samples for Initial Selection Purposes:  Submit manufacturer’s sample boards consisting of actual tiles or sections of tile showing full range of colors, textures and patterns available for each type of tile indicated.  Include samples of grout and accessories involving color selection.

    - Samples for verification purposes:  Submit the following:

      - Samples for each type of tile and for each color and texture required, not less than 12 inch square, on plywood or hardboard backing and grouted.  Provide larger sample board when necessary to show field and accent tiles.

      - Full size samples for each type of trim, accessory and for each color.

      - Six inch long samples of marble stone thresholds and window sills.

    - Master Grade Certificates:  Submit for each shipment and tile type, signed by tile manufacturer and installer.

    - Qualification Data:  Submit certification, as specified under Quality Assurance article, substantiating capabilities and experience of installing firms and installing workers doing similar work.

  - QUALITY ASSURANCE

    - Maintain one copy of TCA Handbook and ANSI A108 Series/A118 Series on site.

    - Source of Materials:  Provide materials obtained from one source for all tile of same type.

      - Provide grout and setting materials from one source.

    - Tile shall conform to requirements of ANSI 137.1, standard grade.

    - Tile installation shall conform to Tile Council of North America (TCA)-Handbook for Ceramic Tile Installation.

    - Manufacturers:  Company specializing in the manufacturing of tile systems with a minimum of 10 years documented experience doing similar work.

    - Installer:  Company specializing in the installation of tile systems with a minimum of 5 years documented experience doing similar work.

    - PRE-INSTALLATI0N MEETING

    - Shall not occur without all submittals approved by the Contractor and accepted by the A/E.  Shall convene a minimum of two weeks before starting work of this section.

    - Required Attendees:

      - Contractor.

      - Tile Installer.

      - Tile System Manufacturer.

      - Installers of floor slab construction to receive tile work.

      - Any other subcontractors associated with tile work.

      - Architect.

      - Owner’s Project Manager.

      - Owner’s Maintenance Foreman.

      - Building Department Representative.

    - The Contractor shall make arrangements for the meeting and notify the parties required to attend.

    - Agenda shall include:

      - Review preparation and installation procedures and coordinating and scheduling required with related work.

      - Review tile system requirements (drawings, specifications, and other contract documents).

      - Review Shop Drawings and associated submittals.

      - Review manufacturer's technical materials.

      - Review and finalize construction schedule related to tile work and verify availability of materials, personnel, equipment and facilities needed to make progress and avoid delays.

      - Review required inspection, testing, certifying and material usage accounting procedures.

      - Tour representative areas of tile floor slabs, inspect and discuss condition of the substrate, floor slopes to drains, outlets, penetrations and other preparatory work performed by other trades.

  - DELIVERY, STORAGE, and HANDLING

    - Deliver and store packaged tile, setting, and grouting materials in original containers with seals unbroken and labels intact until time of use.  Comply with requirements of ANSI A137.1 for labeling sealed tile packages.  Prevent damage or contamination to materials by water, freezing, foreign matter or other causes.

    - Store tile and cementitious materials on elevated platforms, under cover, and in a dry location.

    - Store liquid latexes and emulsive adhesives in unopened containers and protect from freezing and extreme heat.

  - PROJECT CONDITIONS

    - Maintain environmental conditions and protect work during and after installation to comply with referenced standards and manufacturer’s printed recommendations

    - Maintain temperatures in tiled areas during installation and for 7 days after completion comply with ANSI and published manufacturer’s instructions.

  - EXTRA STOCK

    - Deliver minimum of 3 percent of total square feet of each color and pattern of field tile material required for project for maintenance use.

    - Deliver minimum of 3 percent of total linear feet of each color and pattern of accent or edge strip material required for project for maintenance use.

    - Clearly identify each carton.

  - WARRANTY

    - Provide a two-year written labor and material warranty.

    - Should defects develop, including any loss of adhesion to the sub floor or wall surfaces, completely replace tile to the satisfaction of he Owner, at no additional cost to the Owner.

- PRODUCTS

  - MANUFACTURERS

    - Subject to compliance with specified requirements, provide products by one of the following manufacturers (refer to Section 01620-Product Options for Buy American First-Foreign Product Limitations):

      - Floor Tile:

        - Unglazed Porcelain Mosaics:

          - American Olean Division of Dal-Tile International Corp.

          - Crossville, Inc. Dal-Tile Corp.

          - Dal-Tile Corp.

          - Interceramic.

      - Glazed Ceramic Wall Tile (semi-gloss or matte finish, with cushion edges):

        - American Olean Division of Dal-Tile International Corp.

        - Dal-Tile Corp.

        - United States Ceramic Tile Company.

      - Setting Materials:

        - Custom Building Products.

        - Laticrete International, Inc.

        - Mapei Corporation.

      - Latex Portland Cement Grout:

        - Custom Building Products.

        - Laticrete International, Inc.

        - Mapei Corporation.

      - Epoxy Grout:

        - Custom Building Products.

        - Laticrete International, Inc.

        - Mapei Corporation.

  - MATERIALS

    - Toxicity/IEQ:  All tile products, threshold and window sill, setting, grouting, and waterproofing/crack-suppression membrane materials are to comply with GREENGUARD Product Emission Standard for Children & Schools, RFCI FloorScore Program or the Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services.

    - All tile products materials are to contain recycled content.

  - GENERAL

    - ANSI Ceramic Tile Standard:  Comply with ANSI A137.1-American National Standard Specifications for Ceramic Tile for types and grades of tile indicated.

      - Furnish tile complying with “Master Grade” requirements for standard grade materials unless otherwise indicated.

    - ANSI Standard for Tile Installation Materials:  Comply with ANSI A108, A118, and A136.1 for installation products and materials indicated.

    - Colors, Textures and Patterns:  For tile and other products requiring selection of colors, surfaces textures or other appearance characteristics, provide products to match characteristics indicated or, if not otherwise indicated, as selected by Project Consultant from manufacturer’s standard chart.

      - Provide tile trim and accessories that match color and finish of adjoining flat tile.

    - Mounting:  Where factory-mounted tile is required, provide back or edge-mounted tile assemblies as standard with manufacturer unless another mounting method is indicated.

  - TILE PRODUCTS

    - Unglazed Porcelain Mosaics:  Through color body, nominal size not larger than 2 inch x 2 inch, thickness:  1/4 inch, with cushion edges.

    - Glazed Ceramic Wall Tile:  Nominal size 4-1/4 inch x 4-1/4 inch or 6 inch x 6 inch, thickness:  5/16 inch, with cushion edges.

    - Colors as selected by the Project Consultant from full range of manufacturer’s standard color chart.

    - Layout and pattern to be determined using maximum of 3 colors.  Tiles may be selected from Price Groups 1-5.

      - Field Tiles:  Group 1.

      - Accent Tiles:  Groups 1-5.

    - Trim Units and Special Shapes:  Provide tile trim units and special shapes to match characteristics of adjoining flat tile and to comply with the following requirements:

      - Size:  As indicated, coordinated with sizes and coursing of adjoining flat tile, where applicable.

      - Base:  Cove base units, width and height to match wall tile.

      - External Corners:  Bullnose shapes with round out base and top trim special shapes.

      - Internal Corners:  Field-butted square with square in-corner base and top trim special shapes.

  - THRESHOLDS and WINDOW SILLS

    - General:  Provide marble, that is uniform in color and finish, fabricated to sizes, and profiles indicated or required to provide transition between tile surfaces and adjoining finished floor surfaces.

    - Thresholds (natural stone marble):  Provide marble thresholds complying with ASTM C 503 requirements for exterior use and abrasion resistance for uses subject to heavy foot traffic.

      - Provide White Georgia or Madre Cream Alabama marble complying with MIA Group “A” requirements for soundness.

      - Size 2-1/4 inches wide, 1/2 inch thick maximum with beveled edges for handicap accessibility.

    - Window Sills (natural stone marble):  White Carrera marble with gray veins, 5/8 inch thick with beveled top edge.

  - SETTING MATERIALS

    - Organic Adhesive (thin-set):  ANSI A136.1; Type II.  Provide primer-sealer where recommended by manufacturer.

    - Latex/Portland Cement Mortar (thick-set):  Provide materials to comply with ANSI A118.4 as required for installation method designated, unless otherwise indicated.

    - Dry-Set Portland Cement Mortar (thin-set):  Pre-sanded product in accordance with ANSI A118.1 and by manufacturer.

    - Latex/Polymer Modified Portland Cement Mortar (thin-set):  Use one of the following complying with ANSI A118.4 unless otherwise required.

      - Dry-set mortar made of factory-blended cement and aggregates requiring only the addition of water at job site.

- or

      - Dry-set mortar made of factory-blended cement and aggregates requiring only the addition of latex mortar additive such as Laticrete 4237 or Project Consultant and Owner approved equal.

  - GROUTING MATERIALS

    - Polymer-Modified Cement Grout (formerly called latex-Portland cement grout):  Proprietary pre-blended compound of Portland cement selected and graded aggregates, color pigments and chemical additives gaged with latex additives to comply with manufacturer’s directions and ANSI A118.7.

      - Sanded/Unsanded Grout:

        - 1/8 inch wide or less:  Unsanded.

        - 1/8 inch wide or greater:  Sanded.

      - Color:  As selected by Project Consultant.  Provide selection of manufacturer’s full range of colors.

    - Epoxy Grout (use at following spaces:  Kitchen, Locker Room, Shower Room, Janitor Closet, Extractor and Grinder Room, and Group Toilet Rooms):

      - Comply with ANSI A118.3:  100 percent solid, stain resistant, chemical resistant.  Complying with ANSI A118.8.  Certified by manufacturer for intended use.

      - Color:  As selected by Project Consultant.  Provide selection of manufacturer’s full range of colors.

  - WATERPROOFING and CRACK-SUPPRESSION MEMBRANES for THIN-SET SHOWER TILE INSTALLATIONS

    - General:  Manufacturer's standard product that complies with ANSI A118.10.

    - Warerproofing (Shower Liner):

      - Chlorinated-Polyethylene-Sheet Product:  Nonplasticized, chlorinated polyethylene faced on both sides with high-strength, nonwoven polyester fabric, for adhering to latex-Portland cement mortar.

        - Mfr:  Nobleseal Company; Product:  Chloraloy.

- or

      - Polyethylene-Sheet Product:  Polyethylene faced on both sides with fleece webbing for adhering to latex-Portland cement mortar.

        - Mfr:  Schluter Systems LP; Product:  KERDI.

    - Crack-Supression:

      - Chlorinated-Polyethylene-Sheet Product:  Nonplasticized, chlorinated polyethylene faced on both sides with high-strength, nonwoven polyester fabric, for adhering to latex-Portland cement mortar.

        - Mfr:  Nobleseal Company; Product:  CIS.

  - ACCESSORIES and MISCELLANEOUS MATERIALS

    - Thresholds and Metal Edge Strips:  As selected by Project Consultant.

    - Movement and Control Joint Profiles:

      - Schluter Dilex-EDP Stainless Steel Movement Joint.

    - Sealant:  Provide sealant for joints as specified in Section 07920-Joint Sealants for required use, Type, Grade and Class.

- Tile Cleaner:  Neutral cleaner specifically acceptable to tile manufacturer and grout manufacturer.

- EXECUTION

  - EXAMINATION

    - Examine areas and substrate that will receive setting materials and quarry tile, before installation begins.  With the installers present, review for compliance with requirements for installation tolerances and other conditions that will affect performance of tile installation.

    - Acceptable Tolerance:  Comply with tolerances as listed in ANSI A108 Series “Specifications for Installation of Ceramic Tile” for installation condition and method.

      - Slope finished tile to drains 1/4 inch/foot.

    - Verify conditions and surfaces to receive tile to ensure:

      - Surfaces are firm, dry, clean and free of dust and oily or waxy films.

      - Grounds, anchors, plugs, hangers, recess frames, waterproofing, membrane, electrical and mechanical work in or behind tile have been installed prior to start of tile work.

    - Do not proceed with work until defects or conditions adversely affecting quality and execution of quarry tile work are corrected.

    - Verify that concrete slab surfaces are ready for tile installation by testing for moisture emission rate and alkalinity; obtain instructions if test results are not within the following limits:

      - Moisture Emission Rate:  Not greater than 3 lb per 1,000 sq ft per 24 hours when tested using calcium chloride moisture kit for 72 hours.

      - Alkalinity:  pH range of 5-9.

  - PREPARATION

    - Do not use liquid curing compounds or other coatings that may prevent bonding of tile materials to slabs.  Slab shall be dry at times of tile installation.

    - Layout:  Prepare layout of tile work so as to align joints of floor, base, wall tile, and trim when pieces are of the same size.  Lay out so as to center tile fields in both directions in each space or area, to minimize tile cutting, and provide lay out so as to minimize use of tile that are less than one-half tile in size.

      - Locate expansion joints and other joint types to be sealant filled during installation of setting material.  Do not saw cut joints after tile has been installed.

      - Locate joints in tile surfaces directly over joints that occur in concrete or other substrate material.

    - Preparation of Concrete Floors for Setting Beds:

      - Bondability:

        - Where tile is to be installed, concrete slabs should not have air entrainment or other additives in the mix, nor sealers or curing compounds applied without specific approval of mortar and grout manufacturer.

        - Slabs should have steel trowel and fine broom finish and be free of laitance.

        - In case of any question on condition of slab, it should be tested for bondability with a Dillon Dynamometer, and show a tensile bond of not less than 300 psi.

      - Levelness:

        - Before tile is applied, test structural floor for levelness or uniformity of slope by water.  Fill, level, and retest areas as required to meet tolerances specified in Section A-3 of ANSI A108.1 and retest.

        - When specified levelness or uniformity of slope is obtained, prepare floors for setting bed in accordance with ANSI A108.1.

  - INSTALLATION

    - Install tile, setting materials, and grout in accordance with applicable requirements of ANSI A108.1 through A108.13, manufacturer’s instructions, and TCA Handbook recommendations.

    - Refer to Section 09250-Gypsum Board for “Tile Backer Boards”.

    - Setting Methods:

      - Wall Tile:

        - On Gypsum Board in Dry Areas:  ANSI A108.4 (Referenced TCA Method Number W242 Organic Adhesive) with Latex-Portland Cement Grout.

        - On Tile Backer Board in High Moisture or Wet Areas:  ANSI A108.5 (Referenced TCA Method Number W244, W245, B415, or B420 Dry-Set, Latex-Portland Cement Mortar, or Latex/Polymer Modified Portland Cement Mortar) with Latex-Portland Cement Grout; provide expansion joints under provisions of TCA Method EJ171.

      - Floor Tile:

        - Concrete Slab:  ANSI A108.5 (referenced TCNA Method Number F113) with latex Portland cement grout or epoxy grout ANSI 108.6 Epoxy Mortar and Grout and (referenced TCNA Method Number F114).

    - Extend tile work into recesses and under or behind equipment and fixtures, to form a complete covering without interruptions, except as otherwise shown.  Terminate work neatly at obstructions, edges and corners without disrupting pattern or joint alignment.

    - Accurately form intersections and returns.  Perform cutting and drilling of tile without marring visible surfaces.  Carefully grind cut edges of tile abutting trim, finish or built-in items for straight aligned joints.  Fit tile closely to electrical outlets, piping, fixtures and other penetrations so that plates, collars, or covers overlap tile.

    - Lay out tile wainscots to next full tile beyond dimensions indicated.

    - Where acoustic tile ceilings occur, install ceramic wall tile to a line 2 to 4 inches above plane of exposed surface of ceiling.

    - Expansion Joints:  Locate expansion joints and other sealant filled joints, including control, contraction and isolation joints, where indicated, or if not indicated, at spacing and locations recommended in TCA “Handbook for Ceramic Tile Installation”, and approved by Project Consultant.

      - Prepare joints and apply sealant to comply with requirements of referenced standards and sealant manufacturer.

    - Grout tile to comply with referenced installation standards, using grout materials indicated.

    - Mix and install proprietary components to comply with grout manufacturer’s directions.

    - Joints:

      - General:  Place tile joints in uniform width, subject to variance in tolerance allowed by tile size.  Make points watertight, without voids, cracks, excess setting material, or excess grout.

      - Joint widths:  Shall be per manufacturer’s published recommendations.

      - Jointing Pattern: Unless otherwise shown, lay tile in grid pattern.  Align joints when adjoining tiles on floor; base, walls and trim are same size.  Lay out tile work and center tile fields in both directions in each space or on each wall area.  Adjust to minimize tile cutting and to avoid cut tiles less than one-half the tile size.  Provide uniform joint widths, unless otherwise shown.

        - Lay out tile work on floors so that wherever possible no tiles less than half full size occur.

        - Tile mounted in sheets:  Make joints between tile sheets same width as joints within tile sheets so that extent of each sheet is not apparent in finished work.

    - Sound tile after setting.  Replace hollow sounding units.

    - Keep expansion and control joints free of setting material or grout.  Apply sealant to control joints.

    - Allow tile to set for a minimum of 48 hours prior to grouting.

    - Apply sealant to junction of tile and dissimilar materials and junction of dissimilar planes, including perimeter of room.

  - WATERPROOFING and CRACK-SUPPRESSION MEMBRANE INSTALLATION

    - Install waterproofing to comply with ANSI A108.13 and waterproofing manufacturer's written instructions to produce waterproof membrane of uniform thickness bonded securely to substrate.

    - Install crack-suppression membrane to comply with manufacturer's written instructions to produce membrane of uniform thickness bonded securely to substrate.

    - Do not install tile over waterproofing until waterproofing has cured and been tested to determine that it is watertight.

  - GROUTING

    - Installation to comply with ANSI A108.10.

    - Force grout into joints to fill solid.  Remove and re-grout discolored joints.  Fill voids injoint grout.

  - TOLERANCES

    - Finished installation shall be true to a tolerance of + 1/8 inch in 10 feet, and + 1/16 inch within any given running foot.

  - CLEANING and PROTECTION

    - Upon completion of grouting, and before grout is completely dry, clean all tile surfaces to ensure they are free of all foreign matter.  Remove haze (residue) on tile resulting from grouting as soon as possible.

    - Do not clean tile with acid solutions or chemical cleaners unless in accordance with both tile and grout manufacturer’s' written recommendations.

    - Leave installations finished, clean, and free of chipped, broken, cracked, loose or other defective tile work.

    - Protect final tile installation in a manner, and so as to maintain conditions, to ensure tile work will not deteriorate or be damaged at time of Substantial Completion.

    - Protect the entire tile surface area with a heavy-duty non-staining Kraft paper and tape in place.

    - Prohibit foot and wheeled traffic from newly tiled floors for a minimum of 7 days after grouting is completed.

    - Just prior to acceptance remove protective paper and rinse neutral cleaner from surfaces of tile.  Dry and lightly buff.

    - Refer to manufacturer’s literature for additional details.

END OF SECTION