# SECTION 09900 (09 91 00)

## PAINTING

- GENERAL

  - SECTION INCLUDES

    - Surface preparation.

    - Field application of primers, paints, stains, and other coatings for exterior and interior items and surfaces.

  - SYSTEM DESCRIPTION

    - Paint exposed surfaces, except where these Specifications indicate that the surface or material is not to be painted or is to remain natural.  If an item or a surface is not specifically mentioned, paint the item or surface the same as similar adjacent materials or surfaces.  If a color of finish is not indicated, Project Consultant will select from standard colors and finishes available.

      - Painting includes field painting of exposed bare and covered pipes and ducts (including color coding), hangers, exposed steel and iron supports, and surfaces of mechanical and electrical equipment that do not have a factory-applied final finish.

      - Surface preparation, priming, and finish coats specified in this Section are in addition to shop priming and surface treatment specified in other Sections.

    - Do not paint prefinished items, concealed surfaces, finished metal surfaces, operating parts, and labels.

      - Prefinished items include the following factory-finished components:

        - Architectural woodwork.

        - Acoustical wall panels.

        - Toilet enclosures.

        - Metal lockers.

        - Prefinished folding or accordion walls.

        - Elevator entrance doors and frames.

        - Elevator equipment.

        - Finished mechanical and electrical equipment.

        - Light fixtures.

      - Concealed surfaces include walls or ceilings in the following generally inaccessible spaces:

        - Foundation spaces.

        - Furred areas.

        - Ceiling plenums.

        - Utility tunnels.

        - Pipe spaces.

        - Duct shafts.

        - Elevator shafts.

      - Finished metal surfaces include the following:

        - Anodized aluminum.

        - Stainless steel.

        - Chromium plate.

        - Copper and copper alloys.

        - Bronze and brass.

      - Operating parts include moving parts of operating equipment and the following:

        - Valve and damper operators.

        - Linkages.

        - Sensing devices.

        - Motor and fan shafts.

      - Labels:  Do not paint over UL, FMG, or other code-required labels or equipment name, identification, performance rating, or nomenclature plates.

    - Alternates:  Refer to Division 1 Section "Alternates" for description of Work in this Section affected by alternates.

  - REFERENCES

    - American Society for Materials and Testing (ASTM).

      - D16-Terminology for Paint, Related Coatings, Materials, and Applications.

      - D3960-Determining Volatile Organic Compound (VOC) Content of Paints and Related Coatings.

      - D4262-pH of Chemically Cleaned or Etched Concrete Surfaces.

      - D4263-Indicating Moisture in Concrete by the Plastic Sheet Method.

    - Environmental Protection Agency (EPA).

    - Factory Mutual Global (FMG).

    - NACE International the Corrosion Society.

    - GREENGUARD Product Emission Standard for Children & Schools.

    - Green Seal Standard GS-03, Anti-corrosive paints, Second Edition, January 7, 1997.

    - Green Seal Standard GS-11, Paints First Edition, May 20, 1993.

    - National Fire Protection Association (NFPA):  NFPA 30-Flammable and Combustible Liquids Code.

    - Painting and Decorating Contractors Association (PDCA) P4-Responsibility for Inspection and Acceptance of Surfaces Prior to Painting and Decorating.

    - Society of Protective Coatings (SSPC) SP6-Commercial Abrasion Blast.

    - Standard Practice for The Testing Of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers, including 2004 Addenda.  California Department of Health Services. Steel Structures Painting Council.

    - Underwriter’s Laboratory (UL).

  - DEFINITIONS

    - General:  Standard coating terms defined in ASTM D16 apply to this Section.

      - Flat:  Lusterless or matte finish with a gloss range below 15 when measured at an 85 degree meter.

      - Eggshell:  Low-sheen finish with a gloss range between 20 and 35 when measured at a 60 degree meter.

      - Semi-gloss:  Medium-sheen finish with a gloss range between 35 and 70 when measured at a 60 degree meter.

      - Full gloss:  High-sheen finish with a gloss range more than 70 when measured at a 60 degree meter.

  - SUBMITTALS

    - Product Data:  For each paint system indicated.  Include block fillers and primers.

      - Material List:  An inclusive list of required coating materials.  Indicate each material and cross-reference specific coating, finish system, and application.  Identify each material by manufacturer's catalog number and general classification.

      - Manufacturer's Information:  Manufacturer's technical information, including label analysis and instructions for handling, storing, and applying each coating material.

      - Low Emitting Materials:

        - Submit manufacturer’s Material Safety Data Sheet Indicating VOC limits of all products.

        - Submit manufacturer’s for all products that comply with GREENGUARD Product Emission Standard for Children & Schools.

        - Submit manufacturer’s certification for all architectural paint, coating, and primer products applied to interior walls and ceilings that comply with the Green Seal Certification standard GS-11, Paints First Edition, May 20, 1993.

        - Submit manufacturer’s certification for all anti-corrosive and anti-rust paint products applied to interior metal ferrous surfaces that comply with the Green Seal Standard GS-03, Second Edition, January 7, 1997.

    - Samples for Verification:  For each color and material to be applied, with texture to simulate actual conditions, on representative Samples of the actual substrate.

      - Provide stepped Samples, defining each separate coat, including block fillers and primers.  Use representative colors when preparing Samples for review.  Resubmit until required sheen, color, and texture are achieved.

      - Provide a list of materials and applications for each coat of each Sample.  Label each Sample for location and application.

      - Submit Samples on the following substrates for Architect's review of color and texture only:

        - Concrete:  4 inch square samples for each color and finish.

        - Concrete Unit Masonry:  4 by 8-inch samples of masonry, with mortar joint in the center, for each finish and color.

        - Painted Wood:  8 inch square samples for each color and material on hardboard.

        - Stained or Natural Wood:  4 by 8 inch samples of natural or stained-wood finish on representative wood surfaces.

        - Ferrous Metal:  3 inch square samples of flat metal and 6-inch long samples of solid metal for each color and finish.

    - Warranty.

  - QUALITY ASSURANCE

    - Applicator Qualifications:  Paint applicator shall be licensed in the State of Florida or in Broward County and use state or county journeymen.  Provide a legible copy of license and, when applicable a journeyman’s certification attesting to qualification requirements.

      - Certifications:  Paint applicator shall provide a certification attesting to having worked on projects similar in scope to this project for a minimum of 5 years.  Paint applicator not providing such documentation or not having the required experience will be removed from the project and replaced by the Contractor.

    - Source Limitations:  Obtain block fillers and primers for each coating system from the same manufacturer as the finish coats.

    - Benchmark Samples (Mockups):  Provide a full-coat benchmark finish sample for each type of coating and substrate required.  Comply with procedures specified in PDCA P5.  Duplicate finish of approved sample Submittals.

      - Architect will select one room or surface to represent surfaces and conditions for application of each type of coating and substrate.

        - Wall Surfaces:  Provide samples on at least 100 sq. ft.

        - Small Areas and Items:  Project Consultant will designate items or areas required.

      - Apply benchmark samples, according to requirements for the completed Work, after permanent lighting and other environmental services have been activated.  Provide required sheen, color, and texture on each surface.

        - After finishes are accepted, Project Consultant will use the room or surface to evaluate coating systems of a similar nature.

    - Final approval of colors will be from benchmark samples.

    - Conform to ASTM for interpretation of terms used in this Section.

    - Labels:  Do not paint over Underwriter’s Laboratories (UL), Factory Mutual (FM) or other code required labels or equipment name, identification, performance rating, or nomenclature plates.

    - Regulatory Requirements:  Conform to applicable code for flame/fuel/smoke rating requirements for finishes.

    - Single Source Responsibility:  Provide primers, paints, stains and other coatings for exterior and interior items and surfaces by the same manufacturer.

    - Lead Safety:  Beginning April 22, 2010, federal law (EPA’s Final Rule 40 CFR Part 745) shall require that contractors and renovators performing renovation, repair, and painting projects that disturb lead based paint in homes, child care facilities, and schools built before 1978 that a child under age 6 visits regularly, to be certified and follow lead-safe work practices to prevent lead contamination.  Contractors and renovators must be EPA Certified, and projects must comply with the EPA manual “Renovate Right, Important Lead Hazard Information for Families, Child Care Providers and Schools”.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver materials to Project site in manufacturer's original, unopened packages and containers bearing manufacturer's name and label and the following information:

      - Product name or title of material.

      - Product description (generic classification or binder type).

      - Manufacturer's stock number and date of manufacture.

      - Contents by volume, for pigment and vehicle constituents.

      - Thinning instructions.

      - Application instructions.

      - Color name and number.

      - VOC content.

    - Store materials not in use in tightly covered containers in a well-ventilated area at a minimum ambient temperature of 45 degrees F and a maximum temperature of 90 degrees F.  Maintain storage containers in a clean condition, free of foreign materials and residue.

      - Keep storage area neat and orderly.  Remove oily rags and waste daily.

    - Container Labeling: Include manufacturer’s name, type of paint, brand name, brand code, coverage, surface preparation, drying time, cleanup, color designation, and instructions for mixing.

    - Store and mix materials in 1 area only.

    - Comply with health and fire regulations. Take precautionary measures to prevent fire hazards and spontaneous combustion.

  - PROJECT CONDITIONS

    - Allow sufficient time for stucco and plaster to moist cure in accordance with Section 09220-Portland Cement Plaster (Stucco).  The pH factor and moisture level of all areas to receive primer or paint shall be tested and confirmed.

      - pH factor shall be below 10.

      - Moisture content shall be below 80.

      - Provide a written record of each such test.

      - “Hot” primer shall not be permitted until the pH is below 10.

    - Comply with manufacturer's recommendations as to environmental conditions under which coatings and coating systems can be applied.

    - Do not do any painting if the relative humidity exceeds 90 percent.

    - Do not apply finish in areas where dust is being generated.

    - Cover or otherwise protect finished work of other trades and surfaces not being painted concurrently or not to be painted.

    - Do not apply paint in snow, rain, fog, or mist; or when relative humidity exceeds 85 percent; or at temperatures less than 5 degrees F above the dew point; or to damp or wet surfaces.

      - Painting may continue during inclement weather if surfaces and areas to be painted are enclosed and heated within temperature limits specified by manufacturer during application and drying periods.

    - Provide continuous ventilation to maintain surface and ambient temperatures 24 hours before, during and 48 hours after painting.

    - Provide lighting levels of 50 foot-candles at mid-height at substrate surface.

  - WARRANTY

    - Provide a written guarantee, co-signed jointly and severalty by the Contractor, painting subcontractor, and materials manufacturers, against cracking, peeling flaking, chalking and mildew on interior surfaces, and additionally against erosion and unreasonable fading or exterior surfaces, for 8 years; agreeing to repair and repaint surfaces affected by such defects, at no cost to the Owner, including necessary removal or protection of other work, without limit, within 30 days after notification by the Owner, and to perform such work based on the provisions of this section, including extension of the guarantee to cover new work.

  - EXTRA STOCK

    - Provide “Home Store” data (where painter purchased the paints used on the project) include all paint records and the following Home Store information:

      - Name.

      - Address.

      - Telephone number.

      - Store manager name.

      - List of paints purchased by name and type.

    - Provide a 1-gallon container of each color and surface texture of respective finish paints used on the project.

    - Label each container with color, texture and room locations, in addition to the manufacturer’s label.

    - Deliver and store extra stock at the time of substantial completion.

- PRODUCTS

  - MANUFACTURERS

    - The Florida School Plant Management Association’s Certified Products List shall serve as a minimum guideline.

    - Subject to compliance with the specified requirements, provide products by one of the following manufacturers:

      - Benjamin Moore & Co.

      - Coronado Paint Co.

      - PPG Architectural Finishes, Inc.

      - (Basis of Design) Sherwin-Williams Company.

  - PAINT MATERIALS, GENERAL

    - All scope of work shall use selected manufacturer’s complete paint system.

    - Material Compatibility:  Provide block fillers, primers, and finish-coat materials that are compatible with one another and with the substrates indicated under conditions of service and application, as demonstrated by manufacturer based on testing and field experience.

    - Material Quality:  Provide manufacturer's best-quality paint material of the various coating types specified that are factory formulated and recommended by manufacturer for application indicated.  Paint-material containers not displaying manufacturer's product identification will not be acceptable.

      - Proprietary Names:  Use of manufacturer's proprietary product names to designate colors or materials is not intended to imply that products named are required to be used to the exclusion of equivalent products of other manufacturers.

    - Colors:  As selected by Project Consultant in consultation with Owner.

    - Millage noted below is minimum acceptable dry film thickness per coat application.

    - Toxicity/IEQ:  All interior paint and coating products are to comply with the GREENGUARD Product Emission Standard for Children & Schools or any of the following which apply:

      - All architectural paints, coatings, and primers applied to interior walls and ceilings are to comply with the Green Seal Standard GS-11, First Edition, May 20, 1993.

      - All anti-corrosive and anti-rust paints applied to interior metal ferrous surfaces are to comply with Green Seal Standard GS-03, Second Edition, January 7, 1997.

  - VOC CONTENT

    - Products are specified must not exceed the following:

      - Interior Coatings (weight in grams/liter of product):

        - Non-Flat:  150.

        - Flat:  50.

      - Exterior Coatings weight in grams/liter of product minus water):

        - Non-Flat:  200.

        - Flat:  100.

  - CONCRETE UNIT MASONRY BLOCK FILLERS

    - Concrete Unit Masonry Block Filler:  Factory-formulated high-performance latex block fillers.

      - Sherwin-Williams; PrepRite Block Filler (B25W25):  Applied at a dry film thickness of not less than 8 mils per coat.

        - Meets GS-11.

        - Meets MPI MP-33.1 Block Filler, Latex-Base, Interior-Exterior Flat, White.

  - EXTERIOR PRIMERS

    - Exterior Concrete and Masonry Primer-New Construction:  Factory-formulated alkali-resistant acrylic-latex primer for exterior application.

      - Sherwin-Williams; Loxon Acrylic Primer (A24W300):  Applied at a dry film thickness of not less than 3.2 mils per coat.

    - Exterior Concrete and Masonry Primer-for Elastomeric Coatings:  Factory–formulated alkali-resistant acrylic-latex primer for exterior application

      - Sherwin-Williams; Loxon Acrylic Primer (A24W8300): Hot Stucco Applied at a dry film thickness of not less than 3.2 mils per coat.

    - Exterior Concrete and Masonry Primer-for Existing Construction:  Factory–formulated alkali-resistant Acrylic primer for exterior application

      - Sherwin-Williams; Loxon Conditioner (A24W1100):  Applied at a spread rate of 300-400 square feet per gallon.

    - Exterior Wood Primer for Acrylic Enamels:  Factory-formulated alkyd or latex wood primer for exterior application.

      - Sherwin-Williams: PrepRite ProBlock Interior/Exterior Latex Primer/Sealer (B51W620):  Applied at a dry film thickness of not less than 2.3 mils per coat.

- or

      - Sherwin-Williams:  PrepRite ProBlock Interior/Exterior Latex Primer/Sealer (B51W620):  Applied at a dry film thickness of not less than 1.4 mils per coat.

    - Exterior Ferrous-Metal Primer:  Factory-formulated rust-inhibitive metal primer for exterior application.

      - Sherwin-Williams; Pro-Cryl Universal Primer (B66-310 Series):  Applied at a dry film thickness of not less than 2.0 mils per coat (acrylic-based).

        - Meets GS-11.

        - Meets MPI MP-26.3 “Recommended Primer” under Sherwin-Williams DTM Acrylic Gloss Coating (B66 Series).

    - Exterior Zinc-Coated Metal Primer:  Factory-formulated galvanized metal primer for exterior application.

      - Sherwin-Williams; Pro-Cryl Universal Primer (B66-310 Series):  Applied at a dry film thickness of not less than 2.0 mils per coat.

        - Meets GS-11.

        - Meets MPI MP-26.3 “Recommended Primer” under Sherwin-Williams DTM Acrylic Gloss Coating (B66 Series).

    - Exterior Aluminum Primer under Acrylic Finishes:  Factory-formulated acrylic-based metal primer for exterior application.

      - Sherwin-Williams; Pro-Cryl Universal Primer (B66-310 Series):  Applied at a dry film thickness of not less than 2.0 mils per coat.

        - Meets GS-11.

        - Meets MPI MP-26.3 “Recommended Primer” under Sherwin-Williams DTM Acrylic Gloss Coating (B66 Series).

  - INTERIOR PRIMERS

    - Interior Concrete, Masonry, and Brick Primer:  Factory-formulated alkali-resistant acrylic-latex interior primer for interior application.

      - Sherwin-Williams; Promar 200 Low VOC Interior Latex Primer (B28W600):  Applied at a dry film thickness of not less than 1.5 mils per coat.

        - Meets GS-11.

    - Interior Gypsum Board Primer:  Factory-formulated latex-based primer for interior application.

      - Sherwin-Williams; Promar 200 Low VOC Interior Latex Primer (B28W600):  Applied at a dry film thickness of not less than 1.5 mils per coat.

        - Meets GS-11.

    - Interior Plaster Primer:  Factory-formulated latex-based primer for interior application.

      - Sherwin-Williams; Promar 200 Low VOC Interior Latex Primer (B28W600):  Applied at a dry film thickness of not less than 1.5 mils per coat.

        - Meets GS-11.

    - Interior Wood Primer for Acrylic-Enamel:  Factory-formulated acrylic-latex-based interior wood primer.

      - Sherwin-Williams; Harmony Interior Latex Primer (B09W Series):  Applied at a dry film thickness of not less than 1.3 mils per coat.

        - Meets GS-11.

- or

      - Sherwin-Williams: PrepRite ProBlock Latex Primer, B51 Series (4 milswet, 1.4 mils dry)

    - Interior Ferrous-Metal Primer:  Factory-formulated quick-drying rust-inhibitive alkyd-based metal primer.

  - Sherwin-Williams; Pro-Cryl Universal Primer (B66-310 Series): Applied at a dry film thickness of not less than 2.0 mils per coat.

    - Interior Zinc-Coated Metal Primer:  Factory-formulated galvanized metal primer.

      - Sherwin-Williams: Pro-Cryl Universal Primer (B66-310 Series): Applied at a dry film thickness of not less than 2.0 mils per coat.

        - Meets GS-11.

    - Interior Unit Masonry, Gypsum Board and Plaster:  Factory-formulated primer for Epoxy finish.

      - Sherwin-Williams: Promar 200 Low VOC Interior Latex Primer (B28W600):  Applied at a dry film thickness of not less than 1.5 mils per coat.

        - Meets GS-11.

  - EXTERIOR FINISH COATS

    - Exterior Low-Luster Acrylic Elastomeric:  Factory-formulated low sheen 100% Acrylic Elastomeric coating for exterior application.  Note:  Use elastomeric only after receiving approval in writing from the Owner.

      - Sherwin-Williams; SherLastic Elastomeric Coating (A5-100 Series):  Applied at a dry film thickness of not less than 6.0 mils per coat.

    - Exterior Low-Luster Acrylic Paint:  Factory-formulated low-sheen (eggshell) acrylic-latex paint for exterior application.

      - Sherwin-Williams; A-100 Exterior Latex Satin (A82 Series):  Applied at a dry film thickness of not less than 1.3 mils per coat.

    - Exterior Semi-gloss Acrylic Enamel:  Factory-formulated semi-gloss waterborne acrylic-latex enamel for exterior application.

      - Sherwin-Williams: DTM Acrylic Coating (B66-200 Series–Semi-gloss) Applied at a dry film thickness of not less than 2.5 mils per coat.

    - Exterior Full-Gloss Acrylic Enamel for Ferrous and Other Metals:  Factory-formulated full-gloss waterborne acrylic-latex enamel for exterior application.

      - Sherwin-Williams; DTM Acrylic Coating (B66-100 Series-Gloss):  Applied at a dry film thickness of not less than 2.5 mils per coat.

        - Meets GS-11.

        - Meets MPI MP-26.3 Acrylic-Base Gloss Enamel, Metal Surfaces, Whites and Tints.

  - INTERIOR FINISH COATS

    - Interior Flat Acrylic Paint:  Factory-formulated flat acrylic-emulsion latex paint for interior application.

      - Sherwin-Williams: Promar 200 Low VOC Interior Latex Flat (B30-600 Series):  Applied at a dry film thickness of not less than 1.3 mils per coat.

        - Meets GS-11.

    - Interior Flat Latex-Emulsion Size:  Factory-formulated flat latex-based interior paint.

      - Sherwin-Williams: Harmony Interior Latex Flat (B5 Series):  Applied at a dry film thickness of not less than 1.7 mils per coat.

        - Meets GS-11.

    - Interior Low Luster Latex Paint:   Factory-formulated eggshell latex based interior paint.

      - Sherwin-Williams: Harmony Interior Latex Eg-Shel (B9 Series):  Applied at a dry film thickness of not less than 1.6 mils per coat.

        - Meets GS-11.

    - Interior Low-Luster Acrylic Enamel:

      - Sherwin-Williams: Promar 200 Low VOC Interior Latex Eg-Shel (B20-600 Series):  Applied at a dry film thickness of not less than 1.7 mils per coat.

        - Meets GS-11.

    - Interior Semi-gloss Acrylic Enamel:  Factory-formulated semi-gloss acrylic-latex enamel for interior application.

      - Sherwin-Williams; Promar 200 Interior Latex Semi-Gloss (B31-	600 Series):  Applied at a dry film thickness not less than 1.6 mils per coat.

        - Meets GS-11.

    - Interior Full-Gloss Acrylic Enamel:

      - Sherwin-Williams: Pro Industrial Zero VOC Acrylic Gloss (B66W600):  Applied at a dry film thickness of not less than 2.5 mils per coat.

    - Interior Semi-gloss Waterborne Acrylic Epoxy:  Factory-formulated semi-gloss acrylic epoxy coating of interior application.

      - Sherwin-Williams: Water-Based Catalyzed Epoxy (Two Component) (B70W211/B60V15):  Applied at a dry film thickness of not less than 2.5 mils per 	coat.

      - S-W Pro Industrial Pre-Catalyzed Water-based Epoxy, K46W150 Series.

        - Meets GS-11.

      - Benjamin Moore:  IMC Acrylic Epoxy Coating-Semi-Gloss (M43/M44-86).  Applied at a dry film thickness of not less than 1.5 mils.

  - INTERIOR WOOD STAINS AND VARNISHES

    - Open-Grain Wood Filler:  Factory-formulated paste wood filler applied at spreading rate recommended by manufacturer.

      - Sherwin-Williams: Sher-Wood Natural Filler (D70T1).

    - Interior Wood Stain:  Factory-formulated alkyd-based penetrating wood stain for interior application applied at spreading rate recommended by manufacturer.

      - Sherwin-Williams: Minwax 250 VOC Wood Stain

    - Interior Waterborne Clear Satin Varnish:  Factory-formulated clear satin acrylic-based polyurethane varnish applied at spreading rate recommended by manufacturer.

      - 2 Coats-Sherwin-Williams: Wood Classics Waterborne Polyurethane Varnish-Satin (A68 Series):  Applied at a dry film thickness of not less than 0.8 mils per coat.

    - Interior Waterborne Stain Full-Gloss Varnish:

      - 2 Coats-Sherwin-Williams; Wood Classics Waterborne Polyurethane Varnish-Gloss (A68 Series):  Applied at a dry film thickness of not less than 0.8 mils per coat.

- EXECUTION

  - EXAMINATION

    - Examine substrates, areas, and conditions, with applicator present, for compliance with requirements for paint application.  Comply with procedures specified in PDCA P4.

      - Proceed with paint application only after unsatisfactory conditions have been corrected and surfaces receiving paint are thoroughly dry.

      - Start of painting will be construed as Applicator's acceptance of surfaces and conditions within a particular area.

    - Coordination of Work:  Review other Sections in which primers are provided to ensure compatibility of the total system for various substrates.  On request, furnish information on characteristics of finish materials to ensure use of compatible primers.

  - PREPARATION

    - General:  Remove hardware and hardware accessories, plates, machined surfaces, lighting fixtures, and similar items already installed that are not to be painted.  If removal is impractical or impossible because of size or weight of the item, provide surface-applied protection before surface preparation and painting.

      - After completing painting operations in each space or area, reinstall items removed using workers skilled in the trades involved.

    - Cleaning:  Before applying paint or other surface treatments, clean substrates of substances that could impair bond of the various coatings.  Remove oil and grease before cleaning.

      - Schedule cleaning and painting so dust and other contaminants from the cleaning process will not fall on wet, newly painted surfaces.

    - Surface Preparation:  Clean and prepare surfaces to be painted according to manufacturer's written instructions for each particular substrate condition and as specified.

      - Provide pull tests on existing painted surfaces where a different coating will be applied.  Verify with the manufacturer that the coatings are compatible.

      - Provide barrier coats over incompatible primers or remove and reprime.

      - Cementitious Materials:  Prepare concrete, concrete unit masonry, cement plaster, and mineral-fiber-reinforced cement panel surfaces to be painted.  Remove efflorescence, chalk, dust, dirt, grease, oils, and release agents.  Roughen as required to remove glaze.  If hardeners or sealers have been used to improve curing, use mechanical methods of surface preparation.

        - Use abrasive blast-cleaning methods if recommended by paint manufacturer.

        - Determine alkalinity pH and moisture content of surfaces by performing appropriate tests.  If surfaces are sufficiently alkaline to cause the finish paint to blister and burn, correct this condition by approved curing methods in Section 09220-Porland Cement (Stucco).  Do not paint surfaces if moisture content exceeds that permitted in manufacturer's written instructions.

        - Clean concrete floors to be painted with a 5 percent solution of muriatic acid or other etching cleaner.  Flush the floor with clean water to remove acid, neutralize with ammonia, rinse, allow to dry, and vacuum before painting.

      - Wood:  Clean surfaces of dirt, oil, and other foreign substances with scrapers, mineral spirits, and sandpaper, as required.  Sand surfaces exposed to view smooth and dust off.

        - Scrape and clean small, dry, seasoned knots, and apply a thin coat of white shellac or other recommended knot sealer before applying primer.  After priming, fill holes and imperfections in finish surfaces with putty or plastic wood filler.  Sand smooth when dried.

        - Prime, stain, or seal wood to be painted immediately on delivery.  Prime edges, ends, faces, undersides, and back sides of wood, including cabinets, counters, cases, and paneling.

        - If transparent finish is required, back prime with non-yellowing varnish.

        - Backprime paneling on interior partitions where masonry, plaster, or other wet wall construction occurs on backside.

        - Seal tops, bottoms, and cutouts of unprimed wood doors with a heavy coat of varnish or sealer immediately on delivery.

      - Ferrous Metals:  Clean ungalvanized ferrous-metal surfaces that have not been shop coated; remove oil, grease, dirt, loose mill scale, and other foreign substances.  Use solvent or mechanical cleaning methods that comply with the Steel Structures Painting Counsel's recommendations.

        - Blast steel surfaces clean as recommended by paint system manufacturer and according to SSPC-SP 6/NACE No. 3.

        - Treat bare and sandblasted or pickled clean metal with a metal treatment wash coat before priming.

        - Touch up bare areas and shop-applied prime coats that have been damaged.  Wire-brush, clean with solvents recommended by paint manufacturer, and touch up with same primer as the shop coat.

      - Galvanized Surfaces:  Clean galvanized surfaces with nonpetroleum-based solvents so surface is free of oil and surface contaminants.  Remove pretreatment or “passivators” from galvanized sheet metal fabricated from coil stock by mechanical methods SSPC-SP 7.

    - Material Preparation:  Mix and prepare paint materials according to manufacturer's written instructions.

      - Maintain containers used in mixing and applying paint in a clean condition, free of foreign materials and residue.

      - Stir material before application to produce a mixture of uniform density.  Stir as required during application.  Do not stir surface film into material.  If necessary, remove surface film and strain material before using.

      - Use only thinners approved by paint manufacturer and only within recommended limits.

    - Tinting:  Tint each undercoat a lighter shade to simplify identification of each coat when multiple coats of same material are applied.  Tint undercoats to match the color of the finish coat but provide sufficient differences in shade of undercoats to distinguish each separate coat.

  - APPLICATION

    - General:  Apply paint according to manufacturer's written instructions.  Use applicators and techniques best suited for substrate and type of material being applied.

      - Paint colors, surface treatments, and finishes are indicated in the paint schedules.

      - Do not paint over dirt, rust, scale, grease, moisture, scuffed surfaces, or conditions detrimental to formation of a durable paint film.

      - Provide finish coats that are compatible with primers used.

      - The term "exposed surfaces" includes areas visible when permanent or built-in fixtures, grilles, convector covers, covers for finned-tube radiation, and similar components are in place.  Extend coatings in these areas, as required, to maintain system integrity and provide desired protection.

      - Paint surfaces behind movable equipment and furniture the same as similar exposed surfaces.  Paint surfaces behind permanently fixed equipment and furniture with prime coat only.

      - Paint interior surfaces of ducts with a flat, nonspecular black paint where visible through registers or grilles.

      - Paint back sides of access panels and removable or hinged covers to match exposed surfaces.

      - Finish exterior doors on tops, bottoms, and side edges the same as exterior faces.

      - Finish interior of wall and base cabinets and similar field-finished casework to match exterior.

      - Sand lightly between each succeeding enamel or varnish coat.

    - Scheduling Painting:  Apply first coat to surfaces that have been cleaned, pretreated, or otherwise prepared for painting as soon as practicable after preparation and before subsequent surface deterioration.

      - The number of coats and film thickness required are the same regardless of application method.  Do not apply succeeding coats until previous coat has cured as recommended by manufacturer.  If sanding is required to produce a smooth, even surface according to manufacturer's written instructions, sand between applications.

      - Omit primer over metal surfaces that have been shop primed and touchup painted.

      - If undercoats, stains, or other conditions show through final coat of paint, apply additional coats until paint film is of uniform finish, color, and appearance.  Give special attention to ensure that edges, corners, crevices, welds, and exposed fasteners receive a dry film thickness equivalent to that of flat surfaces.

      - Allow sufficient time between successive coats to permit proper drying.  Do not recoat surfaces until paint has dried to where it feels firm and does not deform or feel sticky under moderate thumb pressure, and until application of another coat of paint does not cause undercoat to lift or lose adhesion.

    - Application Procedures:  Apply paints and coatings by brush, roller, spray, or other applicators according to manufacturer's written instructions.

      - Brushes:  Use brushes best suited for type of material applied.  Use brush of appropriate size for surface or item being painted.

      - Rollers:  Use rollers of carpet, velvet-back, or high-pile sheep's wool as recommended by manufacturer for material and texture required.

      - Spray Equipment:  Use airless spray equipment with orifice size as recommended by manufacturer for material and texture required.

    - Minimum Coating Thickness:  Apply paint materials no thinner than manufacturer's recommended spreading rate to achieve dry film thickness indicated.  Provide total dry film thickness of the entire system as recommended by manufacturer.

    - Mechanical and Electrical Work:  Painting of mechanical and electrical work is limited to items exposed in equipment rooms and occupied spaces.

    - Mechanical items to be painted include, but are not limited to, the following:

      - Uninsulated metal piping.

      - Uninsulated plastic piping.

      - Pipe hangers and supports.

      - Tanks that do not have factory-applied final finishes.

      - Visible portions of internal surfaces of metal ducts, without liner, behind air inlets and outlets.

      - Duct, equipment, and pipe insulation having "all-service jacket" or other paintable jacket material.

      - Mechanical equipment that is indicated to have a factory-primed finish for field painting.

    - Electrical items to be painted include, but are not limited to, the following:

      - Switchgear.

      - Panelboards.

      - Electrical equipment that is indicated to have a factory-primed finish for field painting.

    - Block Fillers:  Apply block fillers to concrete masonry block at a rate to ensure complete coverage with pores filled.

    - Prime Coats:  Before applying finish coats, apply a prime coat, as recommended by manufacturer, to material that is required to be painted or finished and that has not been prime coated by others.  Recoat primed and sealed surfaces where evidence of suction spots or unsealed areas in first coat appears, to ensure a finish coat with no bleed-through or other defects due to insufficient application of sealer or primer.

    - Pigmented (Opaque) Finishes:  Completely cover surfaces as necessary to provide a smooth, opaque surface of uniform finish, color, appearance, and coverage.  Cloudiness, spotting, holidays, laps, brush marks, runs, sags, ropiness, or other surface imperfections will not be acceptable.

    - Transparent (Clear) Finishes:  Use multiple coats to produce a glass-smooth surface film of even luster.  Provide a finish free of laps, runs, cloudiness, color irregularity, brush marks, orange peel, nail holes, or other surface imperfections.

      - Provide satin finish for final coats.

    - Stipple Enamel Finish:  Roll and redistribute paint to an even and fine texture.  Leave no evidence of rolling, such as laps, irregularity in texture, skid marks, or other surface imperfections.

    - Completed Work:  Match approved samples for color, texture, and coverage.  Remove, refinish, or repaint work not complying with requirements.

  - FIELD QUALITY CONTROL

    - Owner reserves the right to invoke the following test procedure at any time and as often as Owner deems necessary during the period when paint is being applied:

      - Owner will engage a qualified independent testing agency to sample paint material being used.  Samples of material delivered to Project will be taken, identified, sealed, and certified in the presence of Contractor.

      - Testing agency will perform appropriate tests as required by Owner

      - Owner may direct Contractor to stop painting if test results show material being used does not comply with specified requirements.  Contractor shall remove noncomplying paint from Project site, pay for testing, and repaint surfaces previously coated with the noncomplying paint.  If necessary, Contractor may be required to remove noncomplying paint from previously painted surfaces if, on repainting with specified paint, the 2 coatings are incompatible.

  - CLEANING

    - Cleanup:  At the end of each workday, remove empty cans, rags, rubbish, and other discarded paint materials from Project site.

      - After completing painting, clean glass and paint-spattered surfaces.  Remove spattered paint by washing and scraping without scratching or damaging adjacent finished surfaces.

    - Remove all spilled, splashed or splattered paint from all surfaces.  Leave entire project in a clean condition.

    - Place scrapings, empty cans, consumed brushes, etc. in plastic bags and dispose of in the proper manner by the Contractor.  Place used mineral spirits and other hazardous liquids in an appropriate container and is the responsibility of the Contractor to properly dispose of in full compliance of E.P.A., 	rules and regulations.

    - Do not mar surface finish by cleaning.

    - Leave entire project in a clean condition.

  - PROTECTION

    - Protect work of other trades, whether being painted or not, against damage from painting.  Correct damage by cleaning, repairing or replacing, and repainting, as approved by Architect.

    - Provide "Wet Paint" signs to protect newly painted finishes.  After completing painting operations, remove temporary protective wrappings provided by others to protect their work.

      - After work of other trades is complete, touch up and restore damaged or defaced painted surfaces.  Comply with procedures specified in PDCA P1.

  - EXTERIOR PAINT SCHEDULE (provide the following finish systems) as part of the system selected:

    - Concrete, Stucco, and Brick:  Unless specialty finish is noted:

      - Low-Luster Acrylic Paint:

        - Primer:  Exterior concrete and masonry primer.

        - Finish:  2 coats Exterior low-luster acrylic paint.

      - Semi-gloss Acrylic-Enamel:

        - Primer:   Exterior concrete and masonry primer.

        - Finish:  2 coats Exterior semi-gloss acrylic enamel.

      - Low Luster Acrylic Elastomeric:

        - Primer/Sealer:  Latex masonry sealer

        - Finish:  2 coats Low luster Acrylic elastomeric waterproof coating.

    - Concrete Unit Masonry:

      - Low-Luster Acrylic Paint:

        - Block Filler:  Concrete unit masonry block filler.

        - Finish:  2 coats Exterior low-luster acrylic paint.

      - Semi-gloss Acrylic-Enamel:

        - Block Filler:  Concrete unit masonry block filler.

        - Finish:  2 coats Exterior semi-gloss acrylic enamel.

      - Low Luster Acrylic Elastomeric:

        - Primer/Sealer:  Alkyd masonry sealer

        - Finish:  2 coats Low luster Acrylic elastomeric waterproof coating

    - Ferrous-Metal:  Primer is not required on shop-primed items.

      - Full-Gloss Acrylic-Enamel:

        - Primer:  Exterior ferrous-metal primer.

        - Finish:  2 coats Exterior full-gloss acrylic enamel for ferrous and other metals.

      - Semi-gloss Acrylic-Enamel:

        - Primer:  Exterior ferrous-metal primer

        - Finish:  2 coats Exterior semi-gloss acrylic enamel for ferrous and other metals.

    - Zinc-Coated Metal:

      - Full-Gloss Acrylic-Enamel:

        - Primer:  Exterior galvanized metal primer.

        - Finish:  2 coats Exterior full-gloss acrylic enamel for ferrous and other metals.

      - Semi-gloss Acrylic-Enamel:

        - Primer:  Exterior ferrous-metal primer

        - Finish:  2 coats Exterior semi-gloss acrylic enamel for ferrous and other metals.

    - Aluminum:  Provide the following finish systems over exterior aluminum surfaces:

      - Full-Gloss Acrylic-Enamel Finish:

        - Primer:  Exterior aluminum primer under acrylic finishes.

        - Finish:  2 coats Exterior full-gloss acrylic enamel for ferrous and other metals.

      - Semi-gloss Acrylic-Enamel Finish:

        - Primer:  Exterior aluminum primer under acrylic finishes

        - Finish:  2 coats Exterior semi-gloss acrylic enamel for ferrous and other metals.

    - Wood:

      - Low-Luster Acrylic Paint:

        - Primer:  Exterior concrete and masonry primer.

        - Finish:  2 coats Exterior low-luster acrylic paint.

      - Low Luster Acrylic Elastomeric:

        - Primer/Sealer:  Latex masonry sealer

        - Finish:  2 coats Low luster Acrylic elastomeric waterproof coating.

  - INTERIOR PAINT SCHEDULE

    - Concrete and Brick:

      - Low-Luster Acrylic-Enamel: (ceilings and soffits).

        - Primer:  Interior concrete and masonry primer.

        - Finish:  2 coats Interior low-luster acrylic enamel.

      - Semi-gloss Acrylic-Enamel:

        - Primer:  Interior concrete and masonry primer.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

    - Concrete Unit Masonry:

      - Semi-gloss Acrylic-Enamel:

        - Block Filler:  Concrete unit masonry block filler.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

    - Concrete Unit Masonry, Gypsum Board and Plaster:

      - Semi-gloss Waterborne Acrylic Epoxy:

        - Primer: Epoxy primer.

        - Finish:  2 coats waterborne semi-gloss Acrylic Epoxy Coating.

    - Gypsum Board:

      - Low-Luster Acrylic-Enamel: (ceilings and soffits).

        - Primer:  Interior gypsum board primer.

        - Finish:  2 coats Interior low-luster acrylic enamel.

      - Semi-gloss Acrylic-Enamel Finish:

        - Primer:  Interior gypsum board primer.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

    - Plaster:

      - Low-Luster Acrylic-Enamel: (ceilings and soffits).

        - Primer:  Interior plaster primer.

        - Finish:  2 coats Interior low-luster acrylic enamel.

      - Semi-gloss Acrylic-Enamel Finish:

        - Primer:  Interior plaster primer.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

    - Acoustical Plaster:

      - Flat Acrylic-Latex Finish:

        - Finish:  2 coats Interior flat acrylic paint.

    - Wood:

      - Semi-gloss Acrylic-Enamel:

        - Primer:  Interior wood primer for acrylic-enamel and semi-gloss alkyd-enamel finishes.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

      - Full-Gloss Acrylic-Enamel:

        - Primer:  Interior wood primer for acrylic-enamel and semi-gloss alkyd-enamel finishes.

        - Finish:  2 coats Interior full-gloss acrylic enamel.

    - Ferrous-Metal:

      - Full-Gloss Acrylic-Enamel:

        - Primer:  Interior ferrous-metal primer.

        - Finish:  2 coats Interior full-gloss acrylic enamel.

      - Semi-gloss Acrylic-Enamel:

        - Primer: Interior ferrous-metal primer.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

    - Zinc-Coated Metal:

      - Full-Gloss Acrylic-Enamel:

        - Primer:  Interior zinc-coated metal primer.

        - Finish:  2 coats Interior full-gloss acrylic enamel.

      - Semi-gloss Acrylic-Enamel:

        - Primer: Interior zinc-coated metal primer.

        - Finish:  2 coats Interior semi-gloss acrylic enamel.

    - All-Service Jacket over Insulation:

      - Flat Acrylic Finish:  Add fungicidal agent to render fabric mildew proof.

        - Finish:  2 coats Interior flat latex-emulsion size.

  - INTERIOR STAIN AND NATURAL-FINISH WOODWORK SCHEDULE

    - Stained Woodwork:

      - Waterborne Clear Satin-Varnish:  Wipe wood filler before applying stain.

        - Filler:  Open-grain wood filler.

        - Stain:  Interior wood stain.

        - Finish:  2 coats Interior waterborne clear satin varnish.

      - Waterborne Stain Full-Gloss Varnish Finish:  Wipe filler before applying stain.

        - Filler:  Open-grain wood filler.

        - Stain:  Interior wood stain.

        - Finish:  2 coats Interior waterborne clear gloss varnish.

    - Natural-Finish Woodwork:

      - Waterborne Clear Satin-Varnish:  Wipe wood filler before applying stain.

        - Filler:  Open-grain wood filler.

        - Finish:  2 coats Interior waterborne clear satin varnish.

      - Waterborne Full-Gloss Varnish:  Wipe filler before applying stain.

        - Filler:  Open-grain wood filler.

        - Finish:  2 coats Interior waterborne clear gloss varnish.

END OF SECTION